3GPP TSG-RAN WG4 Meeting #93                                                                  R4-1913329

Reno, US, 18th-22th Nov, 2019

Source:	Samsung

Title:	Discussion on BCS reporting for EN-DC BC

Agenda item:			7.5.2.3

Document for:	Approval

# 1. Introduction

In this contribution, we discuss the BCS reporting for EN-DC BC based on RAN2’s LS R2-1914226 which ask RAN4 to clarify how to report BCS for EN-DC BC.

LS R2-1914226

A terminal which supports an EN-DC configuration shall support:

- If any subsets of the EN-DC configuration do not specify its own bandwidth combination sets in 5.3B, then the terminal shall support the same E-UTRA bandwidth combination sets it signals the support for in E-UTRA CA configuration part of E-UTRA – NR DC and shall support the same NR bandwidth combination sets it signals the support for in NR CA configuration part of E-UTRA – NR DC.

- Else if one of the subsets of the EN-DC configuration specify its own bandwidth combination sets in 5.3B, then the terminal shall support a product set of channel bandwidth for each band specified by E-UTRA bandwidth combination sets, NR bandwidth combination sets, and EN-DC bandwidth combination sets it singnals the support.

RAN2 observe that the existing BCS signalling cannot support the above EN-DC configuration and so RAN2 is working on extending the UE capability signalling. To support it, RAN2 understands that the following types of BCS need to be signalled, at most.

1)	BCS for LTE CA component

2)	BCS for NR CA component

3)	BCS for intra-band EN-DC component

Nevertheless, RAN2 is not sure the generic principle for what condition these three fields have to be included for an EN-DC band combination. For instance, for BCS reporting for EN-DC BC, RAN2 would like to ask RAN4 to clarify how to report BCS for EN-DC BC including:

- Intra-band EN-DC without LTE CA or NR CA, e.g. DC_(n)41AA, DC_3A_n3A

- Intra-band EN-DC with LTE CA and/or NR CA inside, e.g. DC_(n)41CA, DC_41C_n41A

- Intra-band EN-DC with additional inter-band LTE CA and/or NR CA, e.g. DC_2A-(n)71AA, DC_(n)41AA_n77C

# 2. Discussion

In previous RAN4 discussion for EN-DC BCS, RAN4 agreed no E-UTRA BCS information is needed in NR specification in order to reduce UE fragmentation and save RAN4 specification effort with below agreements reached for EN-DC:

- For intra-band EN-DC, Bandwidth Combination Sets shall be defined [1], and sent an LS to RAN2 informing them that Bandwidth Combination Sets will be added for intra-band EN-DC[2]. - A terminal which supports an inter-band EN-DC configuration shall support the same E-UTRA bandwidth combination sets it signals the support for in E-UTRA CA configuration part of E-UTRA – NR DC and shall support the same NR bandwidth combination sets that it signals the support for in NR CA configuration part of E-UTRA – NR DC[3]. RAN4 reply LS to RAN2 [4], there is no separate BCS defined for inter-band EN-DC, and the BCS of LTE and BCS of NR are re-used.

In additional, RAN4 also discussed how to handle the BCS for inter-band EN-DC including intra-band EN-DC and RAN4 agreed Alt2 in [5]:

- Alt. 2: UE shall support a product set of channel bandwidth for each band specified among E-UTRA BCS, NR BCS, and EN-DC BCS.

Based on all above agreements, The UE just supports the BCSs it signals the support not all BCSs belong to E-UTRA CA, NR CA or intra-band EN-DC. And the generic principle to report BCS for EN-DC can refer to below scenarios, the UE shall:

- For Intra-band EN-DC combination

- Regardless with or without E-UTRA CA and/or NR CA

- Intra-band EN-DC bandwidth combination sets have to be reported.

- For Inter-band EN-DC combination

- Not including intra-band EN-DC

- Report E-UTRA CA bandwidth combination sets and NR CA bandwidth combination sets simultaneously.

- If corresponding bandwidth combination sets have been already reported, the same BCS would be applied for inter-band EN-DC combination.

- Including intra-band EN-DC

- Case1: report NR CA bandwidth combination sets and intra-band EN-DC bandwidth combination sets simultaneously, i.e., DC_(n)41AA_n77C.

- Case2: report E-UTRA CA bandwidth combination sets and intra-band EN-DC bandwidth combination sets simultaneously, i.e., DC_2A_(n)71AA.

- Case3: report E-UTRA CA bandwidth combination sets, NR CA bandwidth combination sets and intra-band EN-DC bandwidth combination sets simultaneously, i.e., DC_1A-3A_n3A-n78A-n79A.

Therefore, for EN-DC, UE need to report these three types of BCS, at most.

1)	BCS for LTE CA component

2)	BCS for NR CA component

3)	BCS for intra-band EN-DC component

And for some band combination cases, the UE need report these three types of BCS simultaneously, and support a product set of channel bandwidth for each band specified among E-UTRA CA BCSs, NR CA BCSs, and intra-band EN-DC BCSs.

- Conclusion

In this contribution for EN-DC BCS, we summarized the discussion progress and agreements, and proposed

Proposal 1:

The generic principle to report BCS for EN-DC can refer to below scenarios, the UE shall:

- For Intra-band EN-DC combination

- Regardless with or without E-UTRA CA and/or NR CA

- Intra-band EN-DC bandwidth combination sets have to be reported.

- For Inter-band EN-DC combination

- Not including intra-band EN-DC

- Report E-UTRA CA bandwidth combination sets and NR CA bandwidth combination sets simultaneously.

- If corresponding bandwidth combination sets have been already reported, the same BCS would be applied for inter-band EN-DC combination.

- Including intra-band EN-DC

- Case1: report NR CA bandwidth combination sets and intra-band EN-DC bandwidth combination sets simultaneously, i.e., DC_(n)41AA_n77C.

- Case2: report E-UTRA CA bandwidth combination sets and intra-band EN-DC bandwidth combination sets simultaneously, i.e., DC_2A_(n)71AA.

- Case3: report E-UTRA CA bandwidth combination sets, NR CA bandwidth combination sets and intra-band EN-DC bandwidth combination sets simultaneously, i.e., DC_1A-3A_n3A-n78A-n79A.

Proposal 2: Reply LS to RAN2 in R4-1913330.

- Reference

[1] R4-1804220, “Bandwidth Combination Sets for B41/n41 intra-band EN-DC”, Sprint

[2] R4-1805919, “LS to RAN2 on BCS for intra-band EN-DC and new band combination notation”, Sprint

[3] R4-1907426, “Definition of BCS support in inter-band EN-DC mode”, Qualcomm

[4] R4-1902680, “Reply LS on BCS definition for MR-DC options”, Ericsson

[5] R4-1909062, “Handling of EN-DC BCS”, NTT DOCOMO, INC.