3GPP TSG-RAN4 Meeting #93	R4-1913578

# Reno, Nevada, USA, 18th - 22th Nov, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0135 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TCI state switch (section 8.10) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CR capturing the TCI state switch requirements - The requirement should apply to the target TCI state at least QCLed Type A,B,C to a SSB. - In the known condition for TCI state, the measurement report for the target TCI state should be in-between the last transmission of the RS resource for beam reporting or measurement and the TCI state switch command received. - In the known condition for TCI state, the TCI state shall also remain detectable with the same Spatial Rx parameter during the TCI state switching period. - TOuk=1 when TCI state switching involves QCL-Type A, B, C only in unknown case. - Tfirst-SSB time definition should be updated to differentiate unknown case with QCL TYype-D, and with QCL Type-A,B,C only in MAC-based and RRC-based TCI state switch. - It should clarify only one TCI state configured in TCI state list tci-StatesToAddModList by RRC reconfiguration in the RRC based TCI state switching delay. - During the RRC based TCI state switching, the UE is not required to receive PDCCH/PDSCH or transmit PUCCH/PUSCH during the RRC reconfiguration procedure and on the OFDM symbols occupied by CSI-RS, SSB or SSB and CSI-RS QCL-ed to the target TCI state. - The TCI state restriction for UE receiving on the old TCI state in RRC-based TCI state switch is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the applicable scenario for TCI state switch requirement - Update definition of known TCI state. - Update definition of TOuk - Update definition of Tfirst-SSB - Clarify the scenario for RRC based TCI state delay requirement - Update the schedulign restriction for RRC based TCI state delay - Update the time that UE should receive on the old TCI state in RC-based TCI state switch - Update the wording |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Section 8.10 Active TCI state switching delay is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

8.10	Active TCI state switching delay

8.10.1	Introduction

The requirements in this section apply for a UE configured with one or more TCI state configurations on serving cell in MR-DC or standalone NR. UE shall complete the switch of active TCI state within the delay defined in this section. The requirements in this section only apply to the case where the targe TCI state is at least TypeA, Type B, or TypeC QCL-ed to an SSB.

The active TCI state switching delay may be extended due to channel condition changes when UE conducts the TCI state switching procedure.

8.10.2	Known conditions for TCI state

The TCI state is known if it has been meeting the following conditions:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the time UE finishing the active TCI state switch,

-	TCI state switch command is received within [1280] ms of the last transmission of the RS resource for L1-RSRP reporting for the target TCI state

-	The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command

-	During the period from L1-RSRP reporting to the time UE finishing the active TCI state switch,

-	The TCI state remain detectable

-	The SSB associated with the TCI state remain detectable

-	SNR of the TCI state is ≥ -3dB

Otherwise, the TCI state is unknown.

8.10.3	MAC-CE based TCI state switch delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ THARQ +3 ms +TOk*(Tfirst-SSB + TSSB-proc). The UE shall be able to receive on the old TCI state until slot n+ THARQ +3 ms +TOk*(Tfirst-SSB).

Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.321 [7];

Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE;

TSSB-proc = 2 ms;

TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise.

If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ THARQ +3 ms + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc). The UE shall be able to receive on the old TCI state until slot n+ THARQ +3 ms+ TL1-RSRP +TOuk*(Tfirst-SSB).

Where T L1-RSRP is the time for L1-RSRP measurement for Rx beam refinement, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

TOuk = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement.

TL1-RSRP_Measurement_Period_SSB = 0 and TOuk = 1 for SSB in FR2 and TL1-RSRP_Measurement_Period_CSI-RS = 0 for CSI-RS in FR2, provided that the TCI state switching involves QCL-TypeA, QCL-TypeB or CQL-TypeC only.

Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE, provided that the TCI state switching involves QCL-TypeA, QCL-TypeB or CQL-TypeC only. Otherwise, Tfirst-SSB is time to first SSB transmission after L1-RSRP measurement by UE.

During MAC CE based TCI state switching, the UE is not required to receive PDCCH/PDSCH or transmit PUCCH/PUSCH on the OFDM symbols occupied by CSI-RS, SSB or SSB and CSI-RS QCL-ed to the target TCI state until the end of switching period.

During MAC CE based TCI state switch the UE is allowed an interruption due to one shot UL timing adjustment on the serving or any activated serving cells as defined in clause 8.2.

8.10.4	DCI based TCI switch delay

If the target TCI state is known, when a UE is configured with the higher layer parameter tci-PresentInDCI which is set as 'enabled' for the CORESET scheduling the PDSCH at slot n, UE shall be able to receive PDSCH or transmit PUSCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+timeDurationForQCL, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], the value of timeDurationForQCL is defined in TS 38.306 [14].

The known condition for TCI state defined in clause 8.10.2 is applied.

During DCI based TCI state switch the UE is allowed an interruption due to one shot UL timing adjustment on the serving or any activated serving cells as defined in clause 8.2.

8.10.5	RRC based TCI state delay

If the target TCI state is known, upon receiving PDSCH carrying RRC activation command with only one TCI state in the TCI state list at slot n, UE shall be able to receive PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc). The UE shall be able to receive on the old TCI state until slot n+ TRRC_processing +TOuk*(Tfirst-SSB).

Where TRRC_processing is the RRC processing delay, TSSB-proc and TOk are as defined in clause 8.10.3.

Tfirst-SSB is time to first SSB transmission after RRC processing by the UE;

If the target TCI state is unknown, upon receiving PDSCH carrying RRC activation command with only one TCI state in the TCI state list at slot n, UE shall be able to receive PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ TRRC_processing +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc). The UE shall be able to receive on the old TCI state until slot n+ TRRC_processing + TL1-RSRP +TOuk*(Tfirst-SSB).

Where TRRC_processing is the RRC processing delay, T L1-RSRP, TSSB-proc and TOuk are as defined in clause 8.10.3.

Tfirst-SSB is time to first SSB transmission after RRC processing by the UE, provided that the TCI state switching involves QCL-TypeA, QCL-TypeB or CQL-TypeC only. Otherwise, Tfirst-SSB is time to first SSB transmission after L1-RSRP measurement by UE.

During RRC based TCI state switching, the UE is not required to receive PDCCH/PDSCH or transmit PUCCH/PUSCH during the RRC reconfiguration procedure and on the OFDM symbols occupied by CSI-RS, SSB or SSB and CSI-RS QCL-ed to the target TCI state until the end of switching period.

During RRC based TCI state switch the UE is allowed an interruption due to one shot UL timing adjustment on the serving or any activated serving cells as defined in clause 8.2.

8.10.6	Active TCI state list update delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive PDCCH to schedule PDSCH with the new target TCI state no later than slot n+ THARQ +3ms +TOk*(Tfirst-SSB + TSSB-proc). Where THARQ, Tfirst-SSB, TSSB-proc and TOk are as defined in clause 8.10.3.

End of Change