3GPP TSG-RAN WG4 Meeting #93	R4-1913688

# Reno, NV, USA, 18th–22nd Nov, 2019

Source: 	Huawei, HiSilicon

Title:	TP for TR 37.823: Specific aspects for TDD

Agenda Item:	8.10.2

Document for:	Approval

# 1	Introduction

In the RAN4#92 meeting, a WF for Rel-16 LTE-MTC coexistence with NR is agreed [1], where it specifies that the TDD specific issues should be captured in the TR. This paper proposes the TP for this topic.

# 2	References

[1] R4-1907849, WF for Rel-16 LTE-MTC coexistence with NR

# 3	Text Proposal

## <Start of Text Proposal>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1] 	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-190770, “Revised WID for Additional MTC enhancements for LTE”

[3]		     3GPP TS 36.211, Physical channels and modulation, v13.2.0

[4]                        R1-1903885, Coexistence of LTE-MTC with NR, Ericsson, RAN1#96bis

[5]	                    3GPP TR 37.824 Coexistence between NB-IoT and NR

## <Skip Unchanged Parts>

## 8.4 Specific aspects for TDD

LTE-MTC supports all E-UTRA TDD configurations as specified in 3GPP TS 36.211. In the meantime, NR TDD configurations are very flexible. Not only slots but also individual symbols within a slot can be configured as either DL, UL or flexible (i.e. GP). This makes it possible to match the NR TDD configuration to that of LTE-MTC.

More explicitly, two different patterns may be configured via the IE TDD-UL-DL-ConfigCommon in SIB1. Furthermore, the common settings may be overridden by UE specific configuration: TDD-UL-DL-ConfigDedicated. Table 8.4-1 shows the details of the RRC signalling.

Table 8.4-1: RRC Signaling for NR TDD Configuration

| Field Name | Note |
| --- | --- |
| TDD-UL-DL-ConfigCommon: | Signalled by SIB1 |
| referenceSubcarrierSpacing | Subcarrier spacing in kHz: 15, 30, 60, etc |
| dl-UL-TransmissionPeriodicity | Pattern periodicity in ms |
| nrofDownlinkSlots | Corresponds to DL subframes in LTE-MTC |
| nrofDownlinkSymbols | Corresponds to DwPTS in LTE-MTC |
| nrofUplinkSymbols | Corresponds to UpPTS in LTE-MTC |
| nrofUplinkSlots | Corresponds to UL subframes in LTE-MTC |
| TDD-UL-DL-ConfigDedicated: | UE specific RRC signalling |
| slotIndex | Corresponds to subframe number in LTE-MTC |
| allDownlink | Corresponds to a DL subframe in LTE-MTC |
| allUplink | Corresponds to a UL subframe in LTE-MTC |
| nrofDownlinkSymbols | Corresponds to DwPTS in LTE-MTC |
| nrofUplinkSymbols | Corresponds to UpPTS in LTE-MTC |

## The main limitation of the NR TDD configuration is that the pattern has to be defined in the order of DL, GP and UL. As a result, the radio frame boundary of NR and that of LTE-MTC may not be aligned. In [5], several examples are given, where the NR TDD configuration is matched to that of LTE UL-DL configuration 1 and 4, respectively.

## <End of Text Proposal>