3GPP TSG-RAN4 Meeting #93	R4-1913742

# Reno, USA, 18th Nov – 22th Nov, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0143 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on interruption requirements for NR V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  |  | Date: |  | 2019-10-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce interruption requirements for NR V2X |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following interruption requirements are introduced for NR V2X: - Interruption requirement due to V2X RRC re-configuration - Interruption requirement due to synchronization source change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interruption requirements for NR V2X are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 12.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change>>

12.7	Interruption

12.7.1	Interruptions to WAN due to V2X Sidelink Communication

This sub-clause contains the requirements related to the interruptions on the serving cell(s) due to V2X sidelink communication.

A UE capable of V2X sidelink communication may indicate its interest (initiation or termination) in V2X sidelink communication to the connected gNodeB/eNodeB using IE [SidelinkUEInformationNR].

The UE is allowed an interruption of up to the duration shown in table 12.7.1-1 on the serving cell(s) during the RRC reconfiguration procedure that includes the V2X sidelink communication configuration message [sl-V2X-ConfigDedicatedNR] (setup and release). This interruption is for both uplink and downlink of the serving cell(s).

Table 1: Interruption length at V2X RRC reconfiguration

|  | NR Slot length (ms) | Interruption length (number of slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 5 |  |
| 3 | 0.125 | 9 |  |

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change

This sub-clause contains the requirements related to the interruptions on the V2X sidelink communication due to synchronization source change.

When GNSS synchronization source is configured as the highest priority and

When synchronization source is changed from

GNSS or SyncRef UE that is synchronized to GNSS directly/in-directly

to

gNB/eNB or SyncRef UE that is synchronized to gNB/eNB directly/in-directly or SyncRef UE that has the lowest priority,

UE is allowed to drop V2V SL TX/RX for up to [1] ms.

When gNB/eNB synchronization source is configured as the highest priority and

When synchronization source is changed from

gNB/eNB or SyncRef UE that is synchronized to gNB/eNB directly/in-directly,

to

GNSS or SyncRef UE that is synchronized to GNSS directly/in-directly or SyncRef UE that has the lowest priority,

UE is allowed to drop V2V SL TX/RX for up to [1] ms.

<< End of Change>>