3GPP TSG-RAN WG4 Meeting #93	R4-1914089

Reno, US, 18th– 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 4883 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 36.104 Introduction of PUSCH performance requirements for enhanced HST senario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_high_speed_enh2-Perf |  |  |  |  |  | Date: |  |  | 2019-10-4 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to introduce PUSCH requirements under high speed train conditions assuming a UE velocity of up to 500km/h. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is based on a draft CR (R4-1912778) endorsed in RAN4 #92bis, and additional changes in RAN4 #93 are highlighted in yellow. - Add new test cases for PUSCH performance under high speed train conditions including HST scenario 1-LTE500 and scenario 3-LTE500 - Add HST scenario 1-LTE500 and scenario 3-LTE500 to Annex B.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PUSCH performance under high speed train conditions assuming a UE velocity of up to 500km/h is not ensured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.1, B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.141 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of text proposal-------------

8.2.3.1	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput stated in table 8.2.3.1-1 at the given SNR.

Table 8.2.3.1-1 Minimum requirements of PUSCH for high speed train

| Channel Bandwidth [MHz] | Cyclic prefix | FRC (Annex A) | Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4 | Normal | A3-2 | 1 | 1 | HST Scenario 3 | 30% | -1.5 |
|  |  |  |  |  |  | 70% | 1.9 |
|  |  |  |  |  | HST Scenario 3-LTE500 | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -3.9 |
|  |  |  |  |  |  | 70% | -0.6 |
|  |  |  |  |  | HST Scenario 1-LTE500 Low | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
| 3 | Normal | A3-3 |  | 1 | HST Scenario 3 | 30% | -2.1 |
|  |  |  |  |  |  | 70% | 1.6 |
|  |  |  |  |  | HST Scenario 3-LTE500 | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -4.5 |
|  |  |  |  |  |  | 70% | -1.0 |
|  |  |  |  |  | HST Scenario 1-LTE500 Low | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
| 5 | Normal | A3-4 |  | 1 | HST Scenario 3 | 30% | -2.6 |
|  |  |  |  |  |  | 70% | 1.3 |
|  |  |  |  |  | HST Scenario 3-LTE500 | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.1 |
|  |  |  |  |  |  | 70% | -1.4 |
|  |  |  |  |  | HST Scenario 1-LTE500 Low | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
| 10 | Normal | A3-5 |  | 1 | HST Scenario 3 | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.2 |
|  |  |  |  |  | HST Scenario 3-LTE500 | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.4 |
|  |  |  |  |  |  | 70% | -1.5 |
|  |  |  |  |  | HST Scenario 1-LTE500 Low | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
| 15 | Normal | A3-6 |  | 1 | HST Scenario 3 | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.2 |
|  |  |  |  |  | HST Scenario 3-LTE500 | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.2 |
|  |  |  |  |  |  | 70% | -1.4 |
|  |  |  |  |  | HST Scenario 1-LTE500 Low | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
| 20 | Normal | A3-7 |  | 1 | HST Scenario 3 | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.2 |
|  |  |  |  |  | HST Scenario 3-LTE500 | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.3 |
|  |  |  |  |  |  | 70% | -1.4 |
|  |  |  |  |  | HST Scenario 1-LTE500 Low | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |

--------------Omitted unchanged sections-------------

B.3	High speed train condition

High speed train conditions are as follows:

Scenario 1 / Scenario 1-LTE500: Open space

Scenario 3 / Scenario 3-LTE500: Tunnel for multi-antennas

The high speed train conditions for the test of the baseband performance are two non-fading propagation channels in both scenarios. For BS with Rx diversity defined in scenario 1 / Scenario 1-LTE500, the Doppler shift variation is the same between antennas.

Doppler shift for both scenarios is given by:

(B.3.1)

where  is the Doppler shift and  is the maximum Doppler frequency. The cosine of angle is given by:

, 	(B.3.2)

,                            (B.3.3)

,                            (B.3.4)

where  is the initial distance of the train from BS, and  is BS-Railway track distance, both in meters;  is the velocity of the train in m/s,  is time in seconds.

Doppler shift and cosine angle is given by equation B.3.1 and B.3.2-B.3.4 respectively, where the required input parameters listed in table B.3-1 and B.3-2, and the resulting Doppler shift shown in Figure B.3-1, B.3-2, B.3-3 and B.3-4 are applied for all frequency bands.

Table B.3-1: Parameters for high speed train conditions for UE velocity up to 350 km/h

| Parameter | Value |  |
| --- | --- | --- |
|  | Scenario 1 | Scenario 3 |
|  | 1000 m | 300 m |
|  | 50 m | 2 m |
|  | 350 km/h | 300 km/h |
|  | 1340 Hz | 1150 Hz |

Table B.3-2: Parameters for high speed train conditions for UE velocity up to 500 km/h

| Parameter | Value |  |
| --- | --- | --- |
|  | Scenario 1-LTE500 | Scenario 3-LTE500 |
|  | 1000 m | 300 m |
|  | 50 m | 2 m |
|  | 500 km/h | 500 km/h |
|  | 1944 Hz | 1944 Hz |

NOTE1:	Parameters for HST conditions in table B.3-1, B.3-2, B.3-3 and B.3-4 including  and Doppler shift trajectories presented on figures B.3-1 and B.3-2 were derived from Band1 and are applied for performance verification in all frequency bands.

Figure B.3-1: Doppler shift trajectory for scenario 1

Figure B.3-2: Doppler shift trajectory for scenario 3

Figure B.3-3: Doppler shift trajectory for scenario 1-LTE500

Figure B.3-4: Doppler shift trajectory for scenario 3-LTE500

--------------End of text proposal-------------