3GPP TSG-RAN WG4 Meeting #92				  												R4-1914332

Reno, USA, 18th Nov. 2019 – 22nd Oct. 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR-U – Baseline Spectral Emission Mask

Agenda item:	9.1.1.2 Wideband operations (UE and BS) [NR_unlic-Core]

Document for:	Discussion and agreement

# Introduction

The definition of spectrum emission requirements applicable for NR-U wideband operation have been discussed since the start of the work item [1]. These discussions have resulted in two WF [2] and [3] on the basis on multiple sessions. The agreements so far have been captured in the following:

There is consensus for the following:

- Regional requirements will be addressed by using NS approach

- Companies will further discuss the generic requirements as baseline mask

- Option 1: NR requirements

- Option 2: Modified ETSI mask

- NS specific requirements will be also discussed based on regional regulatory requirements after baseline 	requirements (mask) is agreed.

Different proposals for a spectral emission mask have been presented in the previous meeting with number these proposals trying to align with the ETSI BRAN harmonized standard for 5 GHz (Option 2). It has however been a discussion point on how to interpret the application of the mask in the regions where some of the LBT sub-bands fails CCA. Also, the widening of the first -20dBr intercept point as a function of the channel bandwidth has been questioned. Multiple of these interpretations and proposed compromises has been presented in [2] and [4].

In parallel to the discussions in RAN4 discussions are ongoing in ETSI TC BRAN in relation to the harmonized standard for 5GHz (ETSI EN 301 893) initiated by a LS send from RAN4 [5].

The proposal in this paper presents what is now being used as baseline for the parallel discussions in ETSI TC BRAN.

# Discussion

The baseline for the discussions within ETSI TC BRAN is that the transmit spectral power mask is given as a combination of:

- Carrier spectral emission mask

- In-carrier spectral emission mask.

The first for application to the overall carrier and the latter for application where some of the sub-channels in a multi-channel configuration fails LBT and therefor becomes unavailable (pounctured).

Carrier spectral emission mask:

Figure 1 presents the carrier spectral power mask which have been updated at the -20 dBr intercept point to have a fixed frequency offset of +/- 1 MHz to the band edge.

Note that it is for further study (FFS) if the fixed frequency offset of +/- 1 MHz to the band edge are applicable to channel bandwidths larger than 100 MHz.

Further, it is also FFS if the emission within this 1 MHz offset are to be measured with a resolution bandwidth (RBW) of 1 MHz or 100kHz.

Figure 1  Carrier spectral emission mask

The carrier emission mask is to be applied for the entire initial transmission bandwidth.

In-carrier spectra emission mask:

Figure 2 shows the in-carrier spectral emission mask which is needed when some of the sub-channels in a multi-channel configuration becomes unavailable. Each of these sub-channels are of 20 MHz and is included when the channel bandwidth is larger than 20 MHz. Table 1 explains the interception points given in Figure 2 when the  unavailable channels are placed either within available sub-channels or at the left or right edge of the wideband carrier.

Figure 2 In-carrier spectral emission mask

Table 1: Definition of interception points in Figure 2

| Placement of unavailable sub-channels | A [MHz] | B [MHz] | C [MHz] | D [MHz] | E [MHz] | F [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Within | LE | LE + 1 | LE + 20 | RE - 20 | RE - 1 | RE |
| Left edge | LE | LE + 1 | LE + 20 | LE + N | - | - |
| Right edge | - | - | RE – N | RE - 20 | RE - 1 | RE |
| Note: N is the Nominal Channel Bandwidth (in MHz) of the initial carrier. LE is the frequency of the lower edge of the succeeded LBT sub-band (in MHz). RE is the frequency of the upper edge of the succeeded LBT sub-band (in MHz). |  |  |  |  |  |  |

LO leakage exception:

In Figure 2 it can be noted that there is an exception point (Exp. P) within the in-carrier spectral emission mask. This have been included to accommodate LO leakage which might fall in the unavailable sub-channels in certain LBT outcome scenarios.

Therefore, it is proposed to allow an exception to the emission limits in an unavailable sub-channel up to 27 dB below the total power (-27 dBc) in one single 1 MHz bandwidth within the wideband carrier or a level of -20 dBm/MHz whichever is the greatest.

Application of the spectral emission masks:

The combined spectral emission mask, consisting of the carrier and in-carrier spectral emission mask, with a potential LO leakage exception, is given as:

- For requirements outside the nominal bandwidth for which transmissions were intended, the most stringent of the carrier and in-carrier spectral emission mask apply.

- For requirements where one or more of the sub-channels in a multi-channel configuration are unavailable the in-carrier spectral emission mask apply.

- For unavailable sub-channels in a multi-channel configuration which are located between active sub-channels and the mask edges of the two in-carrier spectral emission masks overlap, the more relaxed of the two in-carrier spectral emission masks applies.

- In a single 1 MHz bandwidth within the region where the in-carrier spectral emission mask apply an exception allows emission up to 27 dB below the total transmit power for the carrier bandwidth (-27 dBc) or to a level of -20 dBm/MHz whichever is the greatest.

The proposed application of the spectral emission mask has been illustrated for different LBT outcomes and channel bandwidths in the appendix.

Proposal 1: 	It is proposed to adapt the spectral emission mask as described whinin this contribution as baseline or at least working assumption for NR-U.

# Conclusion

This contribution presents a proposal for a baseline emission mask applicable for NR-U.

Proposal 1: 	It is proposed to adapt the spectral emission mask as described whinin this contribution as baseline or at least working assumption for NR-U.

# References

- RP-182878	New WID on NR-based Access to Unlicensed Spectrum, Qualcomm

- R4-1910535, WF on Emission Requirements for NR-U, Nokia

- R4-1913059, WF on punctured channel, Charter

- R4-1909260, NR-U - Discussion on spectral emission mask, Nokia, Nokia Shanghai Bell

- R4-1907850, LS to ETSI TC BRAN on Interpretations of EN 301 893, Nokia

# Appendix

The following shows how the proposed mask will present itself for different LBT outcomes for channel/carrier bandwidths of 40, 60 and 80 MHz. Note that the LO exception have not been illustrated in these figures.