3GPP TSG-RAN WG4 Meeting #93	R4-1914358

Reno, NV, USA, 18 – 22 November 2019

Agenda Item:	9.11.2.1

Source:	Ericsson

Title:	Discussion on L1-SINR measurement requirements

Document for:	Discussion

# 1	Introduction

RAN4#92bis agreed with the way forward L1-SINR measurement in eMIMO [1].

- L1-SINR measurement procedure: - FFS measurement period requirement for L1-SINR measurements based on CMR and IMR. - Scenarios of L1-SINR measurement for RRM requirement: - In RAN4#93, RAN4 study and conclude which scenario(s) shall be included for L1-SINR measurement requirement and evaluation (i.e., measurement period and measurement accuracy). - Down-selecting scenarios to define L1-SINR measurement requirement from following possible scenarios: - Scenario 1: L1-SINR measured on CMR only (when dedicated IMR is not configured) - Scenario 1A: CSI-RS based CMR (only with density 3 REs/RB) - Scenario 2: L1-SINR measured on CMR+IMR (when dedicated IMR is configured) - Scenario 2A: SSB based CMR + ZP-IMR - Scenario 2B: CSI-RS based CMR + ZP-IMR - Scenario 2C: SSB based CMR + NZP-IMR (only with density 3 REs/RB) - Scenario 2D: CSI-RS based CMR + NZP-IMR (only with density 3 REs/RB) - Other options are not excluded, depending on RAN1 agreement. - At least the following aspects are identified to be needed for consideration: - Question-1: Whether to define separate requirements for CMR only and CMR+IMR? - Question-2: Whether to define separate requirements for SSB-based CMR and CSI-RS based CMR? - Question-3: Whether to define separate requirements for different IMR configuration?

We show our view on L1-SINR requirements from RAN4 RRM point of view.

# 2	Discussion

## 2.1	Scenarios for L1-SINR measurement requirements

According to RAN1 agreement, L1-SINR is only used for CSI reporting. In this case UE is in CONNECTED mode and the gNB configures CSI-RS for L1-SINR measurement. Therefore we propose RAN4 prioritize CSI-RS based L1-SINR reporting.

Proposal 1: RAN4 prioritizes the following scenarios for L1-SINR measurement requirements:

- Scenario 1A: CSI-RS based CMR (only with density 3 REs/RB for 1-port CSI-RS)

- Scenario 2B: CSI-RS based CMR + ZP-IMR

- Scenario 2D: CSI-RS based CMR + NZP-IMR (only with density 3 REs/RB)

## 2.2	L1-SINR measurement requirements

The way forward agreed in RAN4#92bis [1] lists several questions. We list our view on the questions as follows:

Question-1: Whether to define separate requirements for CMR only and CMR+IMR?

Ericsson: It depends on the L1-SINR accuracy. RAN4 needs to study the L1-SINR measurement accuracy with CMR only and CMR+IMR. If it is observed the accuracies are identical, RAN4 does not need to have separate requirements.

Question-2: Whether to define separate requirements for SSB-based CMR and CSI-RS based CMR?

Ericsson: It depends on the discussion on the scenario (See 2.1). If RAN4 agree to specify both SSB based L1-SINR and CSI-RS based L1-SINR, RAN4 should define the separate requirements as same as L1-RSRP requirements.

Question-3: Whether to define separate requirements for different IMR configuration?

Ericsson: It depends on the L1-SINR accuracy. RAN4 needs to study the L1-SINR measurement accuracy with ZP-IMR and NZP-IMR. If it is observed the accuracies are identical, RAN4 does not need to have separate requirements.

Proposal 2: RAN4 study the L1-SINR accuracy with the following assumption:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | No, ZP-IMR, NZP-IMR |
| Side condition (SNR) | -3dB |
| Density (D) | 3 |
| Number of samples (M) | 1, 3, 5 |
| Number of PRBs | 48 |
| Propagation condition | AWGN |

# 3	Summary

Proposal 1: RAN4 prioritizes the following scenarios for L1-SINR measurement requirements:

- Scenario 1A: CSI-RS based CMR (only with density 3 REs/RB for 1-port CSI-RS)

- Scenario 2B: CSI-RS based CMR + ZP-IMR

- Scenario 2D: CSI-RS based CMR + NZP-IMR (only with density 3 REs/RB)

Proposal 2: RAN4 study the L1-SINR accuracy with the following assumption:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | No, ZP-IMR, NZP-IMR |
| Side condition (SNR) | -3dB |
| Density (D) | 3 |
| Number of samples (M) | 1, 3, 5 |
| Number of PRBs | 48 |
| Propagation condition | AWGN |

# References

- R4-1912742, “WF on RRM Requirements Scope for Rel-16 NR eMIMO WI”, Samsung, Huawei.

# Appendix

RAN1 agreements L1-SINR reporting.

RAN1#94bis Agreement - L1-SINR is supported. L1-RSRQ is not supported. - Companies to study and provide definition of L1-SINR - Study the reporting content, e.g. - Whether CRI/SSBRI is reported - Whether differential group/non-group reporting is applied - Whether L1-RSRP is reported - Study the interference measurement mechanism RAN1#97 Agreement - When dedicated IMR is not configured, - If CMR is based on CSI-RS, when L1-SINR is configured, and interference measurement is performed using CMR with CSI-RS only with density 3 REs/RB for 1-port CSI-RS is used - Spec does not require UE to use SSB for interference measurement - Note: CSI-RS above is CSI-RS for BM - When dedicated IMR is configured, - NW can configure interference measurement for L1-SINR with either of the following options - ZP-IMR only - NZP-IMR only - (WA) ZP-IMR and NZP IMR (interference measurement is taken on both) - Maximum Number of ZP IMR is 1 - If IMR is configured based on NZP IMR only, when L1-SINR is configured, interference measurement is performed only with density 3 REs/RB CSI-RS - If IMR is configured based on ZP IMR only, when L1-SINR is configured, interference measurement is performed using ZP IMR - FFS: interference measurement is performed using CMR additionally - Support of L1-SINR is optional - FFS: Support of NZP IMR and ZP IMR are separate UE capabilities - Note: CSI-RS above is CSI-RS for BM RAN1#98 (1908 Prague) When gNB configures UE to report SSBRI/CRI and corresponding L1-SINR, the following report format is supported. CSI report number; CSI fields. CSI report #n; CRI or SSBRI #1, if reported. CRI or SSBRI #2, if reported. CRI or SSBRI #3, if reported. CRI or SSBRI #4, if reported. SINR #1, if reported. Differential SINR #2, if reported. Differential SINR #3, if reported. Differential SINR #4, if reported. - FFS: range and step size of differential SINR - Differential SINR #N is determined based on the difference between measured SINR corresponding to the CRI/SSBRI #N and the measured SINR corresponding to CRI/SSBRI #1 - The SINR #1 is the largest SINR among reported SINRs - The range of SINR is [-23, 40] dB - The SINR is quantized based on what is specified in 38.133 Agreement For L1-SINR based beam report, in a CSI-reportConfig, if IMR is configured to be based on ZP-IMR only: - CMR and IMR are 1-to-1 mapped from signaling perspective Email discussion by September 20th Yushu (Apple) on the details (such as how measurement of interference is done, the mapping relation between CMR/IMR, QCL, etc) of L1-SINR based beam report using the following as a starting point: For L1-SINR based beam report, in a CSI-reportConfig, if IMR is configured to be based on NZP-IMR only, down-select at least one of the following resource configuration schemes: - Option 1: CMR and IMR are 1-to-1 mapped - Option 2: 1 CMR can be mapped to 1 or more than 1 IMRs - Option 3: 1 IMR can be mapped to 1 or more than 1 CMRs - Option 4: More than 1 IMRs can be mapped to more than 1 CMRs. Agreement Support gNB to configure L1-SINR based beam report for both non-group based and group based beam reporting. RAN1#98bis Agreement - The bit length is fixed to be 4 bits for differential L1-SINR - The step size is fixed to be 1 dB for differential L1-SINR Agreement The value of k for L1-SINR to determine the priority value of CSI report equals to 0. Agreement For NZP-IMR based interference measurement, option 1a is supported - In a CSI-reportConfig, gNB configures a list of N CMR(s) and another list of N IMR(s), and they are 1:1 mapped - For each SINR, interference is measured based on each associated NZP-IMR only - UE may assume that the NZP CSI-RS resource for channel measurement and NZP CSI-RS resource(s) for interference measurement configured for one CSI reporting are QCLed with respect to 'QCL-TypeD’ - FFS: Whether QCL-TypeD can be configured to each NZP IMR - FFS: Each NZP CSI-RS port configured for interference measurement corresponds to an interference transmission layer - FFS: Additional support of option 2a (without RRC signalling impact) Note: There is no consensus in RAN1 on the support of option 2b/2c (which introduces IMR index reporting for L1-SINR) Conclusion How to measure interference for L1-SINR from configured ZP/NZP IMR resources is up to UE implementation. Email discussion to collect possible alternatives to define Z, Z’ and CPU occupancy for L1-SINR till 31st Oct – Yushu (Apple).