TSG-RAN Working Group 4 (Radio) meeting #93	R4-1914532

Reno, US, 18th – 22nd November 2019

Source:	Ericsson

Title:	TP to TR 38.820: Adding information in RX requirement overview tables in subclause 7.4.2.1

Agenda item:	11.4.8.4

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#90-bis in Chongqing) technical background for FR1 and FR2 as well as considerations for the frequency range 7 to 24 GHz was captured in TR 38.820, subclause 7.4 [1].

A text proposal to update the summary tables have been created. The text proposal adds findings and conclusions in the receiver requirement overview tables. In this contribution a text proposal is attached for approval.

# Discussion

Summary of additions;

- Information from [2] is divided and moved to conducted requirement table in Table 7.4.2.1-1 and OTA requirement table in Table 7.4.2.1-2.

- Information captured in subclause 7.4 is summarized in Table 7.4.2.1-1 and Table 7.4.2.1-2.

# Conclusion

At the end of this contribution a text proposal is attached for approval.

# References

[1]	R4-1913080, “DRAFT TR 38.820 V0.3.0”, Huawei

[2]	R4-1911844, “TP to TR 38.820: Requirement approach in subclause 7.4.1 and subclause 7.4.2”, Ericsson

TEXT PROPOSAL:

#### 7.4.2.1	Rx requirements overview

Summary of the conducted and radiated Rx requirements specified in Rel-15 for the NR BS is presented in this subclause. More detailed elaboration on the motivation on selected requirements is provided in dedicated subclauses below.

All the findings captured for the conducted requirements in table 7.4.2.1-1 and related subclauses below are considered to be applicable to the (sub)-range of the 7 – 24 GHz for which the conducted requirements will be found to be feasible during related WI.

While radiated requirements are considered to be applicable to the whole 7 – 24 GHz range, their definitions, values and levels may differ across the 7 – 24 GHz range.

Table 7.4.2.1-1: Overview of conducted Rx requirements for NR BS in 7 – 24 GHz range

| Rx requirement |  | Conclusions from SI | Items to be completed in related WI |
| --- | --- | --- | --- |
| Reference sensitivity level |  | Indicative Noise figure values was concluded in the SI where for 10, 15 and 20 GHz example frequencies, the NF value of 7, 8 and 9 dB respectively. | To be concluded in WI. |
| Dynamic range |  | Requirement will have to be re-calculated to account for an updated NF, IM and the supported set of NRB and SCS. Required SNR for the wanted signal to be re-simulated. For the derivation of the requirement: reuse the 95% throughput threshold, reuse the 16QAM-based FRC (if possible). | Value(s) of the NF, IM and required SNR. Consider capturing this aspect by demod requirements as for FR2. |
| In-band selectivity and blocking | ACS | Use FR1 approach for example frequency 10 GHz | To be concluded in WI when specific frequency bands are defined, and proper co-existence studies are performed. |
|  | In-band blocking | As the 7 to 24 GHz specification has to deal with all the BS types and implementation architectures the in-band blocking level should be set based on the wanted signal to interferer level in the same way as the FR2 levels. The conducted requirement can then be extracted by using the same delta on the conducted REFSENS value. As there are no existing conducted requirements to maintain equivalence to and the 7 to 24 GHz range will primarily consider beam forming systems it is suitable to derive the OTA requirements 1st and then apply the same methodology to the conducted. | Over the 7 to 24 GHz range it is possible there are multiple in-band blocking deltas covering different frequency ranges. |
| Out-of-band blocking | General out-of-band blocking | The interferer level is -15 dBm below 7.125 GHz. | To be concluded in the WI. |
|  | Co-location | Not discussed in the SI. | Need to establish antenna port isolation for specific band in WI. |
| Receiver spurious emissions |  | Not discussed in the SI. | To be concluded in the WI. |
| Receiver intermodulation |  | Not discussed in the SI. | To be concluded in the WI. |
| In-channel selectivity |  | Not discussed in the SI. | To be concluded in the WI. |

Table 7.4.2.1-2: Overview of radiated Rx requirements for NR BS in 7 – 24 GHz range

| OTA Rx requirement |  | Conclusions from SI | Items to be completed in related WI |
| --- | --- | --- | --- |
| OTA sensitivity |  | The OTA sensitivity requirement is a minimum sensitivity requirement and ensures correct operation of the receiver including the integral antenna. OTA sensitivity is a declared parameter, the minimum sensitivity over an associated range of angle of arrivals (RoAoA) are declared (the declaration also allows for the RoAoA to be redirected by non-real time means but this distinction is not needed for simple analysis). | To be concluded in WI. |
| OTA reference sensitivity level |  | The 7 to 24GHz BS is expected to require beam forming and whilst there may be conducted requirements there is no legacy so there is no need to provide equivalence between OTA and conducted requirements. As such the FR2 type of OTA REFSENS requirement may be used even if conducted specifications are specified. The appropriate NF and ranges of expected antenna gain can be agreed when the exact operating bands are known. Indicative Noise figure values was concluded in the SI where for 10, 15 and 20 GHz example frequencies, the NF value of 7, 8 and 9 dB respectively. | To be concluded in WI. |
| OTA dynamic range |  | For FR1-like (sub)-range: reuse FR1 approach of deriving the OTA requirement. For FR2-like (sub)-range: further investigation will be required to decide if the OTA dynamic range requirement in the FR2-like (sub)-range of 7 – 24 GHz range can be skept. Value(s) of the NF, IM and required SNR to be reused from the conducted requirement for 7 – 24 GHz. For the derivation of the requirement: reuse the 95% throughput threshold, reuse the 16QAM-based FRC (if possible). | Evaluations for the FR2-like (sub)-range. |
| OTA in-band selectivity and blocking | OTA ACS | Use FR2 approach for example frequency 10, 15 and 20 GHz | To be concluded in WI when specific frequency bands are defined, and proper co-existence studies are performed. |
|  | OTA in-band blocking | In-band requirements dependent on the sensitivity requirements, as the sensitivity is used as a metric of the receiver performance under interference conditions. The in-band blocking requirement for 7 to 24 GHz must therefore be consistent worth the methodology used for sensitivity. The delta value for the 7 to 24 GHz range would be expected to be between the FR1 and FR2 values (58.7 to 33 dB), however the precise values would have to be found by blocking simulation once the operating frequencies ate known and co-existence simulation parameters have been defined. | Over the 7 to 24 GHz range it is possible there are multiple in-band blocking deltas covering different frequency ranges. To be concluded in WI when specific frequency bands are defined, and proper co-existence studies are performed. |
| OTA out-of-band blocking | General out-of-band blocking | The interferer level is 0.36 V/m below 7.125 GHz and 0.1 V/m above 24.125 GHz. | The interferer signal within the range 7 to 24 GHz is defined in the WI. |
|  | Co-location | Not discussed in the SI. | Need to establish antenna port isolation for specific band in WI. A new concept of injecting the interferer signal is required. |
| OTA receiver spurious emissions |  | Not discussed in the SI. | To be concluded in WI. |
| OTA receiver intermodulation |  | Not discussed in the SI. | To be concluded in WI. |
| OTA in-channel selectivity |  | Not discussed in the SI. | To be concluded in WI. |