3GPP TSG-RAN WG4 Meeting#93                	R4-1914538

Reno, Nevada, US, November 18th – 22nd 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	System simulation results and RRM Requirements NR HST

Agenda item:	9.7.1

Document for:	Approval

Introduction

During the RAN4#92 meeting, the work plan for RRM parts under high speed train scenarios [1] was approved. One part of the objective is to investigate and specify the UE RRM core requirements including cell detection and measurement latencies. To analyze the RRM requirements and their impact on the UE performance in the HST scenario, system level simulations are needed. In this paper we provide new additional system simulation results using a fully dynamic system simulator, for analyzing connected mode RRM performance under high speed train scenarios. We use the 3GPP NR RMa scenario [2] and a more advanced setup with 2 beams per cell (in [5] we presented results using 1 beam per cell).

Discussion

In LTE Rel-14 RAN4 carried out simulations for HST scenario for up 350km/h for PCell only. During that work it was acknowledged that there was a need to tighten some of the UE RRM requirements concerning cell detection and measurement performance during DRX mode. TS 36.133 captures the updated requirements which applies for UE configured with highSpeedEnhancedMeasFlag.

For Rel-16 RAN4 has worked on LTE HST for PCell velocity of up to 500km/h and 350km/h when configured with CA. The requirements for these scenarios were recently agreed and included an additional tightening of several UE RRM requirements including:

- PCell reselection in idle mode

- UE uplink timing

- PCell measurements in connected mode with DRX

- SCell measurements

In order to analyze the impact of a UE in HST conditions in NR deployments, it is necessary to see how the UE performs on system level more realistic RRM requirements, UE latencies and mobility performance. Hence, using the NR HST conditions, it needs to be verified whether the current UE minimum RRM requirements for FR1 enables robust system performance in a dynamic mobile environment.

It needs to be evaluated if, assuming minimum UE requirements, the UE can be assumed to be capable of identifying, measuring and reporting newly detectable cells in a timely manner to the network, such that network can prepare the handover with the target cell and transmit the handover command to the UE in time.

It needs to be evaluated if, assuming minimum UE requirements, the UE can be assumed to be capable of identifying, measuring and reporting newly detectable cells in a timely manner to the network, such that network can prepare the handover with the target cell and transmit the handover command to the UE in time.

If UE minimum performance in terms of cell detection and measurement latencies are too relaxed to guarantee that a UE with minimum performance can provide measurement reports to network timely, leading to HOF’s or RLF’s, the UE minimum requirements are too relaxed. Hence, will the current requirements guarantee well performing and robust mobility in HST scenario, or will they result in late or missed handovers which then leads to RLF and drop of the connection.

If UE minimum performance in terms of cell detection and measurement latencies are too relaxed to guarantee that a UE with minimum performance can provide measurement reports to network timely, leading to HOF’s or RLF’s, then the UE minimum requirements are too relaxed for HST scenario.

When analyzing the performance and latencies, the new and additional aspect introduced in NR, compared to LTE, needs to be considered. NR include support of multi-beam operation and beam management, including beam failure detection and link recovery procedures, and this needs to be considered as it is part of the NR system level operations.

To perform such evaluation, we have implemented a fully dynamic system simulator supporting NR as explained next. Using this fully dynamic NR simulator, in which the UE are moving within the simulated deployment with realistic channel conditions, we have been executing a number of simulations related to NR HST.

Simulation scenario and parameters

We have used a more advanced simulation setup in which we have 2 beams per cell. Although still simplified compared to the maximum support of 8 beams, using 2 beams will give indication about HST performance in multibeam environment.

The general setup is based on the simulation deployment setup used in the LTE HST simulations updated to use the NR RMa scenario [2]. The NR RMa includes pathloss fast fading channel models. Additionally, the support for NR physical layer design is implemented including among other cell detection and measurements and related delay, RLM and needed signaling for mobility.

In the setup we use 2 beams per cell and antenna panel setup is 4 rows and 4 columns of cross-polarized antenna elements. Beams are pointed ±20 degrees in azimuth and -15 degrees in elevation from the antenna boresight. Antenna boresight is pointed south towards the track by 11 degrees from x-axis and gNBs are 100 meters away from the track.

The propagation map of the simulation scenario is illustrated in the following Figure 1:

Figure 1 Propagation map illustration of the simulation scenario.

The complete simulation parameters and assumptions are shown in Table 1.

Table 1: System level simulation assumptions

| Parameters | Value | Comments |
| --- | --- | --- |
| Cell layout | 10 cells | 5 sites; 2 cells per site 2 wrap-around areas from the left and from the right |
| Network synchronization | Synchronized |  |
| Inter-site distance | 1000 m |  |
| Distance between gNB and railroad track | 100 m |  |
| Carrier frequency | 3.6 GHz |  |
| Carrier bandwidth | 5 MHz |  |
| Subcarrier spacing | 15 kHz |  |
| Antenna panel configuration | 3D antenna at gNB, (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1) Omni at UE | One antenna panel at gNBs with 4 rows, 4 columns of cross-polarized elements |
| Receiver types | LMMSE-IRC in DL and UL |  |
| Antenna gain | BS: 17dBi UE: 0dBi |  |
| Antenna height | BS: 35m UE: 1.5m |  |
| Antenna boresight angle with respect to the x axis | ±11 degrees |  |
| Beam configuration | 2 beams per cell, pointing ±20 degrees in azimuth and -15 degrees in elevation from the antenna boresight |  |
| MAC-based beam management | Ideal | Ideal beam management |
| Beam failure and recovery | Disabled | Beam failure and recover are not supported |
| User speed | 500 km/h (138.9 m/s) |  |
| UE distribution | All UEs are generated consecutively in the left most point with the scenario in 1.44 seconds (500 km/h, 30 UE/s). | This is analogical of generating UEs in the train of 200m length. |
| Number of UEs | 63 |  |
| User mobility model | Constant speed, wrap around |  |
| Distance-dependent path loss | NR RMa [2] |  |
| Line-of-sight condition | LOS |  |
| Penetration loss | dB and [3] |  |
| Slow fading/Shadowing | NR RMa [2] |  |
| Fast fading | NR RMa 3D channel model [2] |  |
| Network load | 0, 25%, 50%, 75%, 100% of RBs active in the network |  |
| SSB periodicity | 20ms | Dual beam operation |
| Cell detection delay | Cell detection delay: 600ms or 5* DRX cycles at SINR >= -6dB | Scaling factor 1.5 is not considered. |
| L1 measurement period | Measurement period: 3* DRX cycles at SINR>=-6dB | Already accounting tightened measurement period compared to 38.133 which is minimum 200ms or 1.5*5*DRX cycle. Scaling factor 1.5 is not considered in the simulations. |
| Index reading delay |  | Index reading delay is not considered in the simulations. |
| T310 | 1s |  |
| HO | A3-based | Two different thresholds used (see results – ‘A3th’) |
| A3 event parameters | Threshold: 0, 3 dB Time-To-Trigger: 0 ms |  |
| HO Preparation delay | Constant delay of 50ms |  |
| L3 filtering | Disabled |  |
| DRX | Long cycle values: Off (0 ms), 80ms, 160ms, 320ms, 640ms | Other parameters: DRX inactivity timer: 20 ms DRX on-duration length: 20 ms |
| RACH parameters | Constant delay of 40ms |  |
| RRC measurement quantity | SS-RSRP |  |
| RSRP Measurement Error Std. | 2dB | Random error of +-2dB with a normal distribution is added to the RSRP measurement of each cell. |
| Threshold Qout,SNR | -8 dB |  |
| Threshold Qin,SNR | -6 dB |  |
| RRC messages sent over the air | HO command, HO complete Measurement report Re-Establishment (request, response, complete) |  |
| RLM evaluation period | 10 DRX cycles | Per requirements in 38.133 |
| Simulation time | 120 s |  |

Simulation results

Following sections include simulation results looking at the following aspects:

- Handover positioning map

- Handovers per UE per second ratio

- Ping-pong handover ratio

- Handover failure ratio (RLF+HOF)

In the simulations we only have an NR PCell and do not analyse any impact from inter-frequency, SCell or CA requirements.

Handover Positioning Map

Following figure illustrates where the handover in the simulation occurs. According to the deployment (as illustrated in figure 1) we would expect the handover to happen at the handover regions of the cells. In the simulations event A3 has been used with a threshold of 3dB.

Figure 2 Simulation results showing the position of the handovers.

From the results we can observe following:

- When there is no DRX in use, the handover positions are very well aligned with the expected handover regions (gNBs are located at X=500, 1500, 2500, 3500, 4500). We observe some cases which are misaligned which can be explained as due to slow fading effects.

- When the DRX is increased to 160ms, we observe the handover positions are still well aligned with the expected handover regions, but slightly delayed in the direction of train direction. Reason being the delay associated with the DRX and the impact on UE measurements and reporting delays.

- When the DXR is increased to 640ms we see that the handovers are no longer located at the handover regions which indicates that for HST case using a 640ms DRX in a 50% loaded system seems not to be able to deliver robust mobility.

Handovers per UE per second ratio

Following figure illustrates the handovers per UE per second ratio.

Figure 3 Simulation results showing handover per UE per second ratio.

From the results we can observe following:

- As expected, the number of successful handovers per UE per second is heavily influenced by the used DRX cycle.

  - Longer DRX and increased load has a significant negative impact on the handover success rate.

  - When using shorter DRX the impact from the load is insignificant.

- A3 threshold also has big impact on the number of handovers. As expected, the handover rate drops significantly when DRX is in use. The effect of A3 threshold is only visible for longer DRX cycles and does not have effect when DRX cycles are shorter.

Also, for the 2 beams per cell case, the results are as expected as the UE performance requirements are scaled according to the applied DRX cycle used. I.e. cell detection and measurement latencies can be expected to increase with increased DRX cycle – which of course impacts the handover performance in HST scenario.

Ping-pong handover ratio

Next, we look at the ping pong rate. We have defined ping pong rate as handovers back and forth between same BSs within one second.

Figure 4 Simulation results showing the ping pong handover rate.

We observe following:

- Setting the A3 handover threshold low leads to the highest amount of ping pongs when DRX is short.

- With same A3 threshold, when DRX is long we observe low number of ping pongs.

- When A3 threshold is increased the number of ping pongs are significantly reduced.

These results are as would be expected also in the 2 beams per cell HST deployment. The results show that from system level point of view, the more aggressive handover settings (e.g. short DRX and low A3 threshold) which favour high handover success rate, increases the number of ping pongs in the system. Even in an HST deployment.

Handover failure percentage

Finally, we look at the handover failure percentage. The handover failure percentage is defined as (RLF+HOF)/(RLF+HOF+HO)*100.

Figure 5 Simulation results showing the handover failure percentage.

From the results we see:

- Handover failure rate is heavily impacted by the load in system.

- Handover failure rate is heavily impacted by the DRX cycle applied.

- For shorter DRX cycles (and no DRX) we do not observe a significant amount of handover failures even in high load situations. For intermediate to high load cases we observe increase in handover failures from DRX cycles of 320ms.

- In low network load situations, we do not observe any handover failures even for longest DRX cycle.

These results are as interesting as they show that robust mobility is also achievable in the 2 beams per cell HST scenario. Hence, use of multiple beams in FR1 HST cannot be excluded directly. As the long DRX cycles and high load has significant negative impact on the number of observed failures and handover failure rate, use of longer DRX cycles should not be recommended in HST (however this was also observed in LTE and should be no surprise).

Simulation conclusion and further work

From these simulation results – which differ from the results presented in last meeting by having 2 beams per cell instead of only 1 beam per cell – we can conclude that similar to the 1 beam per cell results, that it is possible to achieve robust mobility in NR FR1 for 500kmh HST scenario. This is achievable at least for short DRX cycles even in high load situations.

These simulations do consider 2 beams per cell and RLM/RLF as defined for NR. However, it should be noted that in these simulation results, the beam management is assumed ideal and hence the BM related delays have not been accounted in the results.

Additionally, the results are already assuming tightened measurement period compared to baseline NR FR1 requirements, and the measurement period in DRX does not consider the 1.5 scaling factor.

Similar for the cell detection which also includes a relaxation 1.5 relaxation when DRX is in use – this has not been accounted in the cell detection delay used in the simulations. Impact from the 1.5 scaling in both cell detection and measurement delay is somehow predictable as it will increase the measurement period and thereby the delay.

From the results with 2 beams per cell, we can conclude that further simulations are needed to get full picture of mobility in NR HST when considering all the UE relaxations allowed in 38.133 related to cell detection, Index reading and measurement period. Current simulation results are still optimistic, as they do not account all expected delays in UE side and how they impact on system level.

However, we do expect that for NR we would need at least a similar tightening of the basic requirements as was introduced in LTE 350 and 500km/h HST case. I.e. it would be necessary tighten cell detection and measurement related requirements for the NR PCell case in NR HST conditions.

A similar tightening of the basic UE requirements as was introduced in LTE 350 and 500km/h HST case, would be necessary also for the NR PCell case in HST conditions.

However, how much the requirements would need to be tightened is open and needs further work.

The basic UE NR RRM requirements for NR HST would need to be tightened at least in a similar manner as done for the LTE HST case.

Baseline for tightened NR HST requirements for PCell can be the LTE 350 and 500km/h HST requirements.

These proposals are baseline proposals for tightening. Further discussion is needed if further tightening is needed due to NR specific design and UE requirements.

From the results we can also conclude that NR HST mobility works with the following conditions:

- 1 or 2 beams are used

- UE NR measurement period requirements for PCell are tightened.

- Longer DRX cycle lengths are not used.

The results are to some extend optimistic as not all latencies are considered in the simulations yet. E.g. the 1.5 scaling factor is not considered. Impact from measurement gaps has not been considered either. Additionally, ideal BM has been assumed while in NR it is not possible to ignore the support of beams and the impact from BM would need additional analysis and discussion.

NR support beam management which needs to be accounted when setting the UE requirements for HST.

However, from the results it already seems very clear that a tightening of the UE measurement period is needed. We propose to use same tightening as agreed for LTE HST case – which is also what have been used in these simulations.

Tighten the UE measurement period under HST scenario from 5 samples to 3 samples.

Additionally, we propose to remove the 1.5 scaling factor when UE is operating in HST scenario.

Remove the 1.5 scaling factor for cell detection when UE is operating in HST conditions.

Remove the 1.5 scaling factor for measurement period when UE is operating in HST conditions

RAN4 would need to simulate and analyse the system performance in the NR HST scenario accounting the following features:

- Cells in which more beams are used. It should be analysed how the system performance is expected to be in a multi-beam deployment. This would include MAC based beam management including beam tracking.

- RAN4 should generate generic UE requirements under well defined conditions. For the NR HST scenario, NR cells in FR1 will support 4 or 8 SSB beams. RAN4 should also look at NR HST performance under the conditions that more than 1 and potentially up to maximum number of beams are used. RAN4 could then discuss if any conditions need to be defined for the NR HST case.

- In the work RAN4 should account also the impact from beam failure detection and link recovery.

- Consider the impact from Index reading delay.

- If proposals 4 and 5 are not agreeable, consider 1.5 scaling factor in cell detection and measurement period latencies.

In the system performance analysis RAN4 should as baseline use the already agreed Rel-15 UE minimum requirements as baseline for performance analysis.

Further system simulations and analysis is needed for the NR HST case accounting of multi-beam operation and beam management.

Further on RRM requirements for NR HST

In the agreed WF [4] a number of topics were listed for further discussion. Next, we look at these and give proposals on the topics.

The candidate solution to enhance the intra-frequency cell reselection

One option was to re-use the cell re-selection requirements specified in Rel-16 LTE HST WI also in NR HST. As the SSB periodicity is shorter than the longest DRX period (paging period) this should be feasible solution.

Use Rel-16 LTE HST cell reselection tightening as baseline, unless shown not to work.

Cell Identification in Connected mode

Non-DRX:

One option was not to change current requirements. According to our simulation results, it seems that under non-DRX conditions there is no need to tighten the current requirements assuming no 1.5 scaling factor is not used. However, in a multibeam scenario (which applies to FR1 as well) we do have the UE requirement related to Index reading:

The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index.

This would need to be discussed in RAN4 as part of the cell detection. Hence, intra-cell SSB detection needs further discussion.

Intra-cell SSB detection needs further discussion.

DRX:

From our simulation results it seems that for shorter DRX cycles it is possible to ensure robust mobility. However, as noted the results are still optimistic and not considering all relaxations on UE side. E.g. the 1.5 scaling factor is not considered in the simulations. Exact tightening needed is difficult to say, but it also seems difficult not to have some tightening of the requirements.

Tightening of the cell identification in connected is needed. Exact numbers are FFS.

RLM

It was discussed that for FR1 no changes are needed for HST scenario and Rel-15 RLM could be re-used directly. However, different from LTE HST is that in NR RAN4 allowed a 1.5 scaling when DRX is applied. While the non-DRX requirements likely can be re-used directly we have concerns with the 1.5 scaling factor when in DRX.

For RLM in DRX RAN4 need to discuss removal of the 1.5 scaling factor.

Similarly, RAN4 also need to discuss the 1.5 scaling used for L1 indication (TIndication_interval) when DRX ≤ 320ms is used.

RAN4 also need to discuss the 1.5 scaling used for RLM L1 indication (TIndication_interval) when DRX ≤ 320ms is used.

The combination of high speed, DRX and prolonged RLM period seems challenging.

Beam Management

Under discussion is SSB based and CSI-RS beam management. As NR FR1 do support up to 8 SSB based beam this would need to be considered when defining the UE requirements. However, we also believe that beam management would need to be simulated in order to account for the system level aspects in addition to the pure LL aspects.

Beam management requirements need further simulations before conclusions can be made in RAN4.

What needs to be considered is beam failure procedure and recovery including potential signaling aspect. It should be noted that for FR1 SSB based BM RAN4 allowed a scaling factor 8 for L1-RSRP measurements.

It is also observed that BFD and L1 indication have applied the same 1.5 scaling factor for DRX cycle lengths less than or equal to 320ms. Having such additional delay in HST scenario would need further discussion in RAN4.

RAN4 also need to discuss the 1.5 scaling used for BFD L1 indication (TIndication_interval) when DRX ≤ 320ms is used.

Conclusion

The document has presented new additional system simulation results for Rel-16 NR HST scenarios at 500 km/h for UE in connected mode. The simulation setup is based on the one used in the simulations [5] updated to use 2 beams per cell instead of one beam per cell. In addition, we also give input on	 open aspects related to HST RRM requirements.

- It needs to be evaluated if, assuming minimum UE requirements, the UE can be assumed to be capable of identifying, measuring and reporting newly detectable cells in a timely manner to the network, such that network can prepare the handover with the target cell and transmit the handover command to the UE in time.

If UE minimum performance in terms of cell detection and measurement latencies are too relaxed to guarantee that a UE with minimum performance can provide measurement reports to network timely, leading to HOF’s or RLF’s, then the UE minimum requirements are too relaxed for HST scenario.

From the results, the following observations and proposals are made:

A similar tightening of the basic UE requirements as was introduced in LTE 350 and 500km/h HST case, would be necessary also for the NR PCell case in HST conditions.

NR support beam management which needs to be accounted when setting the UE requirements for HST.

- The basic UE NR RRM requirements for NR HST would need to be tightened at least in a similar manner as done for the LTE HST case.

- Baseline for tightened NR HST requirements for PCell can be the LTE 350 and 500km/h HST requirements.

- Tighten the UE measurement period under HST scenario from 5 samples to 3 samples.

- Remove the 1.5 scaling factor for cell detection when UE is operating in HST conditions.

- Remove the 1.5 scaling factor for measurement period when UE is operating in HST conditions

- Further system simulations and analysis is needed for the NR HST case accounting of multi-beam operation and beam management.

Additionally, we also discussed the open aspects listed in WF [5] based on which we propose:

Use Rel-16 LTE HST cell reselection tightening as baseline, unless shown not to work.

Intra-cell SSB detection needs further discussion.

Tightening of the cell identification in connected is needed. Exact numbers are FFS.

For RLM in DRX RAN4 need to discuss removal of the 1.5 scaling factor.

RAN4 also need to discuss the 1.5 scaling used for RLM L1 indication (TIndication_interval) when DRX ≤ 320ms is used.

Beam management requirements need further simulations before conclusions can be made in RAN4.

RAN4 also need to discuss the 1.5 scaling used for BFD L1 indication (TIndication_interval) when DRX ≤ 320ms is used.

References

[1]   R4-1908677, Work plan for RRM for RRM part of NR support for high speed train scenario, CMCC

[2]    3GPP TR 38.901 V14.3.0, Study on channel model for frequencies from 0.5 to 100 GHz (Release 14)

[3]    3GPP TR 38.802 V14.2.0 (Table A.2.1-2), Study on New Radio Access Technology Physical Layer Aspects (Release 14)

[4]    R4-1912781, WF on RRM for NR HST, CMCC

[5]    R4-1912480, Initial system simulation results for Rel-16 NR high speed train scenarios, Nokia, Nokia Shanghai Bell