3GPP TSG-RAN WG4 Meeting #93	R4-1914775

# Reno, US, 18th November – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0252 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 on CGI reading of LTE cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Core |  |  |  |  |  | Date: |  |  | 2019-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In WID RP-191601, CGI reading requirements with autonomous gap for NR capable UE should be specified for various scenarios. - CGI reading for LTE cell (NR SA UE, EN-DC UE, NE-DC UE, and NR-DC UE) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Requirements for CGI identification of inter-RAT LTE cell are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirments for CGI identification of an inter-RAT LTE cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.6, 9,4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change #1 >

### 9.4.6	NR − E-UTRAN FDD measurements with autonomous gaps

#### 9.4.6.1	Introduction

The requirements in this clause are specified for CGI identification of an E-UTRA FDD target cell and are applicable for a UE:

-	in RRC_CONNECTED state, and

-	configured with SA or NR-DC or NE-DC operation mode.

The overall CGI reporting delay includes RRC procedure delay defined in clause 12 in TS 38.331 [2] and CGI reporting delay in clause 9.4.6.3.

#### 9.4.6.2	CGI identification of an E-UTRA cell with autonomous gaps

The UE shall identify and report the CGI of an E-UTRA FDD target cell when requested by the network for the purpose of ‘reportCGI’. Only one cell is provided to the UE with cellForWhichToReportCGI for identifying the CGI.The UE may make autonomous gaps in both downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3 of TS 38.331 [2]. Note that a UE is not required to use autonomous gap if [high layer parameter] is set to false. If autonomous gaps are used for measurement with the purpose of ‘reportCGI’, regardless of whether DRX or eDRX_CONN is used or not, or whether SCell(s) are configured or not, the UE shall be able to identify a new CGI of NR cell within:

Tidentify_CGI_interRAT = Tbasic_identify_CGI_interRAT ms

Where:

Tbasic_identify_CGI_interRAT = 150 ms. This is the time period used in the above equation where the maximum allowed time for the UE to identify a new CGI of E-UTRA  cell is defined, provided that the E-UTRA FDD cell has been already identified by the UE.

A cell shall be considered identifiable when following conditions are fulfilled:

-	RSRP related side conditions given in Clause 9.1 in TS 36.133 [15] are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex B.2.3 in TS 36.133 [15] for a corresponding Band

The requirement for identifying the CGI of an E-UTRA cell within Tidentify_CGI_interRAT is applicable when no DRX is used as well as when any of the DRX or eDRX_CONN cycles specified in TS 38.331 [2] is used.

Within the time Tidentify_CGI_interRAT, over which the UE identifies the CGI of an E-UTRA FDD cell, the UE shall fulfil interruption requirements specified in,

-	Clause 8.2.2.2.7 if the UE is configured with SA operation mode,

-	Clause 8.2.3.2.8 if the UE is configured with NE-DC operation mode,

-	Clause 8.2.4.2.6 if the UE is configured with NR-DC operation mode.

#### 9.4.6.3	CGI reporting delay

The CGI reporting delay is defined as the time between a command that will trigger a CGI report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

The CGI reporting delay shall be less than Tidentify_CGI_interRAT defined in clause 9.4.6.2.

< End of change #1 >

< Start of change #2 >

### 9.4.7	NR − E-UTRAN TDD measurements with autonomous gaps

#### 9.4.7.1	Introduction

The requirements in this clause are specified for CGI identification of an E-UTRA TDD target cell and are applicable for a UE:

-	in RRC_CONNECTED state, and

-	configured with SA or NR-DC or NE-DC operation mode.

The overall CGI reporting delay includes RRC procedure delay defined in clause 12 in TS 38.331 [2] and CGI reporting delay in clause 9.4.7.3.

#### 9.4.7.2	CGI identification of an E-UTRA cell with autonomous gaps

The UE shall identify and report the CGI of an E-UTRA TDD target cell when requested by the network for the purpose of ‘reportCGI’. Only one cell is provided to the UE with cellForWhichToReportCGI for identifying the CGI.The UE may make autonomous gaps in both downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3 of TS 38.331 [2]. Note that a UE is not required to use autonomous gap if [high layer parameter] is set to false. If autonomous gaps are used for measurement with the purpose of ‘reportCGI’, regardless of whether DRX or eDRX_CONN is used or not, or whether SCell(s) are configured or not, the UE shall be able to identify a new CGI of NR cell within:

Tidentify_CGI_interRAT = Tbasic_identify_CGI_interRAT ms

Where:

Tbasic_identify_CGI_interRAT = 150 ms. This is the time period used in the above equation where the maximum allowed time for the UE to identify a new CGI of E-UTRA  cell is defined, provided that the E-UTRA TDD cell has been already identified by the UE.

A cell shall be considered identifiable when following conditions are fulfilled:

-	RSRP related side conditions given in Clause 9.1 in TS 36.133 [15] are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex B.2.4 in TS 36.133 [15] for a corresponding Band

The requirement for identifying the CGI of an E-UTRA cell within Tidentify_CGI_interRAT is applicable when no DRX is used as well as when any of the DRX or eDRX_CONN cycles specified in TS 38.331 [2] is used.

Within the time Tidentify_CGI_interRAT, over which the UE identifies the CGI of an E-UTRA FDD cell, the UE shall fulfil interruption requirements specified in,

-	Clause 8.2.2.2.7 if the UE is configured with SA operation mode,

-	Clause 8.2.3.2.8 if the UE is configured with NE-DC operation mode,

-	Clause 8.2.4.2.6 if the UE is configured with NR-DC operation mode.

#### 9.4.7.3	CGI reporting delay

The CGI reporting delay is defined as the time between a command that will trigger a CGI report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

The CGI reporting delay shall be less than Tidentify_CGI_interRAT defined in clause 9.4.7.2.

< End of change #2 >