3GPP TSG-RAN WG4 Meeting #93	R4-1914785

Reno, US, 18 November – 22 November 2019

Source:	ZTE

Title:	Simulation assumption on SIB1 decoding for NR CGI reading

Agenda item:	9.15.1.3

Document for:	Approval

1. Introduction

In this contribution we provide simulation assumptions on SIB1 decoding for NR CGI reading.

2. Simulation assumptions

The simulation assumptions are provided in Table 1.

Table 1: Parameters for SIB1 reading (RMSI acquisition)

| Parameters | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 10 |
| Number of transmitter antennas |  | 1 |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz for 4GHz; 120 kHz for 30GHz |
| Number of Rx antennas |  | 2, 4 (uncorrelated. Option 1: With receiver beamforming Option 2: Without receiver beamforming) |
| CP Length |  | Normal |
| Allocated resource blocks for PDSCH |  | 24 |
| Number of PDSCH symbols |  | 12 |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| PDSCH mapping type |  | Type A |
| dmrs-TypeA-Position |  | 2 |
| dmrs-Type |  | 1 |
| dmrs-AdditonalPositions |  | 2 |
| Dmrs-maxLength |  | 1 |
| SNR |  | -8, -6, -4, -2, 0 |
| Propagation Condition |  | AWGN, EPA5, ETU30, TDL- C with 100ns delay spread |
| UE speed and Doppler Shift | - | 3km/h speed |
| Detection Method |  | Soft combining with 2, 4, 8 repetitions |
| Metrics |  | The number of repetitions required to decode RMSI with 90%-ile success rate. |