3GPP TSG-RAN WG4 Meeting #93	R4-1915000

# Reno, US, 18th November – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6768 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.133 on SRS carrier switching interruption requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh_Core |  |  |  |  |  | Date: |  |  | 2019-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Per WID RP-191601, SRS carrier switching requirements for NR SA, NR-DC, EN-DC and NE-DC should be specified for following cases. - LTE SRS carrier switching impacting NR CC - NR SRS carrier switching impacting LTE CC - NR SRS carrier switching impacting NR CC |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Interruptin requirements for SRS carrier switching impacting LTE CC in EN-DC and NE-DC are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No interruption requirments for NR SRS carrier switching. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.32.2.8, 7.36.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change #1 >

7.32	Interruptions with EN-DC

7.32.1	Introduction

This section contains the requirements related to the interruptions on PCell, and MCG SCell when

NR PSCell is added or released, or

transitions between active and non-active during NR PSCell DRX, or

transitions from NR PSCell non-DRX to DRX, or

SCell in either E-UTRA MCG or NR SCG is added or released, or

SCell in either E-UTRA MCG or NR SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA MCG or NR SCG, or

a downlink bandwidth part (BWP) and/or an uplink BWP is switched in NR PSCell or in any NR SCell, or

SRS carrier based switching is performed between the configured component carriers.

The requirements shall apply for EN-DC.

This section contains interruption requirements when the victim cell is PCell or SCell belonging to MCG. Requirements for interruptions where victim cell is the NR PSCell or an NR SCell belonging to SCG are specified in [50].

For a UE which does not support per-FR measurement gaps, interruptions to the PCell or active MCG SCells may be caused by NR PSCell or NR SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PCell or active MCG SCells may be caused by NR PSCell or NR SCells on FR1 only.

7.32.2	Requirements

< unchanged sections omitted >

7.32.2.X	Interruptions at E-UTRA SRS carrier based switching

A PUSCH-less carrier of SCell is a TDD carrier without PUCCH/PUSCH configured. When a UE needs to transmit periodic or aperiodic SRS [16] and/or non-contention based PRACH on a PUSCH-less carrier of SCell, the UE can perform carrier based switching to one or more PUSCH-less carrier of SCells from a carrier with PUSCH or from another PUSCH-less carrier of SCell prior to transmitting SRS and/or PRACH, provided that:

-	switching is from a configured carrier to another activated TDD carrier;

-	the PUSCH-less carrier of SCells to which SRS carrier based switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission or configured via RRC [2] for periodic SRS transmission;

-	the serving cell, from which SRS carrier based switching is performed and whose UL transmission may therefore be interrupted, is indicated by srs-SwitchFromServCellIndex [2];

-	 the SRS switching is not colliding with any other transmission with higher priority defined in [3];

-	the SRS switching is not colliding with PDCCH in subframe 0 and 5 as specified in [3];

-	for UE, which does not support simultaneous reception and transmission for inter-band TDD CA specified in TS 36.331 [2], and is compliant to the requirements for inter-band CA with uplink in one E-UTRA band and without simultaneous Rx/Tx specified in TS 36.101 [5], the SRS or RACH transmission are not simultaneously scheduled with DL subframe #0 or DL subframe #5 on other CCs.

The UE shall not perform SRS carrier based switching if the above conditions cannot be met.

When SRS carrier based switching is performed between component carriers, the UE is allowed interruptions on PCell or any activated SCell during the switching to the PUSCH-less carrier of SCell,

-	with up to X1 slot as specified in Table 7.32.2.X-1.

When SRS carrier based switching is performed between component carriers, the UE is allowed interruptions on PCell or any activated SCell during the switching to the PUSCH-less carrier of SCell,

-	with up to X1 slot as specified in Table 7.32.2.X-1.

Table 7.32.2.X-1: Interruption length X1 (slot)

TBA

7.32.2.Y	Interruptions at NR SRS carrier based switching

A PUSCH-less carrier of SCell is a TDD carrier without PUCCH/PUSCH configured. When a UE needs to transmit periodic or aperiodic SRS [42] and/or non-contention based PRACH on a PUSCH-less carrier of SCell, the UE can perform carrier based switching to one or more PUSCH-less carrier of SCells from a carrier with PUSCH or from another PUSCH-less carrier of SCell prior to transmitting SRS and/or PRACH, provided that:

-	switching is from a configured carrier to another activated TDD carrier;

-	the PUSCH-less carrier of SCells to which SRS carrier switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission or configured via RRC [38] for periodic SRS transmission or indicated by PDcarrierH for PRACH;

-	the serving cell, from which SRS carrier switching is performed and whose UL transmission may therefore be interrupted, is indicated by srs-SwitchFromServCellIndex [38];

-	 the SRS switching is not colliding with any other transmission with higher priority defined in [51];

The UE shall not perform SRS carrier based switching if the above conditions cannot be met.

When SRS carrier based switching is performed between component carriers, the UE is allowed interruptions on PCell or any activated SCell during the switching to the PUSCH-less carrier of SCell,

-	with up to X1 slot as specified in Table 7.32.2.Y-1.

When SRS carrier based switching is performed between component carriers, the UE is allowed interruptions on PCell or any activated SCell during the switching to the PUSCH-less carrier of SCell,

-	with up to X1 slot as specified in Table 7.32.2.Y-1.

Table 7.32.2.Y-1: Interruption length X1 (slot)

TBA

< End of change #1 >

< Start of change #2 >

7.36	Interruptions with NE-DC

7.32.1	Introduction

This clause contains the requirements related to the interruptions on PSCell and SCG SCells when

transitions between active and non-active during NR PCell DRX, or

transitions from NR PCell non-DRX to DRX, or

SCell in either NR MCG or E-UTRA SCG is added or released, or

SCell in either NR MCG or E-UTRA SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either NR MCG or E-UTRA SCG, or

a downlink bandwidth part (BWP) and/or an uplink BWP is switched in NR PCell or in any NR SCell, or

SRS carrier based switching is performed between the configured component carriers.

The requirements shall apply for NE-DC.

This clause contains interruption requirements when the victim cell is PSCell or SCell belonging to SCG. Requirements for interruptions where victim cell is the NR PCell or an NR SCell belonging to MCG are specified in TS 38.133 [50].

For a UE which does not support per-FR measurement gaps, interruptions to the PSCell or active SCG SCells may be caused by NR PCell or NR SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PSCell or active SCG SCells may be caused by NR PCell or NR SCells on FR1 only.

7.36.2	Requirements

< unchanged sections omitted >

7.36.2.X	Interruptions at E-UTRA SRS carrier based switching

A PUSCH-less carrier of SCell is a TDD carrier without PUCCH/PUSCH configured. When a UE needs to transmit periodic or aperiodic SRS [16] and/or non-contention based PRACH on a PUSCH-less carrier of SCell, the UE can perform carrier based switching to one or more PUSCH-less carrier of SCells from a carrier with PUSCH or from another PUSCH-less carrier of SCell prior to transmitting SRS and/or PRACH, provided that:

-	switching is from a configured carrier to another activated TDD carrier;

-	the PUSCH-less carrier of SCells to which SRS carrier based switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission or configured via RRC [2] for periodic SRS transmission;

-	the serving cell, from which SRS carrier based switching is performed and whose UL transmission may therefore be interrupted, is indicated by srs-SwitchFromServCellIndex [2];

-	 the SRS switching is not colliding with any other transmission with higher priority defined in [3];

-	the SRS switching is not colliding with PDCCH in subframe 0 and 5 as specified in [3];

-	for UE, which does not support simultaneous reception and transmission for inter-band TDD CA specified in TS 36.331 [2], and is compliant to the requirements for inter-band CA with uplink in one E-UTRA band and without simultaneous Rx/Tx specified in TS 36.101 [5], the SRS or RACH transmission are not simultaneously scheduled with DL subframe #0 or DL subframe #5 on other CCs.

The UE shall not perform SRS carrier based switching if the above conditions cannot be met.

When SRS carrier based switching is performed between component carriers, the UE is allowed interruptions on PSCell or any activated SCell during the switching to the PUSCH-less carrier of SCell,

-	with up to X1 slot as specified in Table 7.36.2.X-1.

When SRS carrier based switching is performed between component carriers, the UE is allowed interruptions on PSCell or any activated SCell during the switching to the PUSCH-less carrier of SCell,

-	with up to X1 slot as specified in Table 7.36.2.X-1.

Table 7.36.2.X-1: Interruption length X1 (slot)

TBA

7.36.2.Y	Interruptions at NR SRS carrier based switching

A PUSCH-less carrier of SCell is a TDD carrier without PUCCH/PUSCH configured. When a UE needs to transmit periodic or aperiodic SRS [42] and/or non-contention based PRACH on a PUSCH-less carrier of SCell, the UE can perform carrier based switching to one or more PUSCH-less carrier of SCells from a carrier with PUSCH or from another PUSCH-less carrier of SCell prior to transmitting SRS and/or PRACH, provided that:

-	switching is from a configured carrier to another activated TDD carrier;

-	the PUSCH-less carrier of SCells to which SRS carrier switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission or configured via RRC [38] for periodic SRS transmission or indicated by PDcarrierH for PRACH;

-	the serving cell, from which SRS carrier switching is performed and whose UL transmission may therefore be interrupted, is indicated by srs-SwitchFromServCellIndex [38];

-	 the SRS switching is not colliding with any other transmission with higher priority defined in [51];

The UE shall not perform SRS carrier based switching if the above conditions cannot be met.

When SRS carrier based switching is performed between component carriers, the UE is allowed interruptions on PSCell or any activated SCell during the switching to the PUSCH-less carrier of SCell,

-	with up to X1 slot as specified in Table 7.36.2.Y-1.

When SRS carrier based switching is performed between component carriers, the UE is allowed interruptions on PSCell or any activated SCell during the switching to the PUSCH-less carrier of SCell,

-	with up to X1 slot as specified in Table 7.36.2.Y-1.

Table 7.36.2.Y-1: Interruption length X1 (slot)

TBA

< End of change #2 >