3GPP TSG-RAN WG4 Meeting #93	R4-1915121

# Reno, Nevada, USA, 18 - 22 November, 2019

Title: 	Discussion and simulation results on NR UE HST performance requirements under multi-path fading channel

Source: 	Huawei, HiSilicon

Agenda item:	9.17.2.2.4

Document for:	Discussion

Background

During RAN4#92bis meeting, way forward [1] for NR Rel-16 UE demodulation was approved. In this contribution, we share our views about the UE demodulation requirements for NR Rel-16 HST under multi-path fading channel.

Discussion

Number of DMRS symbols

As per TS 38.822[2], DMRS 1+1+1 is mandatory while DMRS 1+1+1+1 is optional, also 1+1+1+1 has larger overhead. To ensure all UE can support the related HST performances and reduce the overhead as much as possible, DMRS 1+1+1 is preferable.

Proposal 1: For multi-path fading channel, it is proposed to set DMRS 1+1+1.

MCS

- For all the channel models in this way forward, FFS on the MCS - Option 1: MCS4 - Option 2: MCS13 - Option 3: MCS17 - Other MCS are not precluded - MCS should be decided based on whether the maximum throughput can be achieved

As per simulation results in Section 3, we can get the following observations and proposal:

Observation 1: For multi-path fading channel, it is not feasible for the following cases:

| Case Number | CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | Rank | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 10MHz/15kHz | 600 | 2Tx 2Rx ULA Low | 2 | 13 | N/A |
| 6 | 10MHz/15kHz | 600 | 2Tx 2Rx ULA Low | 2 | 17 | N/A |
| 17 | 40MHz/30kHz | 1200 | 2Tx 2Rx ULA Low | 2 | 13 | N/A |
| 18 | 40MHz/30kHz | 1200 | 2Tx 2Rx ULA Low | 2 | 17 | N/A |

Observation 2: For multi-path fading channel, MCS 13 with Rank 1 has better performance considering balance between throughput and SNR.

Proposal 2: It is proposed to adopt MCS 13 and Rank 1 for multi-path fading channel.

Simulations

- Simulation assumption for multi-path fading channel Parameter; Value. FDD 15KHz SCS; TDD 30KHz SCS. Antenna configuration; 2×2; 2×4. DMRS type; type 1. Number of DMRS symbols; Option 1: DMRS 1+1+1 Option 2: DMRS 1+1+1+1. TDD pattern; 7D1S2U, S: 6D 4G 4U. MCS; MCS 4; MCS 13; MCS 17 based on 64QAM table. Propagation condition; TDL-C 300ns. TRS periodicity; 10ms, 2slot pattern. PDSCH mapping; Type A, Start symbol 2, Duration 12. Rank; Rank = 1 and 2. BW; 10MHz; 40MHz. Maximum Doppler shift; 600Hz; 1200Hz. Testing metric; SNR @70% of maximum throughput.

We provide our simulations as per the simulation assumptions in [1] for multi-path fading channel as following:

Table 3-1: Ideal Simulation results for NR PDSCH under multi-path fading channel

| Case Number | CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | Rank | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10MHz/15kHz | 600 | 2Tx 2Rx ULA Low | 1 | 4 | -2.09 |
| 2 |  |  |  |  | 13 | 6.51 |
| 3 |  |  |  |  | 17 | 11.10 |
| 4 |  |  |  | 2 | 4 | 2.08 |
| 5 |  |  |  |  | 13 | N/A |
| 6 |  |  |  |  | 17 | N/A |
| 7 |  |  | 2Tx 4Rx ULA Low | 1 | 4 | -5.63 |
| 8 |  |  |  |  | 13 | 2.87 |
| 9 |  |  |  |  | 17 | 6.95 |
| 10 |  |  |  | 2 | 4 | -1.94 |
| 11 |  |  |  |  | 13 | 7.58 |
| 12 |  |  |  |  | 17 | 13.50 |
| 13 | 40MHz/30kHz | 1200 | 2Tx 2Rx ULA Low | 1 | 4 | -2.42 |
| 14 |  |  |  |  | 13 | 7.31 |
| 15 |  |  |  |  | 17 | 13.55 |
| 16 |  |  |  | 2 | 4 | 2.91 |
| 17 |  |  |  |  | 13 | N/A |
| 18 |  |  |  |  | 17 | N/A |
| 19 |  |  | 2Tx 4Rx ULA Low | 1 | 4 | -5.52 |
| 20 |  |  |  |  | 13 | 3.31 |
| 21 |  |  |  |  | 17 | 7.79 |
| 22 |  |  |  | 2 | 4 | -1.40 |
| 23 |  |  |  |  | 13 | 8.53 |
| 24 |  |  |  |  | 17 | 15.82 |

Figure 3-1: Ideal Simulation results for NR PDSCH under multi-path fading channel

Proposals

In this contribution, we discuss and provide our simulation results on NR UE HST performance requirements under multi-path fading channel model. Our observations and proposals are:

Observation 1: For multi-path fading channel, it is not feasible for the following cases:

| Case Number | CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | Rank | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 10MHz/15kHz | 600 | 2Tx 2Rx ULA Low | 2 | 13 | N/A |
| 6 | 10MHz/15kHz | 600 | 2Tx 2Rx ULA Low | 2 | 17 | N/A |
| 17 | 40MHz/30kHz | 1200 | 2Tx 2Rx ULA Low | 2 | 13 | N/A |
| 18 | 40MHz/30kHz | 1200 | 2Tx 2Rx ULA Low | 2 | 17 | N/A |

Observation 2: For multi-path fading channel, MCS 13 with 2Tx4Rx and Rank 1 has better performance considering balance between throughput and SNR.

Proposal 1: For multi-path fading channel, it is proposed to set DMRS 1+1+1.

Proposal 2: It is proposed to adopt 2Tx4Rx, MCS 13 and Rank 2 for multi-path fading channel.

Reference

- R4-1912808, WF on UE demodulation for NR HST, RAN4#92Bis, CMCC

- TR 38.822 User Equipment (UE) feature list