3GPP TSG-RAN WG4 Meeting #93            	R4-1915857

Reno, USA, 18th –22th Nov, 2019

Agenda item:	9.18.1.2

Source:	Samsung

Title:	Simulation results of Type I PMI test cases

Document for:	Discussion

# 1	Introduction

In RAN4 92b meeting, the WF was approved regarding to the initial simulation assumptions for PMI reporting requirements with Tx port larger than 8 and up to 32. The agreements related to PMI performance requirements were reached in the WF as listed below:

- Type of requirements

  - PMI only

- Frequency range

  - FR1

- Number of Tx ports

  - 16 and 32 for defining requirement

  - Test applicability:

Conduct the tests based on UE capability, for UE supporting 16 and 32 Tx ports, conduct test for 32 Tx ports

- Codebook type

  - Type I single panel codebook

- PMI tests:

  - Wideband PMI

  - FFS on subband  PMI

- PMI tests:

  - Wideband PMI

  - FFS on subband  PMI

- Propagation condition:

  - TDLA30-5 for wideband PMI

  - FFS: for subband PMI

- Test metric:

  - As baseline, Relative throughput ratio between following PMI and random PMI at SNR point corresponding to 90% TP with follow PMI

In this contribution, the initial simulation results are provided for alignment purpose.

# 2	Simulation results

We provide simulation results of PMI tests with 16 and 32 CSI-RS ports under 2Rx and 4Rx, respectively. In the tests, the wideband PMI is considered.

Fig2-1: SNR--Throughput of 16x2 FDD

Fig2-2: SNR--Throughput of 16x4 FDD

Fig2-3: SNR--Throughput of 16x2 TDD

Fig2-4: SNR--Throughput of 16x4 TDD

Fig2-5: SNR--Throughput of 32x2 FDD

Fig2-6: SNR--Throughput of 32x4 FDD

Fig2-7: SNR--Throughput of 32x2 TDD

Fig2-8: SNR--Throughput of 32x4 TDD

Table2-1: SNR point at 90% of peak throughput and TP ratio for FDD

| nTx ports | 16 (4,2,4,4) |  | 32 (4,4,4,4) |  |
| --- | --- | --- | --- | --- |
| NRx | 2 | 4 | 2 | 4 |
| SNR point/TP ratio | 13dB/3.36 | 9.33dB/3.61 | 11.15dB/7.38 | 7.37dB/10.15 |

Table2-2: SNR point at 90% of peak throughput and TP ratio for TDD

| nTx ports | 16 (4,2,4,4) |  | 32 (4,4,4,4) |  |
| --- | --- | --- | --- | --- |
| nRx | 2 | 4 | 2 | 4 |
| SNR point/TP ratio | 13dB/3.27 | 9.95dB/3.62 | 12dB/6.025 | 7.74dB/9.21 |

Considering NR deploys in the high carrier frequency, the large bandwidth can be available, it is beneficial due to the additional frequency selection gain can be expected. So, in our view,  subband PMI requirement should be considered for large antenna port.

Proposal 1:  The subband PMI requirement for 32 Tx port should be defined.

As for channel model, the TDL A30-5 for wideband PMI is agreed.  Due to the small delay spread with 30ns, it is expected that no much of frequency selection gain coming from subband PMI. The subband PMI should be applied for large delay spread. NR Rel-15 has already introduced the requirements with propagation condition under TDLA 30, TDL B 100 A and TDL C 300. In the following figure, the initial comparison with different channel condition with subband PMI is provided.

Table2-3: SNR point at 90% of peak throughput and TP ratio for Sub-band

| 32 (4,4,4,4)*2 FDD Sub-band |  |  |  |
| --- | --- | --- | --- |
| Channel | TDLA-30 | TDLB-100 | TDLC-300 |
| Gain@90% Max TP | 10.99 | 8.75 | 7.51 |

Proposal 2:  If RAN4 agree to introduce the requirement with subband PMI for type I single panel codebook, other channel model with high delay spread can be considered.

# 3	Conclusion

In this contrition, the initial simulation results are provided with Type I PMI for Tx ports larger than 8 and up to 32 cases

Proposal 1:  The subband PMI requirement for 32 Tx port should be defined.

Proposal 2:  If RAN4 agree to introduce the requirement with subband PMI for type I single panel codebook, other channel model with high delay spread can be considered.

# References

R4-1912834,”Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32”, Ericsson, 3GPP RAN4 #92bis

R4-1911409,”Discussion on PMI simulation for 16Tx and 32Tx ports”, Samsung, 3GPP RAN4 #92bis

3GPP TS 36.101 V16.2.0, User equipment (UE) radio transmission and reception (Release 16)

# Appendix

The configurations are showed as following:

Table 1  Common parameters for all cases

| Parameter | Unit | Value |
| --- | --- | --- |
| DMRS parameters |  | Type 1, 1 front loaded symbol + 1 additional DMRS symbol |

Table 2  Simulation assumptions for 16x2

| Parameter |  | Unit | Value | Value |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 40 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  | High XP 16 x 2 (N1,N2) = (4,2) (O1,O2) = (4,4) | High XP 16 x 2 (N1,N2) = (4,2) (O1,O2) = (4,4) |
| Rank |  |  | 2 | 2 |
| MCS |  |  | 20 | 20 |
| UL/DL Pattern |  |  |  | 7D1S2U |
| Correlation |  |  | XP-High | XP-High |
| Beamsteering Model |  |  | 38.101-4 Annex B.2.3.2.3 | 38.101-4 Annex B.2.3.2.3 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5 (4,-) | Row 5 (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 | 16 |
|  | CDM Type |  | CDM4 | CMD4 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3 ) |  | Row 11 (0,2,4,6) | Row 11 (0,2,4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (12,-) | (12,-) |
|  | CSI-RS interval and offset |  | 5/1 | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 5/1 | 5/1 |
| ReportConfigType |  |  | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 1 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 |
| CSI-Report interval and offset |  | slot | 5/1 | 5/1 |
| aperiodicTriggeringOffset |  |  | 0 | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) | (4,2) |
|  | CodebookSubsetRestriction |  | 11111111 | 11111111 |
|  | RI Restriction |  | 00000001 | 00000001 |
| Physical channel for CSI report |  |  | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 | 13 |
| Maximum number of HARQ process |  |  | 4 | 8 |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-3), this reported PMI cannot be applied at the eNB downlink before slot#(n+3). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

Table 3 Antenna configuration for all 32 ports

| Parameter | Unit | Value |
| --- | --- | --- |
| Antenna configuration |  | High XP 32x 2 (N1,N2) = (4,4) (O1,O2) = (4,4) |