3GPP TSG-RAN WG4 #94-e		R4-2001234
Electronic Meeting, 24 Feb–6 Mar 2020

Title:	About multi-band relaxation tests

Source:	OPPO

Agenda item:	8.14.1.9

Document for:	Approval

- Introduction

The multi-band relaxation testability issue has been discussed in RAN4 [1] and also in RAN5 [2][3]. An LS [4] from RAN5 was sent to RAN4 for more consideration.

This paper further discuss how to handle the multi-band relaxation testability issue.

- Discussion

2.1 UE certification issue

According to RAN4 requirement definition, the multi-band relaxation value depends on the bands that UE supports rather than the bands that goes to do certification. This may causes problems in real UE certification as described in [3][4], i.e. UE may declare support several bands and apply larger relaxation values but actually only certify part of the supported bands.

Observation 1: RAN5 found the problem in multi-band relaxation, i.e. UE may declare support several bands and apply larger relaxation values but actually only certify part of the supported bands.

RAN5 has informed GCF and PTCRB about this potential certification problem [5], and got reply from both certification groups [6][7]. It can be known that usually GCF can only certify the bands that have activated WI and also requested by its members which means some of the bands that UE supports may not be certified by GCF, while on the contrary PTCRB request all the bands that UE supports will be tested and certified to avoid the aforementioned multi-band testability issue.

Observation 2: GCF might confront with the issue found by RAN5 due to its certification mechanism, while PTCRB can solve it by testing all the bands that UE supports.

To solve the problem of UE certification, the possible approaches are as below:

| Alt 1: Encourage GCF adopt similar approach like PTCRB, i.e. test all the bands that UE supports | - Can solve GCF certification issue - No impact to RAN4/5 - most straight forward |
| --- | --- |
| Alt 2: Change multi-band relaxation definition, i.e. defined according to the bands that doing certification rather than it supports | - Can solve GCF certification issue - RAN4 Rel-15 core requirements need to be re-evaluated |
| Alt 3: Update core requirements, i.e. change multi-band relaxation from total relax to per band relaxation | - Can solve GCF certification issue - RAN4 Rel-15 core requirements need to be changed |

Multi-band relaxation is tightly connected with the peak EIRP and spherical coverage discussion, any changes to the multi-band relaxation will probably cause other requirements are impacted. Therefore, it is not encouraged to change RAN4 Rel-15 core requirements at this stage. From our understanding, the UE certification issue actually can be solved by Alt 1 above, i.e. encourage GCF could adopt flexible solutions in their certification, for example like PTCRB does.

Observation 3: Any changes to the multi-band relaxation will probably cause other requirements are impacted.

Proposal 1: Encourage GCF could adopt flexible solutions in their certification, for example like PTCRB does to avoid the multi-band relaxation certification issue.

2.2 RAN5 testability re-evaluation issue

The problem is that currently RAN5 evaluates the FR2 testability according to the max multi-band relaxation, i.e. 1.7dB which is UEs that support n257/n258/n260/n261. If in the future, more FR2 bands are introduced then it is anticipated the multi-band relaxation will become larger. Then RAN5 may need to re-evaluate the testability. And according to the LS [4], RAN5 is not intended to reopen the testability discussion.

Actually, during the RAN5 discussion, one solution was discussed [2] that RAN4 could inform RAN5 the largest multi-band relaxation could be in the future releases. Then RAN5 could use this relaxation value to avoid of re-evaluating issue. This is a good solution, however, we would like to encourage RAN5 could adopt a relatively larger values from the testability perspective at beginning to avoid possible future new bands caused testability re-evaluation issue.

Observation 4: RAN5 could use relatively larger relaxation value at the beginning to avoid of re-evaluating issue.

Proposal 2: Encourage RAN5 to adopt a relatively larger values from the testability perspective at beginning to avoid possible future new bands caused testability re-evaluation issue.

- Conclusion

UE certification issue

Observation 1: RAN5 found the problem in multi-band relaxation, i.e. UE may declare support several bands and apply larger relaxation values but actually only certify part of the supported bands.

Observation 2: GCF might confront with the issue found by RAN5 due to its certification mechanism, while PTCRB can solve it by testing all the bands that UE supports.

Observation 3: Any changes to the multi-band relaxation will probably cause other requirements are impacted.

Proposal 1: Encourage GCF could adopt flexible solutions in their certification, for example like PTCRB does to avoid the multi-band relaxation certification issue.

RAN5 testability re-evaluation issue

Observation 4: RAN5 could use relatively larger relaxation value at the beginning to avoid of re-evaluating issue.

Proposal 2: Encourage RAN5 to adopt a relatively larger values from the testability perspective at beginning to avoid possible future new bands caused testability re-evaluation issue.

References

- R4-1915345, “Discussion on the practical outcome of Multiband relaxation in certification”, Ericsson, RAN4#93, Nov 2019

- R5-199572, “Multi-band relaxation impact on conformance test”, DCM, RAN5#85, Nov 2019

- R5-199577, “Discussion on the practical outcome of Multiband relaxation in certification”, Ericsson, RAN5#85, Nov 2019

- R5-199424, “LS on Multiband relaxation for FR2”, Ericsson, RAN5#85, Nov 2019

- R5-195451, “Draft LS on multi-band test cases for FR2 UEs”, RAN5 (Apple)

- R5-195527, “Reply LS to ‘LS on multi-band test cases for FR2 UEs’.”, GCF-CAG

- R5-198898, “Reply LS to 3GPP RAN5 regarding additional MB relaxation information (R5-195451)”, PTCRB / PVG