3GPP TSG-RAN WG4 Meeting #94-e	R4-2001265

# Online, 24th February – 6th March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0482 | rev |  | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 on one shot timing adjustment requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The one shot timing adjustment requirements for UE uplink transmission was further improved in the last meeting, but there are still open issues. The threshold value H has not been finalized yet. The definition of T1 and T2 is not correct and ambiguous. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The threshold value of H is proposed - The definition of T1 and T2 are corrected - “x Tc” is added in the formula. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The one shot timing adjustment requirements for UE uplink transmission are not finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change #1 >

7.1	UE transmit timing

7.1.1	Introduction

The UE shall have capability to follow the frame timing change of the reference cell in connected state. The uplink frame transmission takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in PTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the PTAG. For serving cell(s) in STAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the STAG. UE initial transmit timing accuracy, gradual timing adjustment requirements and one shot timing adjustment requirements are defined in the following requirements.

7.1.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS or it is the PRACH transmission.

The UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell. NTA for PRACH is defined as 0.

(in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA for other channels is not changed until next timing advance is received. The value ofdepends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). is defined in Table 7.1.2-2.

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals ( kHz) | SCS of uplink signals ( kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.1.2-2: The Value of

| Frequency range and band of cell used for uplink transmission | (Unit: TC) |
| --- | --- |
| FR1 FDD band without LTE-NR coexistence case or FR1 TDD band without LTE-NR coexistence case | 25600 (Note 1) |
| FR1 FDD band with LTE-NR coexistence case | 0 (Note 1) |
| FR1 TDD band with LTE-NR coexistence case | 39936 (Note 1) |
| FR2 | 13792 |
| Note 1: The UE identifies based on the information n-TimingAdvanceOffset as specified in TS 38.331 [2]. If UE is not provided with the information n-TimingAdvanceOffset, the default value of is set as 25600 for FR1 band. In case of multiple UL carriers in the same TAG, UE expects that the same value of n-TimingAdvanceOffset is provided for all the UL carriers according to clause 4.2 in TS 38.213 [3] and the value 39936 of can also be provided for a FDD serving cell. Note 2: Void |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell except when the timing advance in clause 7.3 is applied.

Table 7.1.2-3: void

7.1.2.1	Gradual timing adjustment

When the transmission timing error between the UE and the reference timing exceeds Te then the UE is required to adjust its timing to within Te. The reference timing shall be  before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200 ms.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 7.1.2.1-1.

Table 7.1.2.1-1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2 | 60 | 2.5*64*Tc | 2.5*64*Tc |
|  | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

7.1.2.2	One shot timing adjustment

When the magnitude of the T exceeds H then the UE shall adjust its transmission timing in one adjustment only once provided that the following conditions are met at the UE. Otherwise when the the magnitude of the T ≤ H then the UE shall adjust its transmission timing according to the rules defined in clause 7.1.2.1.

-	SSB_RP and SSB Ês/Iot according to Annex B.2.6.1 for a corresponding operating Band,

The UE transmit timing immediately after applying the one shot timing adjustment shall be: T2-(NTA+NTA offset)TC+2(T1-T2) and the UE is required to adjust its timing to within Te. After applying the one shot timing adjustment the UE shall adjust its transmission timing according to the rules defined in clause 7.1.2.1.

Where: T= T1-T2

-	T1 is the previous reception time at the UE,

-	T2 is the current reception time to be used at the UE,

-	H is defined in table 7.1.2.2-1.

Table 7.1.2.2-1: The value of H

| Frequency Range | SCS of uplink signals (KHz) | H |
| --- | --- | --- |
| 1 | 15 | 20*64*Tc |
|  | 30 | 10*64*Tc |
|  | 60 | 5.5*64*Tc |
| 2 | 60 | 5.5*64*Tc |
|  | 120 | 2.5*64*Tc |

< End of change #2 >