3GPP TSG-RAN WG4 Meeting #94-e				  											R4-2001288 Online, 24th Feb. 2020 – 6th Mar. 2020

Source: 	Nokia, Verizon

Title: 		TP for 37.716-11-11 to introduce DC_48_n66

Agenda Item: 			9.3.2 [DC_R16_1BLTE_1BNR_2DL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.716-11-11 to introduce DC_48_n66.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

### 6.1.x	DC_48_n66

#### 6.1.x.1	Operating bands for DC

Table 6.1.x.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_48_n66 | 48 | n66 | No |

#### 6.1.x.2	Configuration for DC

Table 6.1.x.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_48A_n66A | DC_48A_n66A | 48A | n66A |

#### 6.1.x.3	Maximum output power for DC

The maximum output power for the uplink EN-DC configuration for DC_66A_n48A, is already specified in TS 38.101-3 Table 6.2B.1.3-1 and can be reused for DC_48A_n66A.

Table 6.1. x.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_48A_n66A | 23 | +2/-3 |

#### 6.1.x.4	Spurious emission band UE co-existence for DC

The requirements for the uplink EN-DC configuration for DC_66A_n48A, is already specified in TS 38.101-3 Table 6.5B.3.3.2-1 and can be reused for DC_48A_n66A.

Table 6.1. x.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_48_n66 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |

#### 6.1.x.5	MSD analysis for DC

MSD analysis have been conducted for DC_66A_n48A and can be reused for DC_48_n66.

#### 6.1.x.6	∆TIB and ∆RIB values

For DC_48_n66, the TIB,c and RIB,c values are reused from DC_66A_n48A already specified in TS 38.101-3.

Table 6.1.x.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48_n66 | 48 | 0.8 |
|  | n66 | 0.6 |

Table 6.1.x.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_48_n66 | 48 | 0.5 |
|  | n66 | 0.2 |

#### 6.1.x.7	MSD

Based on the co-existence studies for DC_66A_n48A MSD have been defined in TS 38.101-3 Table 7.3B.2.3.5.1-1 and can be reused for DC_48_n66.

Table 6.1. x.7-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n48A | 66 | 1735 | 5 | 25 | 2135 | 4 | IMD5 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | N/A |

*********************** End of the TP ***************************************************

# 3	References