3GPP TSG-RAN WG4 Meeting#94-e                	R4-2001334

E-meeting, February 24th – March 6th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Correction to Active TCI state list update delay

Agenda item:	6.10.8.4

Document for:	Approval

Introduction

RAN4 has defined UE requirement for TCI state switch delays in 38.133 section 8.10 [1]. During the work a number of corrections have been discussed and agreed. However, it seems one requirement needs to be corrected further.

Discussion

RAN4 has defined for when a TCI state can be considered as known TCI state and unknown TCI state in section 8.10.2. The detailed UE requirements then depend on the TCI state condition.

TCI state switch delay requirements have been defined for:

- MAC based PDCCH TCI state activation in section 8.10.3

- MAC based active TCI state set list update in section 8.10.6

- DCI based TCI state switch in section 8.10.4

- RRC based TCI state switch delay in section 8.10.5

Looking further at the requirement related MAC based active TCI state set list requirements in section 8.10.6, these requirements are supposed to cover the UE requirements when the network configured the UE with a new set of TCI states for PDSCH scheduling.

However, the current requirements are referring to the UE requirements for receiving control information in 38.214:

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive PDCCH to schedule PDSCH with the new target TCI state no later than slot n+ THARQ +3ms +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3.

Requirements for update of the PDCCH TCI state (control information) are covered in section 8.10.3. Requirements for updating the active TCI state set for PDSCH should be based on the RAN1 agreement as listed in 38.214 section 5.1.5:

The UE receives an activation command, as described in subclause 6.1.3.14 of [10, TS 38.321], used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

Hence, we have identified two aspects to address:

- Timing requirement is incorrect.

- Current wording is misleading.

Both should be updated.

Timing Requirement update

The basic UE timing requirements needs to be aligned with the RAN1 requirements. Hence it needs to be updated accordingly.

Update UE timing requirement in section 8.10.6 to be aligned with RAN1 requirement.

The timing should be updated to n+3Nslotsubframe,µ

Update the timing requirement to n+3Nslotsubframe,µ

Update of requirement wording

Current wording it misleading as the UE in this case is not configured with a new TCI state which will be used for scheduling. The UE is configured with a new active TCI state set which the network may use for PDSCH scheduling in the future. However, no matter if network uses either of the configured active TCI states in the active TCI state set is up to network and indicated in the DCI format (if used).

What is the UE requirement is when the UE at latest shall have apply the mapping between the newly configured active TCI state set and the codepoints of the DCI field ‘Transmission Configuration Indication’.

Update the current requirement text to reflect the actual delay requirement.

Text Proposal

We propose following text to section 8.10.6:

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE for TCI States Activation/Deactivation for UE-specific PDSCH update at slot n, UE shall be able to receive PDCCH to schedule PDSCH with the new DCI indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' no later than n + THARQ + 3Nslotsubframe,µ + TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3.

Conclusion

RAN4 has defined UE requirement for TCI state switch delays in 38.133 section 8.10 [1]. During the work a number of corrections have been discussed and agreed. However, it seems one requirement needs to be corrected further.

- Update UE timing requirement in section 8.10.6 to be aligned with RAN1 requirement.

- Update the timing requirement to n+3Nslotsubframe,µ

- Update the current requirement text to reflect the actual delay requirement.

References

- 38.133, Release 15.8.0.