3GPP TSG-RAN WG4 Meeting #94-e	R4-2001356

Electronic Meeting, 24 February – 6 March 2020

Agenda Item:	8.17.1.4

Source:	Ericsson

Title:	Beam management for high speed train scenario

Document for:	Discussion

# 1	Introduction

RAN4#93 agreed with the way forward on RRM for NR high speed train scenario [1], which also includes the agreement on beam management as follow:

Beam management - CBD based on SSB/CSI-RS - FFS on whether Rel-15 CBD requirements based on SSB/CSI-RS can be reused for NR HST - BFD based on SSB/CSI-RS - Rel-15 BFD requirements based on SSB/CSI-RS can be reused for NR HST - FFS whether 1.5x relaxation factor shall be used - L1-RSRP based SSB - FFS on whether Rel-15 requirements (measurement delay, measurement accuracy) can be reused for NR HST - L1-RSRP based on CSI-RS - FFS on whether Rel-15 requirements (measurement delay, measurement accuracy) can be reused for NR HST

# 2	Beam failure detection

In Rel-15, the evaluation period of the beam failure detection is specified based on the evaluation period of RLM in-synch. According to WF [1], RAN4 agreed to apply Rel-15 BFD requirements to the HST scenario, but the remaining issue is whether to keep the relaxation factor of 1.5 in the case of DRX<=320ms.

Table 8.5.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P) TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(5 P) TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.5.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, [MBFD P] TCSI-RS) |
| DRX cycle ≤ 320ms | Max(50, [1.5 × MBFD P] Max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | [MBFD P] TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

As we proposed in [2], we prefer to remove the relaxation factor considering the UE velocity of 500km/h.

Proposal 1: 1.5x relaxation factor for DRX <= 320ms is not kept for BFD L1 evaluation period in HST.

# 3	L1-RSRP measurements

If we consider 500km/h at the carrier frequency of 3.6GHz, the maximum Doppler frequency becomes 1,666Hz. We studied the L1-RSRP measurement accuracy based on the parameters used to set the L1-RSRP measurement and accuracy requirements in Rel-15 [3].

## 3.1	SSB based L1-RSRP measurements

Table 1 shows the simulation parameters for SSB based L1-RSRP measurements in HST scenario. We also investigated 1, 3, and 5 SSB samples for HST condition. Note in Rel-15, L1-RSRP measurement requirements are defined based on 1 SSB sample [3].

Table 1	Simulation parameters for SSB based L1-RSRP measurements in HST scenario.

| Parameters | Values |
| --- | --- |
| SCS | 15kHz, 30kHz |
| Side condition (SNR) | -3dB |
| Number of samples (M) | 1 (Baseline), 3, 5 |
| Propagation condition | TDLA30, TDLC300 |
| Maximum Doppler frequency | 1000Hz (=300km/h at 3.6GHz), 1666Hz (=300km/h at 3.6GHz) |

Table 2	SSB based L1-RSRP estimation simulation results.

| SCS | Samples | AWGN [4] | TDLA30-1000 | TDLA30-1666 | TDLC300-1000 | TDLC300-1666 |
| --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 1 | 1.2 (Rel-15) | 1.39 | 1.53 | 1.44 | 1.48 |
|  | 3 |  | 0.71 | 0.71 | 0.79 | 0.78 |
|  | 5 |  | 0.54 | 0.54 | 0.55 | 0.54 |
| 30kHz | 1 | 0.8 (Rel-15) | 0.88 | 0.9 | 0.97 | 0.99 |
|  | 3 |  | 0.46 | 0.49 | 0.58 | 0.58 |
|  | 5 |  | 0.36 | 0.4 | 0.42 | 0.43 |

Table 2 shows our simulation result of the SSB based L1-RSRP estimation. We put the maximum of absolute value of 5%-ile error and 95%-ile error values as the same metric as [4]. For reference we put our results in the static condition, which is used to specify the L1-RSRP measurement accuracy requirements in Rel-15.

If we compare the fading channel case (Doppler=1000Hz or 1666Hz) with AWGN case, it is not observed the accuracy degradation due to the fading channel and/or higher Doppler frequency. With this observation we propose to reuse the Rel-15 SSB based L1-RSRP measurement/accuracy requirements for NR HST.

Observation 1: SSB based L1-RSRP measurement accuracy is not impacted with the Rel-16 NR high speed train scenario.

Proposal 2: Rel-15 SSB based L1-RSRP measurement/accuracy requirements can be reused for Rel-16 NR HST.

According to TS38.133, Rel-15 SSB based L1-RSRP measurement period apples the relaxation factor of 1.5 for the case of DRX cycle <= 320ms. As we discussed in RLM and BFD, we prefer to remove the relaxation factor considering the UE velocity.

Table 3	SSB based L1-RSRP measurement period in Rel-15 NR.

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Proposal 3: 1.5x relaxation factor for DRX cycle <= 320ms is not kept for L1-RSRP measurement period in HST.

## 3.2	CSI-RS based L1-RSRP measurements

Table 4 shows the simulation parameters for CSI-RS based L1-RSRP measurements in HST scenario. Like SSB-based L1-RSRP, we investigated 1, 3, and 5 CSI-RS samples and larger BW for HST scenario. Note Rel-15 assumed 1 CSI-RS sample with 48PRBs [3].

Table 4	Simulation parameters for CSI-RS based L1-RSRP measurements in HST scenario.

| Parameters | Values |
| --- | --- |
| SCS | 15kHz, 30kHz |
| Side condition (SNR) | -3dB |
| Density (D) | 3 |
| Number of samples (M) | 1 (Baseline), 3, 5 |
| Number of PRBs | 48 (Baseline), 96 |
| Propagation condition | TDLA30, TDLC300 |
| Maximum Doppler frequency | 1000Hz, 1666Hz |

Table 5	CSI-RS based L1-RSRP estimation simulation results.

| SCS | Samples | PRB | AWGN [4] | TDLA30-1000 | TDLA30-1666 | TDLC300-1000 | TDLC300-1666 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 1 | 48 | 0.77 (Rel15) | 1.78 | 1.77 | 1.77 | 1.67 |
|  |  | 96 |  | 0.98 | 0.97 | 0.91 | 0.87 |
|  | 3 | 48 |  | 0.83 | 0.83 | 0.9 | 0.93 |
|  |  | 96 |  | 0.48 | 0.52 | 0.48 | 0.5 |
|  | 5 | 48 |  | 0.62 | 0.64 | 0.79 | 0.73 |
|  |  | 96 |  | 0.41 | 0.37 | 0.37 | 0.38 |
| 30kHz | 1 | 48 | 0.55 (Rel15) | 1.01 | 1.16 | 1.18 | 1.26 |
|  |  | 96 |  | 0.59 | 0.65 | 0.59 | 0.62 |
|  | 3 | 48 |  | 0.54 | 0.57 | 0.66 | 0.7 |
|  |  | 96 |  | 0.34 | 0.35 | 0.35 | 0.38 |
|  | 5 | 48 |  | 0.42 | 0.46 | 0.52 | 0.54 |
|  |  | 96 |  | 0.29 | 0.3 | 0.27 | 0.27 |

Table 5 shows our simulation results of the CSI-RS based L1-RSRP estimation. We set the same metric as SSB based L1-RSRP. Unlike SSB-based L1-RSRP results, it is observed the measurement accuracy degradation due to fading and/or higher Doppler frequency, i.e., from 0.77dB to 1.67-1.77dB in the case of SCS=15kHz.

Observation 2: CSI-RS based L1-RSRP accuracy is degraded significantly due to the Rel-16 NR high speed train scenario.

It is also observed that the increase of CSI-RS samples in time domain and/or frequency domain can maintain the same accuracy as the static condition. However, more CSI-RS samples in time domain is not good solution because it increases the overhead and reduces the network capacity. We also don’t want to assume wider bandwidth for CSI-RS also because it restricts the network deployment.

Table 6 shows Rel-15 CSI-RS based L1-RSRP measurement period. As we discussed so far, the relaxation factor of 1.5 is applied for DRX cycle <= 320ms. Considering the very high UE speed, we prefer to remove this factor as well as SSB based L1-RSRP measurement period requirements.

Proposal 4: Rel-15 CSI based L1-RSRP measurement requirements can be reused for Rel-16 NR HST, but 1.5x relaxation factor for DRX cycle <= 320ms is not kept for L1-RSRP measurement period.

Proposal 5: RAN4 investigate the impact of CSI-RS based L1-RSRP measurement accuracy performance, if necessary, for Rel-16 HST scenario.

Table 6	CSI-RS based L1-RSRP measurement period in Rel-15.

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

# 4	Candidate beam detection

Candidate beam detection (CBD) is the procedure UE detects new beam(s) whose L1-RSRP exceeds the signalled threshold. Table 7 and Table 8 show the evaluation periods for FR1 SSB based CBD and FR1 CSI-RS based CBD, respectively, specified in Rel-15 TS38.133. As it is observed, the L1-RSRP measurements for CBD is based on 3 samples for both SSB and CSI-RS. According to our simulation results in Table 2 and Table 5, it is not observed that the accuracy degradation due to higher Doppler shift in the case of 3 samples. Therefore we propose Rel-15 Candidate beam detection requirements can be reused for Rel-16 NR HST.

Table 7	Evaluation period for FR1 SSB based CBD in Rel-15.

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Ceil(3 P) TSSB |
| DRX cycle > 320ms | Ceil(3 P) TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8	Evaluation period for FR1 CSI-RS based CBD in Rel-15.

| Configuration | TEvaluateC_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(MCBD P) TCSI-RS) |
| DRX cycle > 320ms | Ceil(MCBD P) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

MCBD = 3, if the CSI-RS resource configured in the set  is transmitted with Density = 3.

Proposal 6: Rel-15 Candidate beam detection requirements can be reused for Rel-16 NR HST.

# 5	Summary

Proposal 1: 1.5x relaxation factor for DRX <= 320ms is not kept for BFD L1 evaluation period in HST.

Observation 1: SSB based L1-RSRP measurement accuracy is not impacted with the Rel-16 NR high speed train scenario.

Proposal 2: Rel-15 SSB based L1-RSRP measurement/accuracy requirements can be reused for Rel-16 NR HST.

Proposal 3: 1.5x relaxation factor for DRX cycle <= 320ms is not kept for L1-RSRP measurement period in HST.

Observation 2: CSI-RS based L1-RSRP accuracy is degraded significantly due to the Rel-16 NR high speed train scenario.

Proposal 4: Rel-15 CSI based L1-RSRP measurement requirements can be reused for Rel-16 NR HST, but 1.5x relaxation factor for DRX cycle <= 320ms is not kept for L1-RSRP measurement period.

Proposal 5: RAN4 investigate the impact of CSI-RS based L1-RSRP measurement accuracy performance, if necessary, for Rel-16 HST scenario.

Proposal 6: Rel-15 Candidate beam detection requirements can be reused for Rel-16 NR HST.

# References

- R4-1915887, “WF on RRM for NR HST”, CMCC.

- R4-2001355, “RLM for NR HST”, Ericsson.

- R4-1904760, “Simulation summary of L1-RSRP accuracy”, Ericsson.

- R4-1904820, “Way forward on L1-RSRP accuracy requirements”, Ericsson, Nokia, CMCC.