3GPP TSG-RAN WG4 Meeting #94-e	R4-2001460

# Electronic Meeting, 24th Feb - 6th Mar, 2020

Title: 	Discussion on the UL timing adjustment

Source: 	Huawei, HiSilicon

Agenda item:	8.17.2.2.3

Document for:	Discussion

Background

During RAN4#93 meeting, way forward [1] for NR Rel-16 BS demodulation was approved. In this contribution, we share our views about the UL timing adjustment for NR Rel-16 HST.

Discussion

SRS transmission

As per WF [1], the agreements related to SRS transmission for UL timing adjustment are extracted as shown below:

- SRS transmission (optional) - FDD - Slot #1 in radio frames - TDD - The last symbol in the special slot (Further check is needed) - 15kHz SCS: last symbol in slot #3 in radio frames - 30kHz SCS: last symbol in slot #7 in radio frames

In LTE, comparing TS 36.104 [2] and TS 36.141 [3], we can find out that there is inconsistence in two specs about SRS transmission for UL timing adjustment, as Table 2.1-1 shows below:

Table 2.1-1 Inconsistence in TS 36.104 and TS 36.141

| TS 36.104 | TS 36.141 |
| --- | --- |
| Subframes in which sounding RS is transmitted (Note 1) - For FDD: subframe #1 in radio frames - For TDD: UpPTS in subframe #1 in radio frames Note 1. The transmission of SRS is optional. | Subframes in which sounding RS is transmitted (Note 1) - For FDD: subframe #1 in radio frames - For TDD: UpPTS in each radio frame Note 1. The transmission of SRS is optional. |

Figure 2.1-1 SRS and PUSCH configuration corresponded to TS 36.104 and TS 36.141

Figure 2.1-1 shows the difference between two specs clearly. The issue is whether 1 or 2 special slots is used for SRS transmission in a radio frame. As per contribution [4], we believe that version in TS 36.104 is recognized by the companies at that time.

Observation 1: There is inconsistence in TS 36.104 and TS 36.141 for SRS configuration, and TS 36.141 needs to be corrected.

Testing model for UL timing adjustment in NR is align with LTE. As per TS 36.104, the time difference between the reference timing and the first tap is according the following equation:

In the last meeting, it comes to an agreement that A and ∆ω is scaled for 30 kHz SCS. The following figure shows TA value changes for 15 kHz and 30 kHz SCS for moving UE.

Figure 2.1-2 TA value changes for moving UE

As we can see in Figure 2.1-2, the time interval of two adjacent TA command is about 800ms to 2600ms for 15kHz SCS and about 800ms to 5460ms for 30kHz SCS. It is to say that adjustment of TA value is not frequently, transmitting SRS signal one slot per radio frame is sufficient in current model.

Observation 2: Adjustment of TA value is not frequently, transmitting SRS signal one slot per radio frame is sufficient in current model.

Proposal 1: Transmit SRS (optional) for uplink timing advance requirement in:

  - FDD

    - Slot #1 in radio frames

  - TDD

    - The last symbol in the special slot

      - 15kHz SCS: last symbol in slot #3 in radio frames

      - 30kHz SCS: last symbol in slot #7 in radio frames

Other parameters

There are also some parameters needed to be determined, we list them in the following table with our preferable value.

Table 2.2-1 Parameters needed to be determined

| Parameter | Value |
| --- | --- |
| DMRS type | type1 |
| symbols length | 14 for both PUSCH type A and B |
| start symbol index | 0 for both PUSCH type A and B |
| resource allocation type | type 1 |
| SRS bandwidth configuration | BSRS=0, CSRS=11 for 40RBs BSRS=0, CSRS=21 for 80RBs |
| SRS transmission comb | 2 |

Proposal 2: Adopt parameters in Table 2.2-1 for UL timing adjustment.

Simulation assumption

As per above discussion and agreements before, we can summarize the simulation assumption as the following Table 2.3-1.

Table 2.3-1 Simulation assumption

| Parameter | value |
| --- | --- |
| Transform precoding | Disabled |
| Antenna configuration | 1x2 |
| Number of layers | 1 |
| SCS/CBW | 15kHz: 10MHz 30kHz: 40MHz |
| Reference signal | DMRS type 1 with 1+1+1 |
| PUSCH mapping type | type A and Type B |
| l0 | 2 |
| Start symbol index | 0 |
| Symbols length | 14 |
| MCS | 16 |
| Carrier frequency | 15kHz: 2.1GHz 30kHz: 3.6GHz |
| Propagation condition | Scenario Y 15kHz: A=10us, △ω=0.13s-1 30kHz: A=5us, △ω=0.26s-1 |
| Allocated RBs for PUSCH | 15kHz SCS/10MHz CBW: 25 continuously allocated RBs for each UE Starting PRB index: Moving UE 0, Stationary UE 25 30kHz SCS/40MHz CBW: 50 continuously allocated RBs for each UE Starting PRB index: Moving UE 0, Stationary UE 50 |
| Allocated RBs for SRS | 15kHz SCS/10MHz CBW: SCS:40 contiguously allocated RBs Starting PRB index: 0 30kHz SCS/40MHz CBW: SCS:80 contiguously allocated RBs Starting PRB index: 0 |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions | 4 |
| TDD configuration | 15kHz: 3D1S1U, S=10D:2G:2U 30kHz: 7D1S2U, S=6D:4G:4U |
| PUSCH transmission | FDD: Slots #0, #2, #4, #6, and #8 in radio frames TDD 15kHz: slots #4 and #9 in radio frames TDD 30kHz: slots #8, #9, #18 and #19 in radio frames |
| SRS transmission (optional) | FDD: Slot #1 in radio frames TDD 15kHz: last symbol in slot #3 in radio frames TDD 30kHz: last symbol in slot #7 in radio frames |
| SRS bandwidth configuration | BSRS=0, CSRS=11 for 40RBs BSRS=0, CSRS=21 for 80RBs |
| SRS transmission comb(optional) | 2 |
| resource allocation type | type 1 |
| Testing metric | SNR@70% of maximum throughput for the moving UE |

Proposals

In this contribution, we share our views on UL timing adjustment, and our observations and proposals are:

Observation 1: There is inconsistence in TS 36.104 and TS 36.141 for SRS configuration, and TS 36.141 needs to be corrected.

Observation 2: Adjustment of TA value is not frequently, transmitting SRS signal one slot per radio frame is sufficient in current model.

Proposal 1: Transmit SRS (optional) for uplink timing advance requirement in:

  - FDD

    - Slot #1 in radio frames

  - TDD

    - The last symbol in the special slot

      - 15kHz SCS: last symbol in slot #3 in radio frames

      - 30kHz SCS: last symbol in slot #7 in radio frames

Proposal 2: Adopt parameters in Table 2.2-1 for UL timing adjustment.

Reference

- R4-1915886, Way forward on NR HST PUSCH demodulation requirements, RAN4#93, Huawei, HiSilicon

- TS 36.104, Base Station (BS) radio transmission and reception

- TS 36.141, Base Station (BS) conformance testing

- R4-090151, Consideration on UL timing adjustment  test case for TDD, RAN4#49bis, CATT