3GPP TSG-RAN WG4 Meeting #94-e	R4-2001473

# Electronic meeting, 24th Feb - 6th Mar, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | 0156 | rev | -- | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.104: Introduction of PRACH demodulation requirements for NR HST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-Perf |  |  |  |  |  | Date: |  |  | 2020-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR introduces PRACH performance requirements of high speed train scenario including UE velocity of up to 350km/h and 500km/h. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new tables for introducing PRACH performance requirements of high speed train scenario of two types of speed.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirement part of PRACH under high speed train condition is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2, A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-1, TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first change>

8.4.2	PRACH detection requirements

8.4.2.1	General

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. For AWGN and TDLC300-100, a timing estimation error occurs if the estimation error of the timing of the strongest path is larger than the time error tolerance given in Table 8.4.2.1-1.

The requirements for high speed mode restricted set type A (table 8.4.2.2-4) and high speed mode restricted set type B (table 8.4.2.2-5) are only valid for the base stations supporting high speed mode restricted set type A and restricted set type B respectively.

Table 8.4.2.1-1: Time error tolerance for AWGN and TDLC300-100

| PRACH preamble | PRACH SCS (kHz) | Time error tolerance |  |
| --- | --- | --- | --- |
|  |  | AWGN | TDLC300-100 |
| 0 | 1.25 | 1.04 us | 2.55 us |
| A1, A2, A3, B4, C0, C2 | 15 | 0.52 us | 2.03 us |
|  | 30 | 0.26 us | 1.77 us |

The test preambles for normal mode are listed in table A.6-1 and the test parameter msg1-FrequencyStart is set to 0. The test preambles for high speed mode restricted set type A are listed in A.6-3 and the test preambles for high speed mode restricted set type B are listed in A.6-4.

8.4.2.2	Minimum requirements

The probability of detection shall be equal to or exceed 99% for the SNR levels listed in Tables 8.4.2.2-1 to 8.4.2.2-3.

Table 8.4.2.2-1: PRACH missed detection requirements for Normal Mode, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | AWGN | 0 | [-14.5] |
|  |  | TDLC300-100 Low | 400 Hz | [-6.6] |
|  | 4 | AWGN | 0 | [-16.7] |
|  |  | TDLC300-100 Low | 400 Hz | [-11.9] |
|  | 8 | AWGN | 0 | [-18.9] |
|  |  | TDLC300-100 Low | 400 Hz | [-15.8] |

Table 8.4.2.2-2: PRACH missed detection requirements for Normal Mode, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | AWGN | 0 | [-9.3] | [-12.6] | [-14.2] | [-16.8] | [-6.3] | [-12.5] |
|  |  | TDLC300-100 Low | 400 Hz | [-2.1] | [-4.8] | [-6.6] | [-8.8] | [0.8] | [-4.9] |
|  | 4 | AWGN | 0 | [-11.6] | [-14.3] | [-16.0] | [-19.0] | [-8.7] | [-14.1] |
|  |  | TDLC300-100 Low | 400 Hz | [-7.3] | [-10.3] | [-11.7] | [-13.8] | [-4.3] | [-10.2] |
|  | 8 | AWGN | 0 | [-13.8] | [-16.7] | [-18.2] | [-21.2] | [-11.1] | [-16.6] |
|  |  | TDLC300-100 Low | 400 Hz | [-11.0] | [-13.9] | [-15.2] | [-17.3] | [-8.1] | [-13.9] |

Table 8.4.2.2-3: PRACH missed detection requirements for Normal Mode, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | AWGN | 0 | -9.1 | -12.0 | -13.8 | -16.5 | -6.1 | -11.9 |
|  |  | TDLC300-100 Low | 400 Hz | -2.8 | -5.7 | -7.4 | -9.9 | 0.1 | -5.6 |
|  | 4 | AWGN | 0 | -11.4 | -14.2 | -15.9 | -19.0 | -8.6 | -14.1 |
|  |  | TDLC300-100 Low | 400 Hz | -7.2 | -10.4 | -12.0 | -14.5 | -4.5 | -10.4 |
|  | 8 | AWGN | 0 | -13.7 | -16.6 | -18.1 | -21.1 | -11.0 | -16.5 |
|  |  | TDLC300-100 Low | 400 Hz | -10.7 | -13.7 | -15.1 | -17.6 | -7.8 | -13.7 |

Table 8.4.2.2-4: PRACH missed detection requirements for High speed Mode restricted set type A, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |
|  | 4 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |
|  | 8 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |

Table 8.4.2.2-5: PRACH missed detection requirements for High speed Mode restricted set type B, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 2334 Hz | TBD |
|  | 4 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 2334 Hz | TBD |
|  | 8 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 2334 Hz | TBD |

<END of first change>

<Start of second change>

A.6	PRACH Test preambles

Table A.6-1: Test preambles for Normal Mode in FR1

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | 1.25 | 13 | 22 | 32 |
| A1, A2, A3, B4, C0, C2 | 15 | 23 | 0 | 0 |
|  | 30 | 46 | 0 | 0 |

Table A.6-2: Test preambles for Normal Mode in FR2

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| A1, A2, A3, B4, C0, C2 | 60 | 69 | 0 | 0 |
|  | 120 | 69 | 0 | 0 |

Table A.6-3: Test preambles for High speed Mode restricted set type A

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | 1.25 | 15 | 384 | 0 |

Table A.6-4: Test preambles for High speed Mode restricted set type B

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | 1.25 | 15 | 30 | 30 |

<End of second change>