3GPP TSG-RAN WG4 Meeting #94-e	R4-2001488

# Electronic Meeting, 24th Feb – 6th Mar, 2020

Title: 	Discussion on URLLC BS performance requirements for low latency

Source: 	Huawei, HiSilicon

Agenda item:	8.9.1.3

Document for:	Discussion

Background

According to the WF[1] from the last meeting, when considering the BS demodulation requirements for low latency, the demodulation requirements PUSCH mapping Type B was agreed to be verified and the UL transmission with grant free was FFS.

- Introduce PUSCH demodulation requirements to verify the support of PUSCH mapping Type B with non-slot configured with fewer symbols than Rel-15

- FFS requirements for UL transmission with grant free/UL configured grant

In this paper, the PUSCH demodulation requirements for mapping Type B is introduced, and UL transmission with grant free is discussed.

Discussion

PUSCH mapping Type B

As PUSCH mapping Type B supports [1,…,14] symbols and 14-symbol was defined in Rel-15. For URLLC service, non-slot configuration is considered. The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. Table 2.1-1 illustrates the key parameters for PUSCH mapping Type B test.

As this requirement is defined for URLLC service and the URLLC information normally carried by fewer symbols, we would like to propose symbol length of 4 to verify mapping Type B. TS38.104 has already been defined the mapping Type B with symbol length of 14, in order to make minimal changes to the existing specification, some parameters in TS38.104 can be reused. The purpose of this requirements is to verify the mapping Type B, so only PUSCH without transform precoding is defined and typical settings of SCS and bandwidth for FDD & TDD are configured.

Table 2.1-1 Key parameters for PUSCH mapping Type B test

| Parameter |  | Value |
| --- | --- | --- |
| Fraction of maximum throughput |  | 70% |
| Transform precoding |  | Disabled |
| Frequency range |  | FR1 |
| Duplex mode |  | FDD/TDD |
| SCS and BW |  | FDD: 15KHz—5MHz TDD: 30KHz--10MHz |
| Number of Tx |  | 2 |
| Number of Rx |  | 2 |
| Antenna configuration |  | ULA low |
| Uplink-downlink allocation for TDD |  | 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| MCS Table |  | Table 3, MCS 5 |
| Propagation condition |  | TDLB100-400 Low TDLC300-100 Low |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | 0 |
| Time domain resource assignment | PUSCH mapping type | Type B |
|  | Start symbol | 0 |
|  | Allocation length | 4 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |

Proposal 1: To verify mapping Type B, we propose to use symbol length is 4 and start symbol is 0.

Proposal 2: 15 KHz SCS is configured for FDD mode, and 30KHz SCS is configured for TDD mode.

Proposal 3: UL-DL pattern ‘7D1S2U (S=6D+4G+4U)’ is used for TDD.

Proposal 4: We propose the number of Tx antennas is 2 and the number of Rx antennas is 2.

Proposal 5: Only requirements for PUSCH with transform precoding disabled is defined.

Proposal 6: We propose to use MCS5 from MCS table 3.

UL transmission with grant free

For uplink grant free transmission, the uplink resource is reserved for uplink transmission. There is no change of reference signals and uplink channel structure. Although the BS may be required to detect if there is data transmitted, there would be no need to introduce the new demodulation performance requirements to verify uplink grant free transmission.

Proposal 7: There is no need to introduce the new demodulation performance requirements to verify uplink grant free transmissions.

Proposals

In this contribution, we discussed PUSCH mapping Type B and UL grant free. Following has been proposed:

Proposal 1: To verify mapping Type B, we propose to use symbol length is 4 and start symbol is 0.

Proposal 2: 15 KHz SCS is configured for FDD mode, and 30KHz SCS is configured for TDD mode.

Proposal 3: UL-DL pattern ‘7D1S2U (S=6D+4G+4U)’ is used for TDD.

Proposal 4: We propose the number of Tx antennas is 2 and the number of Rx antennas is 2.

Proposal 5: Only requirements for PUSCH with transform precoding disabled is defined.

Proposal 6: We propose to use MCS5 from MCS table 3.

Proposal 7: There is no need to introduce the new demodulation performance requirements to verify uplink grant free transmissions.

Reference

- R4-1915913, “Way forward on NR URLLC demodulation and CSI requirements”, Huawei, HiSilicon.