3GPP RAN WG4 Meeting #94-e                                      R4-2001549

Online, 24th February – 6th March 2020

Agenda item:	8.5.5.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRC connection mobility control for IAB

Document for:	Discussion

- Introduction

In last meeting RAN4 93, the RRM requirements about IAB MT are discussed. In this paper, we present the analysis about the RRM requirements of RRC connection mobility control in terms of RRC Re-establishment, random access and RRC release with redirection.

- Discussion

RRC Re-establishment

The RRC Re-establishment will be initiated when the IAB MT loses the RRC connection. The corresponding RRM requirement of UE in TS 38.133 define the time during for cell identification, system information acquisition, delay uncertainty in acquiring the PRACH occasion, etc. But for IAB MT, there are some IAB-specific features, which will impact the RRM requirements and should be considered when defining the related requirements for IAB MT.

As specified in TS 38.133, UE assume the SSB periodicity is no longer than 20 ms, or there is no requirements. For IAB MT, as defined in RAN1 [1], IAB MT may assume SSB periodicity for initial access is 16 radio frames (as opposed to the 2 radio frames assumption made by a Rel-15 UE). Thus, significant longer time for cell identification should be defined.

Observation 1: IAB MT may assume SSB periodicity for initial access is 16 radio frames (as opposed to the 2 radio frames assumption made by a Rel-15 UE).

Proposal 1: The time for cell identification in RRC re-establishment should be extended considering a longer SSB periodicity. There is no requirements if the SSB transmission periodicity is larger than 160ms.

For an IAB node, the maximum number of SMTC windows can be configured is 4 according to RAN1’s design as shown below:

RAN1#96 Agreement: - For IAB node discovery and measurement, the maximum number of SMTC windows that can be configured for an IAB node is 4. RAN1#97 Agreements: The configurable values of the parameters in the SSB reception configurations including SMTC for IAB node discovery and measurement are provided in the following - SMTC window periodicity: - 5ms, 10ms, 20ms, 40ms, 80ms, 160ms, 320ms, 640ms, 1280ms - SMTC window timing offset: - [0, ..., (number of subframes within SMTC window periodicity) – 1] - SMTC window duration: - [1, …, 5] (subframes) - FFS larger than 5 - List of physical cell IDs to be measured - [cell ID 0, …, cell ID M-1 ] - FFS value of M - SSB-ToMeasure Same as Rel-15 RAN1#97 Agreements: - For IAB node discovery and measurement - The maximum SMTC window duration is 5ms. - The maximum number of PCIs per SMTC (maxNrofPCIsPerSMTC) is 64.

It can be observed that the IAB node should support up to 4 SMTC configurations and the maximum periodicity is 1280ms. As mentioned in the observation 1, UE assumes that the SSB transmission periodicity is 160ms for initial access. By adopting the same principle in TS 38.133, the IAB MT is not required to successfully identify a cell when the SMTC window is larger than 160ms when the serving cell is in low SNR

Proposal 2: IAB MT is not required to successfully identify a cell when the SMTC window is larger than 160ms and the serving cell is in low SNR.

RRC release with redirection

As mentioned in observation 1, the same principle should be adopted in RRC release with redirection requirement that there is no requirement when the SSB transmission periodicity is larger than 160ms.

Proposal 3: There is no requirement when the SSB transmission periodicity is larger than 160ms.

- Conclusions

Observation 1: IAB MT may assume SSB periodicity for initial access is 16 radio frames (as opposed to the 2 radio frames assumption made by a Rel-15 UE).

Proposal 1: The time for cell identification in RRC re-establishment should be extended considering a longer SSB periodicity. There is no requirements if the SSB transmission periodicity is larger than 160ms.

Proposal 2: IAB MT is not required to successfully identify a cell when the SMTC window is larger than 160ms and the serving cell is in low SNR.

Proposal 3: There is no requirement when the SSB transmission periodicity is larger than 160ms.

References

[1] R1-1903809 “LS on initial access SSB periodicity assumption for IAB MT”