3GPP TSG-RAN WG4 Meeting #94-e                 	R4-2001688

Online, 24th Feb 2020 - 6th Mar 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Rel-16 HST BS demodulation PRACH remaining issues and simulation results

Agenda item:	8.17.2.2.2

Document for:	Discussion

Introduction

In the RAN4#93 meeting, the foundations of Rel-16 PUSCH HST BS demodulation performance requirements were mostly finalized and captured in a WF [1], which also requested the ideal and impaired results delivery for RAN4#94:

Simulation Results - Interested companies are encouraged to provide the ideal and impairment simulation results in the next meeting for NR HST PRACH requirement

Following the email discussions before the meeting, the only remaining point under discussion is the inclusion of TDL-C-100 requirements, for short PRACH formats.

In this contribution we discuss the final open issue on fading channels and provide our simulation results.

Short PRACH formats and fading channels

Following the email discussions before the meeting, the only remaining point under discussion is the inclusion of TDL-C-100 requirements, for short PRACH formats.

It is our understanding that RAN4#92bis decided to include TDL-C fading channel requirements with FO=400Hz, independently of the PRACH format [2]:

PRACH - Channel model - AWGN - TDL-C fading channel, Frequency offset is 400Hz - PRACH format - For 350km/h velocity, use PRACH format 0 - For 500km/h velocity, use PRACH format A2/B4/C2 - FFS if PRACH format 0 shall be used - Restricted set for PRACH format 0 - Both Type A and Type B - Test case applicability is FFS

In the WF of RAN4#93 [1] all companies then converged to TDL-C-100, when agreeing on Timing error tolerance:

Test Preamble Parameters - Timing error tolerance for AWGN and TDLC 300-100 - Reuse the time error tolerance for corresponding NR PRACH normal performance requirements

RAN4 to include TDL-C-100 fading channel requirements with frequency offset of 400Hz for all agreed PRACH preamble formats.

PRACH Simulation results

Unless otherwise stated in the following tables, the simulation setup follows the HST PRACH baseline as captured in [2] and [1]:

Table 1: PRACH HST baseline configuration

| Parameter | Value |  |
| --- | --- | --- |
|  | v = 350km/h | v = 500km/h |
| Frequency range | FR1 |  |
| Number of Tx | 1 |  |
| Number of Rx | 2Rx, 4Rx, 8Rx |  |
| Formats | 0 | A2, B4, C2 |
| SCS | 1.25kHz | 15kHz, 30kHz |
| Restricted set format 0 | {Set A, Set B} | N/A |
| (Ncs, logical sequence index, v) | Set A: (15, 384, 0) Set B: (15, 30, 30) | 15kHz: (23, 0, 0) 30kHz: (46, 0, 0) |
| Max timing error | AWGN: 1.04us TDLC300: 2.55us | 15kHz, AWGN: 0.52us 15kHz, TDLC300: 2.03us 30kHz, AWGN: 0.26us 30kHz, TDLC300: 1.77us |
| Maximum Doppler shift | Set A: {1340 Hz, 625 Hz} Set B: {2334 Hz, 625Hz} | 15kHz: 1740 Hz 30kHz: 3334 Hz |
| Carrier frequency (GHz) | Set A: 2.1 GHz Set B: 3.6 GHz | 15 kHz: 1.8 GHz 30 kHz: 3.6 GHz |
| Propagation condition | {AWGN with max Doppler shift from above, TDLC300-100 with FO=400Hz} |  |
| Testing metric | SNR @ 0.1% false alarm probability and 99% detection probability. |  |

PRACH 1.25kHz SCS Type A

Table 1: PRACH 1.25kHz SCS Type A

PRACH 1.25kHz SCS Type B

Table 1: PRACH 1.25kHz SCS Type B

  - PRACH 15kHz SCS

Table 1: PRACH 15kHz SCS

  - PRACH 30kHz SCS

Table 1: PRACH 30kHz SCS

Conclusion

In this contribution we discussed the final open issue on including fading channel requirements for short PRACH format and provided our simulation results for NR_HST Rel-16 PRACH BS demodulation performance requirements.

We have made the following proposals:

- RAN4 to include TDL-C-100 fading channel requirements with frequency offset of 400Hz for all agreed PRACH preamble formats.

- References

- R4-1915871, Way forward on NR HST PRACH requirements, Samsung, RAN4#93.

- R4-1912729, Way forward on NR HST PRACH demodulation, Samsung, RAN4#92-Bis.