3GPP TSG RAN WG4 Meeting #94-e	R4-2001847

Electronic Meeting, Feb. 24-March 06, 2020

Agenda Item:	8.1.4.3

Source: 	Ericsson

Title:	Analysis of open issues in RRC re-direction in NR-U

Document for:	Discussion

- Introduction

In RAN4#92bis meeting RAN4 agreed on the value of L1,max, which is the maximum number of SMTC occasions not available at the UE due to DL LBT failures [1]. However, the other parameter, L2,max, which is the maximum number of RACH occasions not available at the UE due to UL LBT failures, is subject to RAN2 agreements related to consistent UL failures. Another open issue is the UE behaviour when a number (L1) SMTC occasions unavailable at the UE exceeds L1,max.

In this paper we address the open issues related to the RRC connection release with redirection requirements.

# UE behaviour upon L1 exceeding L1,max

In [2], the following agreements regarding introduced of L1 and L1,max in the cell search requirements for RRC connection release with redirection were made:

Tidentify-NR is as follows:

| Frequency range (FR) of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (680 ms, ([11] + L1) x TDMTC) |

Maximum allowed value of L1 is TBD. L1 is the number of DMTC cycles not available at the UE.

In [3], the following agreements regarding L1,max were made:

- L1,max=[8] when TSMTC ≤ 40 ms

- L1,max=[4] when TSMTC > 40 ms

Under high DL LBT failures (e.g. under heavy traffic) the number of times the SMTC occasions becomes unavailable at the UE may exceed the maximum allowed value e.g. 8 when TSMTC = 40 ms. In RRC connection release procedure the cell search delay (Tidentify-NR) also includes SS-RSRP measurement, which in turn includes L1 filtering e.g. 5 or more samples. It is not feasible to obtain SS-RSRP measurement for the RRC release with redirection by averaging samples which are excessively separated in time. Therefore, it is important that the UE retart the measurement when L1 exceeds L1,max. It is also likely that under very heavy DL LBT failure the UE does not succed in obtaining the SSB of the target cell. In order to save UE power consumtion the UE should be allowed to abandon the identification of the cell on the indicated NR-U carrier if the UE cannot identify the target cell after restarting the measurement more than N number of times.

- Observation # 1: The cell search delay (Tidentify-NR) in RRC connection release procedure also includes L1 filtered SS-RSRP measurement.

- Proposal # 1: When L1 exceeds L1,max, the UE shall restart the identification of the target cell on the carrier configured for RRC connection release with redirection.

- Proposal # 2: Due to L1 exceeding L1,max, the UE is required to restart the identification of the target cell on the carrier configured for RRC connection release with redirection at least N times; where N =3. Upon exceeding N the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304.

# UE behaviour upon L2 exceeds L2,max

L2 is the number of PRACH occasions that are unavailable for PRACH transmission due to LBT failure and L2, max maximum allowed number of missed PRACH occasions (L2, max). According to the previous agreements in [1] the need for defining L2, max is subject to RAN2 decision on the UL LBT failure mechanism.

According to the latest version of the RAN2 running stage-2 CR to TS 38.300 the following has been captured regarding the UE behaviour upon reaching consistent uplink LBT failures. In RRC release with redirection only PCell is involved. As noted in the highlighted text that under consistent uplink LBT failure on RACH resources in PCell, the UE shall declare the RLF. RAN2 is defining the UE behaviour in the stage-3 MAC specification, TS 38.321.

When the UE detects consistent uplink LBT failures, it takes actions as specified in TS 38.321 [6]. The detection is per BWP and based on all uplink transmissions within this BWP. When consistent uplink LBT failures are detected on SCell(s), the UE reports this to the corresponding gNB (MN for MCG, SN for SCG) via MAC CE on a different serving cell other than the SCell (s) where the failures were detected. If no resources are available to transmit the MAC CE, a Scheduling Request (SR) can be transmitted by the UE. When consistent uplink LBT failures are detected on SpCell, the UE switches to another UL BWP with configured RACH resources on that cell, initiates RACH, and reports the failure via MAC CE. When multiple UL BWPs are available for switching, it is up to the UE implementation which one to select. For PSCell, if consistent uplink LBT failures are detected on all the UL BWPs with configured RACH resources, the UE declares SCG RLF and reports the failure to the MN via SCGFailureInformation. For PCell, if the uplink LBT failures are detected on all the UL BWP(s) with configured RACH resources, the UE declares RLF.

Also according to the latest version of the RAN2 running CR to TS 38.331, the procedure to configure the UE with paraneters comprising an UL LBT failure detection timer and a maximum number of UL LBT failure count, which trigger consistent uplink LBT failures is being specified, as shown below:

#### –	LBT-FailureRecoveryConfig

The IE LBT-FailureRecoveryConfig-r16 is used to configure the parameters used for detection of consistent uplink LBT failures for operation with shared spectrum channel access, as specified in TS 38.321 [3].

LBT-FailureRecoveryConfig information element

LBT-FailureRecoveryConfig-r16 ::=            SEQUENCE {

lbt-FailureInstanceMaxCount                         ENUMERATED {ffsValue}      OPTIONAL,   -- Need

lbt-FailureDetectionTimer                           ENUMERATED {ffsValue}      OPTIONAL,   -- Need

}

LBT-FailureRecoveryConfig field descriptions

lbt-FailureDetectionTimert Timer for consistent uplink LBT failure detection (see TS 38.321 [3]).

lbt-FailureInstanceMaxCount This field determines after how many consistent uplink LBT failure events the UE triggers uplink LBT failure recovery (see TS 38.321 [3]).

Based on the above analysis our conclusion is that RAN4 does not need to define parameter, L2, max , because consistent LBT failure preventing the UE from sending PRACH to the target cell is taken care of the RAN2 procedure.

- Observation # 2: According to the RAN2 procedure the UE upon reaching consistent uplink LBT failures on PRACH in PCell shall declare RLF.

- Proposal # 3: The parameter, maximum allowed number of missed PRACH occasions (L2, max), shall not be specified by RAN4.

- Summary

In this paper we have analysed the open issues related to the UE behaviour upon reaching maximum allowed missed SMTC occasions and upon experiencing consistent uplink LBT failures on PRACH a target PCell in RRC connection release with redirection procedure. Following are the proposals based on the analysis:

- Observation # 1: The cell search delay (Tidentify-NR) in RRC connection release procedure also includes L1 filtered SS-RSRP measurement.

- Proposal # 1: When L1 exceeds L1,max, the UE shall restart the identification of the target cell on the carrier configured for RRC connection release with redirection.

- Proposal # 2: Due to L1 exceeding L1,max, the UE is required to restart the identification of the target cell on the carrier configured for RRC connection release with redirection at least N times; where N =3. Upon exceeding N the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304.

- Observation # 2: According to the RAN2 procedure the UE upon reaching consistent uplink LBT failures on PRACH in PCell shall declare RLF.

- Proposal # 3: The parameter, maximum allowed number of missed PRACH occasions (L2, max), shall not be specified by RAN4.

# References

- R4-1912851, WF on NR-U RRM Requirements, Ericsson

- R4-1910010, Ad hoc minutes for NR-U RRM, Ericsson

- R4-1915777, WF on NR-U RRM Requirements, Ericsson