3GPP TSG RAN WG4 Meeting #94-e	R4-2001893

Electronic Meeting, February 24-March 06, 2020

Agenda Item:	8.5.4.2.1

Source: 	Ericsson

Title:	TP to TS 38.174: In-band emissions

Document for:	Approval

- Introduction

After RAN4#92bis the IAB TS [1] is email approved. Section 2 provides text proposal on 9 In-band emissions for the IAB TS 38.174 (Integrated access and backhaul radio transmission and reception).

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

#### 9.6.2.4	In-band emissions

##### 9.6.2.4.1	General

The in-band emission is defined as the average across 12 sub-carriers and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the IAB MT output power in a non–allocated RB to the IAB MT output power in an allocated RB.

The basic in-band emissions measurement interval is identical to that of the EVM test.

The requirement is verified with the test metric of In-band emission (Link=TX beam peak direction, Meas=Link angle).

This requirement is verified with the maximum declared IAB MT power configured.

##### 9.6.2.4.2	Minimum Requirement for IAB MT of IAB type 2-O

The relative in-band emission shall not exceed the values specified in Table 9.6.2.4.2-1 for IAB MT.

Table 9.6.2.4.2-1: Requirements for in-band emissions for IAB MT

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Maximum declared Output power > 27 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 27 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 17 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | 4 dBm ≤ Output power ≤ 17 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (PRB - 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if NRB is odd, or in the two RBs immediately adjacent to the DC frequency if NRB is even but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Figure 5.3.3-1). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Figure 5.3.3-1). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB= 1 or RB= -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: PRB is the transmitted power per allocated RB, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-1916000, “Updated IAB TS”, Qualcommn Inc.