3GPP TSG-RAN4 Meeting #94	R4-2002641

Athens, Greece 24th  – 28th Febuary, 2020

Source:	Verizon, Nokia, Qualcomm

Title:	TP for TR 38.716-02-00: CA_n2A_n66A

Agenda item:	9.2.2

Document for:	Approval

# Introduction

In RAN#86 meeting, the revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) [1]. This contribution is a text proposal for TR 38.716-02-00 to include CA_n2A_n66A.

# Reference

[1] 	RP-192615 [RAN86] Revised WID for NR CA_DC 2 band DL with up to 2 band UL_r3_clean.doc, ZTE

# Text Proposal

<Start of Text Proposal>

## 6.x	CA_n2-n66

### 6.x.1	Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1		Operating bands for CA

Table 6.x.1.1-1: CA band combination of band n2+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n2+n66

| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum Aggregated bandwidth [MHz] | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n66A | - | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |

#### 6.x.1.3		UE co-existence studies

Table 6.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n2-n66.

# Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 | 7400 | 7640 |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 | 6840 | 7120 |

Based on the table above, there is no harmonic relation.

# Table 6.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3860 | 3980 | 5790 | 5970 | 7720 | 7960 |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 |

Based on above tables, there is no harmonic mixing relation.

#### 6. x.1.4		∆TIB and ∆RIB values

For CA_n2-n66, the TIB,c and RIB,c values are given in tables below. For this combination, the results are mainly reused from CA_2_66.

Table 6. x.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2-n66 | n2 | 0.5 |
|  | n66 | 0.5 |

Table 6. x.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n2-n66 | n2 | 0.3 |
|  | n66 | 0.3 |

#### 6. x.1.5		REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

<Start of Text Proposal>