3GPP TSG-RAN WG4 Meeting #94-e-Bis		R4-2003183

E-meeting, 20 April – 30 April, 2020

Agenda item:	6.9.1.3

Source:	Intel Corporation

Title:	Discussion on BS performance requirements for URLLC

Document for:	Discussion

# Introduction

In the previous RAN4 meeting way forward on NR URLLC BS performance requirements was approved [1]. In this paper we provide our view on remaining open issues.

# Discussion

## FR1 high reliability requirements

In the previous RAN4 meeting, the multiple agreements were reached on FR1 test design for high reliability verification and the following topics are still open:

- PUSCH aggregation level - Option 1: 2 - Option 2: 4 - Option 3: 2, 4 - Option 3: 8 - Number of PRBs - Option 1: 25 - Option 2: Full bandwidth - Option 3: Other options not precluded (depending on bandwidth set) - Whether to introduce DFT-s-OFDM: - Option 1: No - Option 2: Yes

Aggregation factor

In the previous RAN4 meeting, the following TDD patterns were agreed for requirements definition:

- 15kHz SCS: 3D1S1U, S=10D: 2G: 2U

- 30kHz SCS: 7D1S2U, S=6D: 4G: 4U

Based on current Rel-15 procedure, in case aggregation factor is configured than “The UE shall repeat the TB across the pusch-AggregationFactor consecutive slots applying the same symbol allocation in each slot” (from 38.214). Also, based on 38.213:

If a UE is scheduled by a DCI format 0_1 to transmit PUSCH over multiple slots, and if tdd-UL-DLConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a slot from the multiple slots, at leastone symbol from a set of symbols where the UE is scheduled PUSCH transmission in the slot is a downlink symbol, the UE does not transmit the PUSCH in the slot.

In Figure 1 we provide the illustration of PUSCH transmissions in case pusch-AggregationFactor = 8.

Figure 1. Allocation of repetition transmissions.

From this figure we can observe that number of effective repetitions can be different from number of configured repetitions for TDD scenarios (i.e. 2 effective and 8 configured). If we configured 2 or 4 repetitions for SCS 15 kHz then there will be no repetitions effectively. However, we always will have 2 effective repetitions in case number of configured receptions 2, 4 or 8 for 30 kHz SCS. Same time, number of effective and configured receptions is same for FDD scenarios. Taking into account that maximum number of effective receptions in agreed TDD patterns is 2, we suggest to define FDD requirements with pusch-AggregationFactor = 2.

Number of PRBs

All existing requirements for CP-OFDM waveform are defined with full BW allocation. We suggest to reuse such assumption for FR1 URLLC high reliability requirements.

DFT-S-OFDM

As a part of this WID, we focus on verification of URLLC features. In particular, support of MCS Table 3 and slot aggregation will be verified in high reliability requirements. Support of DFT-S-OFDM is already verified in dedicated Rel-15 requirements. Same time, if BS supports “Low SE” MCS table and slot aggregation for CP-OFDM waveform then it supports these features for DFT-S-OFDM waveform. Therefore, we think that definition FR1 URLLC high reliability requirements is sufficient only for CP-OFDM waveform.

Proposal 1:	Use the following assumptions for FR1 URLLC high reliability requirements:

- pusch-AggregationFactor: 2 for FDD and TDD 30 kHz, 8 for TDD 15 kHz

- Number of PRBs: Full bandwidth

- Waveform: Only CP-OFDM

## FR1 PUSCH mapping Type B requirements

In the previous RAN4 meeting, the following agreements were reached on test design for PDSCH mapping Type B and PDSCH processing capability 2 verification:

- Starting symbol: 0 - PUSCH aggregation level is 1. - TDD pattern: - 15kHz SCS: 3D1S1U, S=10D: 2G: 2U - 30kHz SCS: 7D1S2U, S=6D: 4G: 4U - Antenna configuration: 1x2, low - Waveform: CP-OFDM - Channel condition: TDLC300-100 Low for FR1. - No PUSCH performance requirements for UL transmission with grant free/UL configured grant. - Number of HARQ transmission: 1 - SCS/BW: - 15 kHz/10 MHz and 30 kHz/40 MHz with defined test applicability rule - Other sets are not precluded - Symbol length (L) - Option 1: 4os - Option 2: 2os - Option 4: 7os - Option 5: 2os and 7os - DM-RS configuration Type 1 with single symbol - Proposals for symbol lengths of 7os if agreed - Option 1:1+0 - Option 2: 1+1 - MCS - Option 1: MCS 5 from Table 3 - Option 2: MCS 21 (658/1024) from Table 2 - Number of PRB - Option 1: full bandwidth - Option 2: A fixed number of RB - Test metrics - Option 1: 70% throughput - Option 2: 10% BLER (= 90% throughput) - Whether to introduce DFT-s-OFDM: - Option 1: No - Option 2: Yes

Symbol length

Based on our understanding short PUSCH transmission (2 or 4 OFDM symbols) is more typical for URLLC use case. Therefore, we suggest to use one of these options for URLLC requirements definition.

MCS

Based on current agreement we have the following options on MCS:

- Option 1: MCS 5 from Table 3

- Option 2: MCS 21 (658/1024) from Table 2

Based on our understanding, Option 2 is related to Table 6.1.4.1-2 from TS 38.214. This table is only applicable to scenarios with DFT-S-OFDM waveform and using of MCS from this table can be discussed only in case definition of URLLC DFT-S-OFDM requirements will be agreed. Therefore, we propose to use MCS5 from Table 3 to define CP-OFDM URLLC requirements.

Number of PRB

All existing requirements for CP-OFDM waveform are defined with full BW allocation. We suggest to reuse such assumption for FR1 URLLC high reliability requirements.

Test metrics

Purpose of these requirements is verification of processing of PUSCH Type B with small time domain allocation. Therefore, using of baseline test metric (i.e. 70% @ Max throughput) should be sufficient to meet target purpose.

DFT-s-OFDM

Similar to our discussion for FR1 high reliability requirements, we think that definition requirements for one waveform (i.e. CP-OFDM) should be sufficient to verify PUSCH Type B processing.

Proposal 2:	Use the following parameters for FR1 PUSCH mapping type B requirements:

- Waveform: Only CP-OFDM

- Symbol length: 2os or 4os

- MCS: MCS 5 from Table 3

- Number of PRBs: Full bandwidth

- Test metrics: 70% throughput

## FR2 UL URLLC requirements

In the previous RAN4 meeting, the following agreements were reached on FR2 testing of high reliability:

- Whether to define BS FR2 URLLC performance requirements for high reliability - Option 1: Do not define - Option 2: Define. - Test applicability rule for FR1 and FR2 performance requirements if both requirements are defined - Option 1: Based on BS declaration of support FR1 or FR2 - Option 2: If BS supports both FR1 and FR2, the performance requirements for both FR1 and FR2 should be tested - SCS&BW for FR2 if FR2 will be defined - 60kHz SCS - Option 1: 50/100MHz - Option 2: No test - Option 3: 50 MHz - 120kHz SCS - Option 1: 50/100/200MHz - Option 2: No test - Option 3: 100 MHz

We would like to note that definition of FR2 requirements for Type B mapping with small PUSCH duration is also open.

Based on our understanding, there may be such situation that BS supports only FR1 or only FR2. In such case, we can face the situation that URLLC functionality cannot be verified for BS which supports only FR2 operation. Therefore, we think that it is rather important to verify that BS, operating in FR2 region, supports “Low SE” MCS table, slot-aggregation and PUSCH mapping Type B with small duration.

Proposal 3:	Define PUSCH FR2 URLLC requirements to verify the following functionality: “Low SE” MCS table, slot-aggregation and PUSCH mapping Type B with small duration.

In the previous meeting, different options on CBW/SCS combinations were considered in case FR2 requirements will be defined. Taking into account limited timelines and workload, we think that requirements can be defined for one channel bandwidth per SCS. Therefore, we suggest to consider 60kHz/50MHz and 120kHz/100MHz as CBW/SCS candidates for FR2 URLLC UL requirements definition.

Proposal 4:	Define PUSCH FR2 URLLC requirements only for 60kHz/50MHz and 120kHz/100MHz.

# Conclusion

In this paper we provided view on URLCC BS performance requirements and made the following proposals:

Proposal 1:	Use the following assumptions for FR1 URLLC high reliability requirements:

- pusch-AggregationFactor: 2 for FDD and TDD 30 kHz, 8 for TDD 15 kHz

- Number of PRBs: Full bandwidth

- Waveform: Only CP-OFDM

Proposal 2:	Use the following parameters for FR1 PUSCH mapping type B requirements:

- Waveform: Only CP-OFDM

- Symbol length: 2os or 4os

- MCS: MCS 5 from Table 3

- Number of PRBs: Full bandwidth

- Test metrics: 70% throughput

Proposal 3:	Define PUSCH FR2 URLLC requirements to verify the following functionality: “Low SE” MCS table, slot-aggregation and PUSCH mapping Type B with small duration.

Proposal 4:	Define PUSCH FR2 URLLC requirements only for 60kHz/50MHz and 120kHz/100MHz.

# References

- R4-2002429 “Way forward on NR URLLC BS performance requirements”, Huawei, HiSilicon, RAN4 #94-e, February 2020.