3GPP TSG-RAN WG4 Meeting #94-e-Bis                                        	R4-2003242

Electronic meeting, 20th – 30th Apr., 2020

Source: 	LG Electronics

Title: 	TP on summary of interference studies for new x bands DL (x=3, 4, 5) with 2 bands UL inter-band CA combinations

Agenda item:	5.6.2

Document for:	Approval

1. Introduction

This contribution provides self-interference analysis results for new adding LTE-A carrier aggregation x bands (x=3, 4, 5) downlink (DL) with 2 bands uplink (UL) in Rel-16.

For 3 bands DL with 2 bands UL CA combinations, we analyze the impact to the 3rd own receiving band by the harmonics and IMD products from the dual uplink transmission.

For 4 bands DL with 2 bands UL CA combinations, we analyze the impact to the 3rd and 4th own receiving bands by the harmonics and IMD products from the dual uplink transmission.

For 5 bands DL with 2 bands UL CA combinations, we analyze the impact to the 3rd, 4th and 5th own receiving bands by the harmonics and IMD products from the dual uplink transmission.

Based on the self-desense analysis, we propose that RAN4 needs to define MSD requirements for only 3 bands DL with 2 bands UL CA combinations since all of the self-desense problems were already covered in low band CA combinations for 4 bands DL with 2 bands UL CA and 5 bands DL with 2 bands UL CA.

### ****************** Start of the TP in subclause 5.1.5 of TR36.716-03-02 ************************

### 5.1.5	Summary of interference studies

Table 5.1.5-1 summarizes self-interference problems for LTE-A inter-band carrier aggregation (CA) 3 bands DL with 2 bands UL.

Table 5.1.5-1: Summary of self-interference analysis for 3 bands DL with 2 bands UL CA

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | Intermodulation to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_3A-11A-18A | CA_3A-11A | - | 5th IMD | - | 4.9 dB |
| CA_3A-11A-26A | CA_3A-11A | - | 5th IMD | - | 4.9 dB |
| CA_1A-3A-42C | CA_1A-42C | - | - | - | N/A |
| CA_1A-3A-42C | CA_3A-42C | - | - | - | N/A |
| CA_2A-4A-13A | CA_2A-13A | - | 4th IMD | - | 7.6 dB |
|  | CA_4A-13A | - | 4th IMD | - | 6.2 dB |
| CA_2A-2A-4A-5A | CA_2A-5A | - | 4th IMD | - | 7.6 dB |
|  | CA_4A-5A | - | 2nd IMD 5th IMD | - | - 2nd and 5th IMD problems were already covered in Table 7.3.1A-0f:2DL/2UL_4A-5A of TS36.101. |
| CA_2A-2A-5A-66A-66A CA_2A-5A-66A, CA_2A-5A-66B, CA_2A-5A-66C, CA_2A-5B-66A, CA_2A-5B-66B, CA_2A-5B-66C, CA_2A-2A-5A-66A, CA_2A-2A-5A-66B, CA_2A-2A-5A-66C, CA_2A-5A-66A-66A, CA_2A-5B-66A-66A | CA_2A-5A | - | 4th IMD | - | 7.2 dB |
|  | CA_5A-66A | - | - | - | N/A |
| CA_2A-5A-66A | CA_2A-66A | - | - | - | N/A |
| CA_2A-5A-66A-66A | CA_2A-66A | - | - | - | N/A |
|  | CA_5A-66A | - | - | - | N/A |
| CA_2A-5A-48A | CA_2A-48A | - | - | - | N/A |
|  | CA_5A-48A | - | 3rd IMD | - | FFS |
| CA_2A-5A-48C CA_2A-5A-48D | CA_2A-48A | - | - | - | N/A |
|  | CA_5A-48A | - | 3rd IMD | - | FFS |
|  | CA_2A-5A | 2nd harmonic at high frequency band edge | 3rd IMD | - | FFS |
| CA_2A-5A-46D | CA_2A-5A | 3rd Harmonic | 4th IMD 5th IMD | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for MSD since the requirements do not need to apply in exclusion zone in Table 7.3.1A-0eA of TS36.101. |
| CA_5A-46D-66A | CA_5A_46A | - | 5th IMD | - | 0.3 dB |
|  | CA_5A_66A | 3rd Harmonic | 4th IMD 5th IMD | - | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for MSD since the requirements do not need to apply in exclusion zone in Table 7.3.1A-0eA of TS36.101. |
| CA_2A-13A-66A-66B CA_2A-13A-66A, CA_2A-2A-13A-66A, CA_2A-13A-66A-66A, CA_2A-13A-66B, CA_2A-13A-66C, | CA_2A-13A | - | 4th IMD | - | 7.2 dB |
|  | CA_13A-66A | - | 4th IMD | - | 6.2 dB |
| CA_2A-13A-66A, CA_2A-13A-66A-66A | CA_2A-66A | - | - | - | N/A |
| CA_2A-13A-48A | CA_2A-48A | - | - | - | N/A |
|  | CA_13A-48A | - | 3rd IMD | - | FFS |
| CA_2A-13A-48A-48C | CA_2A-13A | 2nd Harmonic at high frequency band edge | - | - | 2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS 36.101. |
| CA_13A-46D-66A | CA_13A-66A | 3rd Harmonic | 4th IMD 5th IMD | - | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for MSD since the requirements do not need to apply in exclusion zone in Table 7.3.1A-0eA of TS36.101. |
| CA_2A-13A-46D CA_2A-13A-46A-46D CA_2A-13A-46A-46C CA_2A-13A-46C CA_2A-13A-46A-46A CA_2A-13A-46A CA_2A-13A-46E | CA_2A-13A | 3rd Harmonic | 4th IMD | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for MSD since the requirements do not need to apply in exclusion zone in Table 7.3.1A-0eA of TS36.101. |
| CA_1A-3A-38A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
| CA_2A-12A-66A | CA_2A-12A | 3rd Harmonic impact from B12 to B66 | - | - | - 3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_2A-66A | - | - | - | N/A |
|  | CA_12A-66A | 3rd Harmonic impact from B12 to B66 | Side lobe impact of 4th IMD | - | - 3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. - 0 dB |
| CA_1A-3A-42D | CA_1A-3A | 2nd Harmonic impact from B3 to B42 | 4th IMD | Yes | - Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - 4th IMD problem was already covered in Table 7.3.1A0g:3DL/2UL_CA_1A-3A-42A/CA_1A-3A of TS36.101. |
|  | CA_1A-42A | - | - | - | N/A |
|  | CA_3A-42A | - | - | - | N/A |
|  | CA_1A-42C | - | - | - | N/A |
|  | CA_3A-42C | - | - | - | N/A |
| CA_2A-46E-48A, CA_2A-46D-48A CA_2A-46D-48C CA_2A-46C-48C CA_2A-46D-48A CA_2A-46A-48C CA_2A-46C-48A CA_2A-46A-48A | CA_2A-48A | 3rd Harmonic | 2nd IMD 3rd IMD | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS 36.101 - No need to study for MSD since the requirements do not need to apply in exclusion zone in Table 7.3.1A-0eA of TS36.101 |
| CA_2A-48A-66A CA_2A-48A-66A-66A, CA_2A-48D-66A, CA_2A-48C-66A-66A, CA_2A-48D-66A-66A, CA_2A-48E-66A, CA_2A-48E-66A-66A | CA_2A-48A | - | 4th IMD | - | - 12.1 dB |
|  | CA_48A-66A | - | 2nd IMD 5th IMD | - | - 28.3 dB for IMD2 - 0 dB for IMD5 |
| CA_2A-48A-66A, CA_2A-48A-66A-66A, CA_2A-48D-66A CA_2A-48C-66A-66A, CA_2A-48D-66A-66A, CA_2A-48E-66A, CA_2A-48E-66A-66A | CA_2A-66A | 2nd Harmonics | 2nd IMD | - | FFS |
| CA_3A-8A-38A | CA_3A-8A | - | 2nd IMD 3rd IMD | - | - 26.4 dB for IMD2 - 15.7 dB for IMD3 |
| CA_1A-1A-3C-5A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-5A | - | - | - | N/A |
|  | CA_3A-5A | - | - | - | N/A |
| CA_1A-1A-3C-28A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-28A | - | 5th IMD | - | - 4 dB for IMD5 |
|  | CA_3A-28A | 3rd Harmonic | 4th IMD | - | - 3rd harmonic impact from B28 to B1 was covered in Table 7.3.1A-0a of TS36.101. - 11 dB for IMD5 |
| CA_1A-3A-3A-7A, CA_1A-3A-3A-7A-7A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-7A | - | - | - | N/A |
|  | CA_3A-7A | - | - | - | N/A |
| CA_2A-14A-30A | CA_2A-14A | - | - | - | N/A |
|  | CA_14A-30A | - | - | - | N/A |
| CA_2A-14A-66A CA_2A-2A-14A-66A CA_2A-14A-66A-66A CA_2A-2A-14A-66A-66A CA_2A-14A-66A-66A-66A | CA_2A-14A | - | 4th IMD | - | -FFS for MSD |
|  | CA_14A-66A | - | 4th IMD | - | -FFS for MSD |
| CA_14A-30A-66A CA_14A-30A-66A-66A | CA_14A-30A | - | - | - | N/A |
|  | CA_14A-66A | - | - | - | N/A |
| CA_2A-46E-66A CA_2A-46D-66A CA_2A-46C-66A CA_2A-46A-66A | CA_2A-66A | 3rd harmonic | 3rd IMD 5th IMD | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS 36.101. |
| CA_13A-48A-66A CA_13A-48A-66A-66A CA_13A-48C-66A | CA_13A-48A | - | 3rd IMD | - | -17.1 dB |
|  | CA_13A-66A | 2nd harmonic | 5th IMD | - | - 2nd harmonic impact from B66 to B48 was covered in Table 7.3.1A-0a of TS36.101. -0 dB (no impact from 5th IMD) |
|  | CA_48A-66A | - | - | - | N/A |
| CA_46D-48C-66A CA_46C-48C-66A CA_46A-48C-66A CA_46D-48A-66A CA_46C-48A-66A CA_46A-48A-66A | CA_48A-66A | 3rd harmonic | 2nd IMD 3rd IMD | - | - No need to study for 3rd harmonic impact since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for MSD since the requirements do not need to apply in exclusion zone in Table 7.3.1A-0eA of TS36.101. |
| CA_1A-7A-20A | CA_1A-7A | - | 5th IMD | - | MSD is proposed to be 0 dB as the similar CA combination CA_1A-5A-7A has a very small MSD requirement (i.e., 1 dB) which should not be specified. |
|  | CA_1A-20A | - | - | - | N/A |
|  | CA_7A-20A | - | - | - | N/A |
| CA_1A-3A-3A-8A | CA_1A-3A | - | - | Yes | - Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-8A | - | - | - | N/A |
|  | CA_3A-8A | - | - | - | N/A |
| CA_1A-7A-8A CA_1A-7A-7A-8A | CA_7A-8A | - | - | - | N/A |
| CA_1A-7A-7A-8A | CA_1A-7A | - | - | - | N/A |
|  | CA_1A-8A | 3rd harmonic | - | - | - 3rd harmonic impact from B8 to B7 was covered in Table 7.3.1A-0a of TS36.101. |
| CA_2A-13A-48C | CA_2A-48A | - | - | - | N/A |
|  | CA_13A-48A | - | 3rd IMD | - | FFS |
|  | CA_2A-13A | 2nd harmonic at high frequency band edge | - | - | -2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS36.101. |
| CA_2A-13A-48D | CA_13A-48A | - | 3rd IMD | - | FFS |
|  | CA_2A-48A | - | - | - | N/A |
| CA_13A-48C-66A-66A, CA_13A-48D-66A-66A | CA_48A-66A | - | - | - | N/A |
|  | CA_13A-48A | - | 3rd IMD | - | - 3rd IMD problem was already covered in Table 7.3.1A0g:3DL/2UL_CA_13A-48A-66A/CA_13A-48A of TS36.101. |
|  | CA_13A-66A | 2nd harmonic | 5th IMD | - | -2nd harmonic impact from B66 to B48 was covered in Table 7.3.1A-0a of TS36.101. - No MSD is needed. |
| CA_13A-48D-66A | CA_48A-66A | - | - | - | N/A |
|  | CA_13A-48A | - | 3rd IMD | - | - 3rd IMD problem was already covered in Table 7.3.1A0g:3DL/2UL_CA_13A-48A-66A/CA_13A-48A of TS36.101. |
| CA_5A-48A-66A, CA_5A-48C-66A, CA_5A-48A-66A-66A, CA_5A-48D-66A-66A, CA_5A-48C-66A-66A | CA_48A-66A | - | - | - | N/A |
|  | CA_5A-66A | 2nd harmonic | 5th IMD | - | -2nd harmonic impact from B66 to B48 was covered in Table 7.3.1A-0a of TS36.101. -FFS for IMD |
|  | CA_5A-48A | - | - | - | N/A |
| CA_5A-48D-66A | CA_48A-66A | - | - | - | N/A |
|  |  | CA_5A-48A |  | - |  | - |  | - |  | N/A |  |
| CA_1A-18A-41C | CA_1A-18A | - | - | - | N/A |
|  |  | CA_1A-41A |  | - |  | 5th IMD |  | - |  | FFS |  |
|  |  | CA_1A-41C |  | - |  | 5th IMD |  | - |  | FFS |  |
|  | CA_18A-41A | - | - | - | N/A |
|  | CA_18A-41C | - | - | - | N/A |
| CA_1A-18A-41A | CA_1A-18A | - | - | - | N/A |
|  |  | CA_1A-41A |  | - |  | 5th IMD |  | - |  | FFS |  |
|  | CA_18A-41A | - | - | - | N/A |
| CA_2A-12A-30A | CA_2A-12A | - | 4th IMD | - | - It was already covered in Table 7.3.1A-0g of TS36.101. |
|  | CA_2A-30A | - | - | - | N/A |
|  | CA_12A-30A | - | - | - | N/A |
| CA_2A-12A-66A | CA_2A-12A | 3rd harmonic | - | - | -3rd harmonic impact from B12 to B66 was already covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_2A- 66A | - | - | - | N/A |
|  |  | CA_12A-66A |  | - |  | - |  | - |  | N/A |  |
| CA_2A-30A-66A | CA_2A-30A | - | - | - | N/A |
|  | CA_2A-66A | - | - | - | N/A |
|  | CA_30A-66A | - | - | - | N/A |
| CA_12A-30A-66A | CA_12A-30A | 3rd harmonic | - | - | -3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_12A-66A | - | - | - | N/A |
|  |  | CA_30A-66A |  | - |  | - |  | - |  | N/A |  |
| CA_2A-29A-30A | CA_2A-30A | - | - | - | N/A |
| CA_2A-29A-66A | CA_2A-66A | - | - | - | N/A |
| CA_29A-30A-66A | CA_30A-66A | - | - | - | N/A |
| CA_2A-12A-66A-66A | CA_2A-12A | 3rd harmonic | - | - | -3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_2A-66A | - | - | - | N/A |
|  |  | CA_12A-66A |  | - |  | - |  | - |  | N/A |  |
| CA_12A-30A-66A-66A | CA_12A-30A | 3rd harmonic | - | - | -3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_12A-66A | - | - | - | N/A |
|  |  | CA_30A-66A |  | - |  | - |  | - |  | N/A |  |
| CA_2A-2A-12A-30A | CA_2A-12A | - | 4th IMD | - | - It was already covered in Table 7.3.1A-0g of TS36.101. |
|  | CA_2A-30A | - | - | - | N/A |
|  | CA_12A-30A | - | - | - | N/A |
| CA_2A-2A-12A-66A | CA_2A-12A | 3rd harmonic | - | - | -3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_2A-66A | - | - | - | N/A |
|  |  | CA_12-66A |  | - |  | - |  | - |  | N/A |  |
| CA_2A-2A-5A-30A | CA_2A-5A | - | - | - | N/A |
|  |  | CA_2A-30A |  | - |  | 4th IMD |  | - |  | FFS |  |
|  | CA_5A-30A | - | - | - | N/A |
| 2A-2A-12A-66A-66A | CA_2A-12A | 3rd harmonic | - | - | -3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_2A-66A | - | - | - | N/A |
|  |  | CA_12A-66A |  | - |  | - |  | - |  | N/A |  |

Table 5.1.5-2 summarizes self-interference problems for LTE-A inter-band CA 4 bands DL with 2 bands UL.

Table 5.1.5-2: Summary of self-interference analysis for 4 bands DL with 2 bands UL

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | Intermodulation to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_2A-13A-48C-66A | CA_2A-13A | 2nd Harmonic at high frequency band edge | 4th IMD into B66 | - | - 2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS 36.101. - 4th IMD problem was already covered in 3DL_2A-13A-66A_2UL_2A-13A |
|  | CA_13A-66A | 2nd Harmonic | 4th IMD into B2 5th IMD into B48 | - | - 2nd harmonic impact from B66 to B48 was covered in Table 7.3.1A-0a of TS 36.101. - 4th IMD problem was already covered in 3DL_2A-13A-66A_2UL_13A-66A - 5th IMD problem was already covered in 3DL_13A-48A-66A_2UL_13A-66A |
| CA_2A-13A-48A-66A, CA_2A-13A-48C-66A, CA_2A-13A-48A-66A-66A, CA_2A-13A-48C-66A-66A, CA_2A-13A-48D-66A, CA_2A-13A-48D-66A-66A | CA_2A-66A | 2nd harmonic impact from B2 to B48 | 2nd IMD into B48 | - | - 2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS 36.101. |
|  | CA_2A-48A | - | 4th IMD into B66 | - | - 4th IMD problem was already covered in 3DL_2A-48A-66A_2UL_2A-48A. |
|  | CA_48A-66A | - | 2nd IMD into B2 | - | - 2nd IMD problem was already covered in 3DL_2A-48A-66A_2UL_48A-66A. |
|  | CA_13A-66A | 2nd harmonic impact from B13 to B48 | 4th IMD into B2 5th IMD into B48 | - | - 2nd harmonic impact from B66 to B48 was covered in Table 7.3.1A-0a of TS 36.101. - 4th IMD problem was already covered in 3DL_2A-13A-66A_2UL_13A-66A - 5th IMD problem was already covered in 3DL_13A-48A-66A_2UL_13A-66A |
|  | CA_13A-48A | - | 3rd IMD into B2 | - | - 3rd IMD problem will be covered in 3DL_2A-13A-48A_2UL_13A-48A. |
| CA_2A-13A-48A-48A-66A | CA_2A-13A | 2nd Harmonic at high frequency band edge | 4th IMD into B66 | - | - 2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS 36.101. - 4th IMD problem was already covered in 3DL_2A-13A-66A_2UL_13A-66A |
|  | CA_13A-66A | 2nd Harmonic | 4th IMD into B2 5th IMD into B48 | - | - 2nd harmonic impact from B66 to B48 was covered in Table 7.3.1A-0a of TS 36.101. - 4th IMD problem was already covered in 3DL_2A-13A-66A_2UL_13A-66A - 5th IMD problem was already covered in 3DL_13A-48A-66A_2UL_13A-66A |
| CA_1A-3A-8A-38A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-8A | 2nd Harmonic impact from B8 to B3 |  | - | - 2nd harmonic impact from B8 to B3 was covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_3A-8A | - | 2nd,3rd IMDs into B38 | - | - MSD for 2nd and 3rd IMDs into B38 was already covered in 3DL_3A-8A-38A_2UL_3A-8A. |
| CA_2A-46E-48A-66A CA_2A-46D-48C-66A CA_2A-46C-48C-66A CA_2A-46D-48A-66A CA_2A-46A-48C-66A CA_2A-46C-48A-66A CA_2A-46A-48A-66A | CA_2A-48A | 3rd Harmonic impact from B2 to B46 | 2nd, 3rd IMDs into B46 4th IMD into B2 and B66 | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for 2nd and 3rd IMDs into B46 since Reference sensitivity for CA with band 46 is specified in Table 7.3.1A-0eA of TS36.101. - MSD for 4th IMD into B2 and B66 was already covered in 3 bands DL_2A-48A-66A_2 bands UL_2A-48A. |
|  | CA_48A -66A | 3rd Harmonic impact from B66 to B46 | 2nd,3rd IMDs into B46 5th IMD into B2 |  | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for 2nd and 3rd IMDS into B46 since Reference sensitivity for CA with band 46 is specified in Table 7.3.1A-0eA of TS36.101. - MSD for 5th IMD into B2 was already covered in 3 bands DL_2A-48A-66A_2 bands UL_48A-66A. |
| CA_1A-3A-41C-42C | CA_1A-3A | 2nd Harmonic impact from B3 to B42 | 4th IMD into B42 5th IMD into B41 | Yes | - Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - 4th IMD into B42 problem was already covered in Table 7.3.1A-0g: 3 bands DL_1A-3A-42A/2 bands UL_1A-3A of TS36.101. - 5th IMD into B41 problem was already covered in Table 7.3.1A-0g: 3bands DL_1A-3A-41/2 bands UL_1A-3A. |
|  | CA_1A-42A | - | 4th IMD into B41 | - | - For 4th IMD into B41, no need to study since B41 and B42 are always synchronized. |
|  | CA_1A-42C | - | 4th IMD into B41 | - | - For 4th IMD into B41, no need to study since B41 and B42 are always synchronized. |
|  | CA_3A-42A | - | - | - | N/A |
|  | CA_3A-42C | - | - | - | N/A |
| CA_1A-3A-41C-42A | CA_1A-3A | 2nd Harmonic impact from B3 to B42 | 4th IMD into B42 5th IMD into B41 | Yes | - Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - 4th IMD into B42 problem was already covered in Table 7.3.1A-0g: 3 bands DL_1A-3A-42A/2 bands UL_1A-3A of TS36.101. - 5th IMD into B41 problem was already covered in Table 7.3.1A-0g: 3bands DL_1A-3A-41/2 bands UL_1A-3A. |
|  | CA_1A-42A | - | 4th IMD into B41 | - | - For 4th IMD into B41, no need to study since B41 and B42 are always synchronized. |
|  | CA_3A-42A | - | - | - | N/A |
| CA_1A-3A-41A-42C | CA_1A-3A | 2nd Harmonic impact from B3 to B42 | 4th IMD into B42 5th IMD into B41 | Yes | - Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - 4th IMD into B42 problem was already covered in Table 7.3.1A-0g: 3 bands DL_1A-3A-42A/2 bands UL_1A-3A of TS36.101. - 5th IMD into B41 problem was already covered in Table 7.3.1A-0g: 3bands DL_1A-3A-41/2 bands UL_1A-3A. |
|  | CA_1A-42A | - | 4th IMD into B41 | - | - For 4th IMD into B41, no need to study since B41 and B42 are always synchronized. |
|  | CA_1A-42C | - | 4th IMD into B41 | - | - For 4th IMD into B41, no need to study since B41 and B42 are always synchronized. |
|  | CA_3A-42A | - | - | - | N/A |
|  | CA_3A-42C | - | - | - | N/A |
| CA_1A-3A-41A-42A | CA_1A-3A | 2nd Harmonic impact from B3 to B42 | 4th IMD into B42 5th IMD into B41 | Yes | - Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - 4th IMD into B42 problem was already covered in Table 7.3.1A-0g: 3 bands DL_1A-3A-42A/2 bands UL_1A-3A of TS36.101. - 5th IMD into B41 problem was already covered in Table 7.3.1A-0g: 3bands DL_1A-3A-41/2 bands UL_1A-3A. |
|  | CA_1A-42A | - | 4th IMD into B41 | - | - For 4th IMD into B41, no need to study since B41 and B42 are always synchronized. |
|  | CA_3A-42A | - | - | - | N/A |
| CA_2A-14A-30A-66A CA_2A-2A-14A-30A-66A CA_2A-14A-30A-66A-66A | CA_2A-14A |  | 4th IMD into B66 | - | - MSD for 4th IMD into B66 will be analyzed in 3 bands DL_2A-14A-66A_2 bands UL 2A-14A. |
|  | CA_14A-66A |  | 4th IMD into B2 | - | - MSD for 4th IMD into B2 will be analyzed in 3 bands DL_2A-14A-66A_2 bands UL 14A-66A. |
| CA_2A-2A-14A-30A-66A CA_2A-14A-30A-66A-66A | CA_14A-30A | - | - | - | N/A |
| CA_2A-13A-46E-66A CA_2A-13A-46D-66A CA_2A-13A-46C-66A CA_2A-13A-46A-66A | CA_2A-13A | 3rd harmonic impact from B2 to B46 | 4th IMD into B46 4th IMD into B66 | - | - No need to study for 3rd harmonic impact since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for MSD since the requirements do not need to apply in exclusion zone in Table 7.3.1A-0eA of TS36.101. - MSD for 4th IMD into B66 was already covered in Table 7.3.1A-0g of TS36.101. |
| CA_1A-3A-7A-20A | CA_1A-3A | - | - | Yes | - Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-7A | - | 5th IMD into B20 | - | - MSD for 5th IMD into B20 will be analyzed in 3 bands DL 1A-7A-20A 2 bands UL 1A-7A, |
|  | CA_1A-20A | - | - | - | N/A |
|  | CA_3A-7A | - | - | - | N/A |
|  | CA_3A-20A | - | 2nd and 3rd IMDs into B7 | - | - MSD for 2nd IMD into B7 was already covered in Table 7.3.1A-0g of TS36.101. |
|  | CA_7A-20A | - | - | - | N/A |
| CA_1A-3A-7A-8A | CA_1A-8A | -2nd harmonic impact from B8 to B3 -3rd harmonic impact from B8 to B7 | - | - | - 2nd harmonic impact from B8 to B3 was already covered in Table 7.3.1A-0a of TS36.101. - 3rd harmonic impact from B8 to B7 was already covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_7A-8A | 2nd harmonic impact from B8 to B3 | - | - | - 2nd harmonic impact from B8 to B3 was already covered in Table 7.3.1A-0a of TS36.101. |
| CA_1A-3A-3A-7A-8A CA_1A-3A-7A-7A-8A CA_1A-3A-3A-7A-7A-8A | CA_1A-3A | - | - | Yes | - Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-7A | - | 5th IMD into B8 | - | - MSD for 5th IMD into B8 will be analyzed in 3 bands DL 1A-7A-7A-8A 2 bands UL 1A-7A, |
|  | CA_1A-8A | - 2nd harmonic impact from B8 to B3 - 3rd harmonic impact from B8 to B7 | - | - | - 2nd harmonic impact from B8 to B3 was already covered in Table 7.3.1A-0a of TS36.101. - 3rd harmonic impact from B8 to B7 was already covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_3A-7A | - | 3rd IMD into B8 | - | - MSD for 3rd IMD into B8 was already covered in Table 7.3.1A-0g of TS36.101. |
|  | CA_3A-8A | -2nd harmonic impact from B8 to B7 | 2nd and 3rd IMD into B7 | - | - 2nd harmonic impact from B8 to B7 was already covered in Table 7.3.1A-0a of TS36.101. - MSD for 2nd and 3rd IMDs into B7 was already covered in Table 7.3.1A-0g of TS36.101. |
|  | CA_7A-8A | 2nd harmonic impact from B8 to B3 | - | - | - 2nd harmonic impact from B8 to B3 was already covered in Table 7.3.1A-0a of TS36.101. |
| CA_2A-5A-48A-66A, CA_2A-5A-48C-66A, CA_2A-5A-48A-66A-66A, CA_2A-5A-48C-66A-66A, CA_2A-5A-48D-66A, CA_2A-5A-48D-66A-66A | CA_2A-66A | 2nd harmonic impacts from B2 and B66 to B48 | 2nd IMD into B48 | - | - 2nd harmonic impacts from B2 and B66 to B48 was already covered in Table 7.3.1A-0a of TS36.101. - 2nd IMD problem will be covered in 3DL_2A-48A-66A_2UL_2A-66A. |
|  | CA_2A-48A | - | 4th IMD into B66 | - | - 4th IMD problem was already covered in 3 bands DL_2A-48A-66A_2 bands UL_2A-48A. |
|  | CA_48A-66A | - | 2nd IMD into B2 | - | - 2nd IMD problem was already covered in 3 bands DL_2A-48A-66A_2 bands UL_48A-66A. |
|  | CA_5A-66A | 2nd harmonic impact from B66 to B48 | 5th IMD into B48 | - | - 2nd harmonic impact from B66 to B48 was already covered in Table 7.3.1A-0a of TS36.101. - 5th IMD problem will be covered in 3DL_5A-48A-66A_2UL_5A-66A. |
|  | CA_5A-48A | - | 3rd IMD into B2 | - | - 3rd IMD problem will be covered in 3DL_2A-5A-48A_2UL_5A-48A. |
|  | CA_2A-5A | 2nd harmonic impact from B2 to B48 | 3rd IMD into B48 4th IMD into B66 | - | - 2nd harmonic impact from B2 to B48 was already covered in Table 7.3.1A-0a of TS36.101. - 3rd IMD problem will be covered in 3DL_2A-5A-48A_2UL_2A-5A. - 4th IMD problem was already covered in 3DL_2A-5A-66A_2UL_2A-5A. |
| CA_2A-5A-48C-66A-66A | CA_2A-66A | 2nd harmonic impact from B2 and B66 to B48 | 2nd IMD into B48 | - | - 2nd harmonic impacts from B2 and B66 to B48 was already covered in Table 7.3.1A-0a of TS36.101. - 2nd IMD problem will be covered in 3DL_2A-48A-66A_2UL_2A-66A. |
|  | CA_2A-48A | - | 4th IMD into B66 | - | - 4th IMD problem was already covered in 3 bands DL_2A-48A-66A_2 bands UL_2A-48A. |
|  | CA_48A-66A | - | 2nd IMD into B2 | - | - 2nd IMD problem was already covered in 3 bands DL_2A-48A-66A_2 bands UL_48A-66A. |
|  | CA_5A-66A | 2nd harmonic impact from B66 to B48 | 5th IMD into B48 | - | - 2nd harmonic impact from B66 to B48 was already covered in Table 7.3.1A-0a of TS36.101. - 5th IMD problem will be covered in 3DL_5A-48A-66A_2UL_5A-66A. |
|  |  | CA_5A-48A |  | - |  | 3rd IMD into B2 |  | - |  | - 3rd IMD problem will be covered in 3DL_2A-5A-48A_2UL_5A-48A. |  |
| CA_2A-12A-30A-66A | CA_2A-12A | 3rd harmonic impact from B12 to B66 | 4th IMD into B30 | - | - 3rd harmonic impact from B12 to B66 was already covered in Table 7.3.1A-0a of TS36.101. - 4th IMD problem was already covered in 3DL_2A-12A-30A_2UL_2A-12A. |
|  | CA_2A-30A | - | - | - | N/A |
|  | CA_2A-66A | - | - | - | N/A |
|  | CA_12A-30A | 3rd harmonic impact from B12 to B66 | - | - | - 3rd harmonic impact from B12 to B66 was already covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_12A-66A | - | - | - | N/A |
|  | CA_30A-66A | - | - | - | N/A |
| CA_2A-29A-30A-66A | CA_2A-30A | - | - | - | N/A |
|  | CA_2A-66A | - | - | - | N/A |
|  | CA_30A-66A | - | - | - | N/A |
| CA_2A-2A-12A-30A-66A | CA_2A-12A | 3rd harmonic impact from B12 to B66- | 4th IMD into B30 | - | - 3rd harmonic impact from B12 to B66 was already covered in Table 7.3.1A-0a of TS36.101. - 4th IMD problem was already covered in 3DL_2A-12A-30A_2UL_2A-12A. |
|  | CA_2A-30A | - | - | - | N/A |
|  | CA_2A-66A | - | - | - | N/A |
|  | CA_12A-30A | 3rd harmonic impact from B12 to B66 | - | - | - 3rd harmonic impact from B12 to B66 was already covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_12A-66A | - | - | - | N/A |
|  | CA_30A-66A | - | - | - | N/A |

Table 5.1.5-3 summarizes self-interference problems for LTE-A inter-band CA 5 bands DL with 2 bands UL.

Table 5.1.5-3: Summary of self-interference analysis for 5 bands DL with 2 bands UL

<Table will be added if there are proposed in related basket WID>

Based on the MSD test configuration as summarized in Table 5.1.5.1, 5.1.5.2, and 5.1.5.3, it is proposed to define the following REFSENS exceptions and test configurations in the core specifications for 3 bands DL/2 bands UL inter-band CA with IMD problems.

Table 5.1.5.4 Proposed 3 bands DL/ 2 bands UL inter-band CA REFSENS exceptions and test configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_3A-11A-18A | CA_3A-11A | 3 | 1730 | 5 | 25 | 1825 | 5 | N/A | FDD | IMD5 |
|  |  | 11 | 1442.9 | 5 | 25 | 1490.9 | 5 |  |  |  |
|  |  | 18 | 823.7 | 5 | 25 | 868.7 | 5 | 4.9 |  |  |
| CA_3A-11A-26A | CA_3A-11A | 3 | 1730 | 5 | 25 | 1825 | 5 | N/A | FDD | IMD5 |
|  |  | 11 | 1442.9 | 5 | 25 | 1490.9 | 5 |  |  |  |
|  |  | 26 | 823.7 | 5 | 25 | 868.7 | 5 | 4.9 |  |  |
| CA_2A-4A-13A | CA_2A-13A | 2 | 1855 | 5 | 25 | 1935 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 4 | 1746 | 5 | 25 | 2146 | 5 | 7.6 |  |  |
|  | CA_4A-13A | 4 | 1750 | 5 | 25 | 2150 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 780 | 5 | 25 | 749 | 5 |  |  |  |
|  |  | 2 | 1860 | 5 | 25 | 1940 | 5 | 6.2 |  |  |
| CA_2A-2A-4A-5A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 4 | 1732 | 5 | 25 | 2132 | 5 | 7.6 |  |  |
| CA_2A-5B-66A-66A CA_2A-5A-66A, CA_2A-5A-66B, CA_2A-5A-66C, CA_2A-5B-66A, CA_2A-5B-66B, CA_2A-5B-66C, CA_2A-2A-5A-66A, CA_2A-2A-5A-66B, CA_2A-2A-5A-66C, CA_2A-5A-66A-66A, CA_2A-2A-5A-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | 7.2 |  |  |
| CA_2A-13A-66A, CA_2A-2A-13A-66A, CA_2A-13A-66A-66A, CA_2A-13A-66B, CA_2A-13A-66C, CA_2A-13A-66A-66B | CA_2A-13A | 2 | 1860 | 5 | 25 | 1940 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 66 | 1736 | 5 | 25 | 2156 | 5 | 7.2 |  |  |
|  | CA_13A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | 6.2 | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 | N/A |  |  |
|  |  | 66 | 1762 | 5 | 25 | 2162 | 5 |  |  |  |
| CA_2A-12A-66A | CA_12A-66A | 2 | 1907.5 | 5 | 25 | 1987.5 | 5 | 0 | FDD | IMD4 |
|  |  | 12 | 713.5 | 5 | 25 | 743.5 | 5 | N/A |  |  |
|  |  | 66 | 1712.5 | 5 | 25 | 2112.5 | 5 |  |  |  |
| CA_2A-48A-66A | CA_48A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | 28.3 | FDD-TDD | IMD2 |
|  |  | 48 | 3695 | 5 | 25 | 3695 | 5 | N/A |  |  |
|  |  | 66 | 1735 | 5 | 25 | 2135 | 5 | N/A |  |  |
|  |  | 2 | 1895 | 5 | 25 | 1975 | 5 | 0 | FDD-TDD | IMD5 |
|  |  | 48 | 3620 | 5 | 25 | 3620 | 5 | N/A |  |  |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | N/A |  |  |
| CA_3A-8A-38A | CA_3A-8A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD-TDD | IMD2 |
|  |  | 8 | 885 | 5 | 25 | 930 | 5 | N/A |  |  |
|  |  | 38 | 2605 | 5 | 25 | 2605 | 5 | 26.4 |  |  |
|  |  | 3 | 1745 | 5 | 25 | 1840 | 5 | N/A | FDD-TDD | IMD3 |
|  |  | 8 | 895 | 5 | 25 | 940 | 5 | N/A |  |  |
|  |  | 38 | 2595 | 5 | 25 | 2595 | 5 | 15.7 |  |  |
| CA_1A-1A-3C-28A | CA_1A-28A | 1 | 1975 | 5 | 25 | 2165 | 5 | N/A | FDD | IMD5 |
|  |  | 3 | 1731 | 5 | 25 | 1826 | 5 | 4.0 |  |  |
|  |  | 28 | 708 | 5 | 25 | 763 | 5 | N/A |  |  |
|  | CA_3A-28A | 1 | 1949 | 5 | 25 | 2139 | 5 | 11.0 | FDD | IMD4 |
|  |  | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A |  |  |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A |  |  |
| CA_2A-48A-66A CA_2A-48C-66A | CA_2A-48A | 2 | 1905 | 5 | 25 | 1985 | 5 | N/A | FDD-TDD | IMD4 |
|  |  | 48 | 3560 | 5 | 25 | 3560 | 5 | N/A |  |  |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | 12.1 |  |  |
| CA_2A-14A-66A CA_2A-2A-14A-66A CA_2A-14A-66A-66A CA_2A-2A-14A-66A-66A CA_2A-14A-66A-66A-66A | CA_2A-14A | 2 | 1870 | 5 | 25 | 1950 | 5 | N/A | FDD | IMD4 |
|  |  | 14 | 793 | 5 | 25 | 763 | 5 | N/A |  |  |
|  |  | 66 | 2154 | 5 | 25 | 2154 | 5 | TBD |  |  |
|  | CA_14A-66A | 2 | 1874 | 5 | 25 | 1954 | 5 | TBD | FDD | IMD4 |
|  |  | 14 | 793 | 5 | 25 | 763 | 5 | N/A |  |  |
|  |  | 66 | 1770 | 5 | 25 | 2190 | 5 | N/A |  |  |
| CA_13A-48A-66A | CA_13A-48A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD-TDD | IMD3 |
|  |  | 48 | 3695 | 5 | 25 | 3695 | 5 | N/A |  |  |
|  |  | 66 | 1731 | 5 | 25 | 2131 | 5 | 17.1 |  |  |
|  | CA_13A-66A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD-TDD | IMD5 |
|  |  | 48 | 3626 | 5 | 25 | 3626 | 5 | 0.0 |  |  |
|  |  | 66 | 1730 | 5 | 25 | 2130 | 5 | N/A |  |  |
| CA_1A-7A-20A | CA_1A-7A | 1 | 1960 | 5 | 25 | 2150 | 5 | N/A | FDD | IMD5 |
|  |  | 7 | 2540 | 10 | 50 | 2660 | 10 | N/A |  |  |
|  |  | 20 | 841 | 5 | 25 | 800 | 5 | 0.0 |  |  |
| CA_2A-5A-48A CA_2A-5A-48C CA_2A-5A-48D | CA_5A-48A | 2 | 1882 | 5 | 25 | 1962 | 5 | TBD | FDD-TDD | IMD3 |
|  |  | 5 | 839 | 5 | 25 | 884 | 5 | N/A |  |  |
|  |  | 48 | 3640 | 5 | 25 | 3640 | 5 | N/A |  |  |
| CA_2A-5A-48C CA_2A-5A-48D | CA_2A-5A | 2 | 1905 | 5 | 25 | 1985 | 5 | N/A | FDD-TDD | IMD3 |
|  |  | 5 | 844 | 5 | 25 | 889 | 5 | N/A |  |  |
|  |  | 48 | 3593 | 5 | 25 | 3593 | 5 | TBD |  |  |
| CA_2A-13A-48A CA_2A-13A-48C CA_2A-13A-48D | CA_13A-48A | 2 | 1903.5 | 5 | 25 | 1983.5 | 5 | TBD | FDD-TDD | IMD3 |
|  |  | 13 | 784.5 | 5 | 25 | 753.5 | 5 | N/A |  |  |
|  |  | 48 | 3552.5 | 5 | 25 | 3552.5 | 5 | N/A |  |  |
| CA_2A-48A-66A, CA_2A-48D-66A, CA_2A-48E-66A, CA_2A-48A-66A-66A, CA_2A-48C-66A-66A, CA_2A-48D-66A-66A, CA_2A-48E-66A-66A | CA_2A-66A | 2 | 1855 | 5 | 25 | 1935 | 5 | N/A | FDD-TDD | IMD2 |
|  |  | 48 | 3625 | 5 | 25 | 3625 | 5 | TBD |  |  |
|  |  | 66 | 1770 | 5 | 25 | 2190 | 5 | N/A |  |  |
| CA_5A-48A-66A, CA_5A-48C-66A, CA_5A-48A-66A-66A, CA_5A-48D-66A-66A, CA_5A-48C-66A-66A | CA_5A-66A | 5 | 829 | 5 | 25 | 874 | 5 | N/A | FDD-TDD | IMD5 |
|  |  | 48 | 3622 | 5 | 25 | 3622 | 5 | TBD |  |  |
|  |  | 66 | 1760 | 5 | 25 | 2180 | 5 | N/A |  |  |
| CA_1A-18A-41C | CA_1A-41C | 1 | 1970 | 5 | 25 | 2160 | 5 | N/A | FDD-TDD | IMD5 |
|  |  | 18 | 823 | 5 | 25 | 868 | 5 | TBD |  |  |
|  |  | 41 | 2521 | 5 | 25 | 2521 | 5 | N/A |  |  |
| CA_1A-18A-41A CA_1A-18A-41C | CA_1A-41A | 1 | 1970 | 5 | 25 | 2160 | 5 | N/A | FDD-TDD | IMD5 |
|  |  | 18 | 823 | 5 | 25 | 868 | 5 | TBD |  |  |
|  |  | 41 | 2521 | 5 | 25 | 2521 | 5 | N/A |  |  |
| CA_2A-2A-5A-30A | CA_2A-30A | 2 | 1870 | 5 | 25 | 1950 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 835 | 5 | 25 | 880 | 5 | TBD |  |  |
|  |  | 30 | 2310 | 5 | 25 | 2355 | 5 | N/A |  |  |

5.2	RRM specific

[Editor Note] It will be updated in future

<End of TP>