3GPP TSG-RAN WG4 Meeting # 94-e-Bis                                                       R4-2003315

Electronic Meeting, 20 – 30 Apr., 2020

Title: 	Discussion of IAB-MT feature list

Source: 	CATT

Agenda item:	6.5.1.3

Document for:	Approval

- Introduction

In RAN#87e meeting, IAB mandatory features were discussed. No agreement was reached and RAN WGs were tasked to discuss it further. This contribution provides our views for RF and RRM features.

- Discussion

The following proposals in RP-200501 were endorsed in RAN#87e meeting,

-- RAN WGs to investigate which of the mandatory Rel-15 UE features (as defined in TR 38.822) can be optional for basic operation of (and if found useful, for different classes of IAB-MTs as defined by RAN4).

-- RAN WGs should strive to minimize specification impact.

Based on the guidance, we reviewed the RF/RRM features in TR 38.822 and summarized the preliminary views for the mandatory features.

Table 1: Preliminary views on IAB RF/RRM features

| Features | Index | Feature group | Mandatory/Optional | Preliminary Views | Justification |
| --- | --- | --- | --- | --- | --- |
| 1. System parameter | 1-1 | 60kHz of subcarrier spacing for FR1 | Optional with capability signalling |  |  |
|  | 1-2 | 64QAM modulation for FR2 PDSCH | Mandatory without capability signalling | No change |  |
|  | 1-3 | 64QAM for PUSCH | Mandatory without capability signalling | No change |  |
|  | 1-4 | 256QAM for PDSCH | Mandatory with capability signalling for FR1 | No change |  |
|  |  |  | Optional with capability signalling for FR2 |  |  |
|  | 1-5 | 256QAM for PUSCH | Optional with capability signalling (for both FR1 and FR2) |  |  |
|  | 1-6 | pi/2-BPSK for PUSCH | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 | Optional for both FR1 and FR2 | It’s low possibility that backhaul link uses BPSK modulation. |
|  | 1-7 | pi/2-BPSK for PUCCH format 3/4 | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 | Optional for both FR1 and FR2 | The same with above. |
|  | 1-8 | Active BWP switching delay | Mandatory to support either type 1 or type 2 with capability signalling | No change |  |
|  | 1-9 | Support of EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Optional with capability signalling |  |  |
|  | 1-10 | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Mandatory to support either type 1 or type 2 with capability signalling if UE reports its capability in 1-10 as 1) LTE and NR UL Transmission in the shared carrier via TDM only, or 3) LTE and NR UL transmission in the shared carrier via FDM or TDM | No change |  |
|  | 1-11 | 7.5kHz UL raster shift | Mandatory in the SUL bands with uplink sharing either from UE perspective or from network perspective 7.5KHz raster shift as mandatory without capability signalling. 7.5kHz UL raster shift is mandatory for the bands described in clause 5.4.2.1 of Release 15 TS 38.101-1. RAN4 can revisit the above bands in the future release. 7.5KHz raster shift is not mandatory for other LTE refarming band except the bands which were agreed to support 7.5kHz UL raster shift as mandatory | No change |  |
| 2. UE RF | 2-1 | Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC | For FR1, all the bandwidths listed in TS38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC. The bandwidths listed in the slide #3 of R4-1805985 are mandatory with a single CC. 90MHz is optional for n41, n77, n78. For FR2, the set of mandatory CBW is 50, 100, 200 MHz. | Need some change. Already send LS to RAN2. | According to approved LS R4-1916165, RAN4 decided that IAB-MT declares all the supported channel bandwidths rather than having a mandatory set as current UEs. |
|  | 2-2 | Simultaneous reception or transmission with same or different numerologies in CA | Same numerology for intra-band NR CA including both continuous and non-continuous is mandatory with capability in both FR1 and FR2. Two mixed numerologies between FR1 band(s) and FR2 band(s) in DL and UL are mandatory with capability if UE supports inter-band NR CA including both FR1 band(s) and FR2 band(s). Optional for other cases. | No change |  |
|  | 2-3 | Non-contiguous intra-band CA frequency separation class for FR2 | Mandatory to support a frequency separation class within {I, II, III} specified in TS38.101-2 with capability if UE supports non-contiguous CA in FR2 | No change |  |
|  | 2-4 | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | Mandatory/Optional support depends on band combination and captured in TS 38.101-3 | No change |  |
|  | 2-5 | Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) | Mandatory/Optional support depends on band combination and captured in TS 38.101-1, TS 38.101-2 and TS 38.101-3 | No change |  |
|  | 2-6 | Asynchronous FDD-FDD intra-band EN-DC DC | Optional with capability signalling |  |  |
|  | 2-7 | Almost contiguous UL CP-OFDM | Optional with capability signalling |  |  |
|  | 2-8 | UE power class | Mandatory to support at least one power class with capability. The capability signalling is absent if UE supports only default power class | Need some change. Send LS to RAN2 to be aware of this. | IAB-MT declares the output power. |
|  | 2-9 | Simultaneous reception and transmission for SA SUL band combinations | Mandatory/Optional support depends on band combination and captured in TS 38.101-1 | No change |  |
|  | 2-10 | Multiple frequency band indication | Mandatory without capability signalling | Not needed | Supported bands are declared by the manufacturer. |
|  | 2-11 | Modified MPR behaviour | Optional with capability signalling |  |  |
|  | 2-12 | Multiple NS/P-Max | Mandatory without capability signalling | Not needed | There’s no roaming issue for wide area and medium range IAB-MT, so it should be declared by the manufacturer. NS is related to A-MPR, IAB-MT RF requirements are different with UE. |
|  | 2-13 | Maximum uplink duty cycle for FR1 power class 2 UE | Optional with capability signalling. The capability signalling is absent if UE supports 50% |  |  |
|  | 2-14 | Power boosting for Pi/2 BPSK for power class 3 UE | Optional with capability signalling |  |  |
|  | 2-15 | Maximum uplink duty cycle for FR2 | Optional with capability signalling |  |  |
|  | 2-16 | PA architectures for intra-band EN-DC | Mandatory to support either single or dual PA architectures with capability if UE supports intra-band EN-DC configuration in uplink. The capability signalling is absent if UE supports single PA architecture. | Absent should be ok. | This is related to MPR, the RF requirements of IAB-MT are different with UE. |
|  | 2-17 | PA architectures for intra-band UL CA | Mandatory to support either single or dual PA architectures with capability if UE supports intra-band CA configuration in uplink. The capability signalling is absent if UE supports single PA architecture | Absent should be ok. | This is related to MPR, the RF requirements of IAB-MT are different with UE. |
| 3. Baseband | 3-1 | Independent measurement gap configurations for FR1 and FR2 | Optional with capability signalling |  |  |
|  | 3-2 | Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement | Optional with capability signalling |  |  |
|  | 3-3 | Short measurement gap | Optional with capability signalling |  |  |
|  | 3-4 | SU-MIMO Interference Mitigation advanced receiver | Optional without capability signalling |  |  |

The changes are summarized as followings,

- 1-6 (pi/2-BPSK for PUSCH) and 1-7 (pi/2-BPSK for PUCCH format 3/4) are changed to optional for FR2.

- 2-1 (Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC) and 2-8 (UE power class) need to be considered by RAN2. Send LS to RAN2 to be aware of the views on 2-8.

- 2-10 (Multiple frequency band indication) and 2-12 (Multiple NS/P-Max) are not needed.

- 2-16 (PA architectures for intra-band EN-DC) and 2-17 (PA architectures for intra-band UL CA) can be set absent.

A LS needs to be sent to RAN2 to let RAN2 know RAN4 views and consider if there’s any signaling imapct. A draft LS is privided in [3].

- Conclusion

This contribution provides our views for IAB RF and RRM features according to R15 UE features. The changes are summarized as followings,

- 1-6 (pi/2-BPSK for PUSCH) and 1-7 (pi/2-BPSK for PUCCH format 3/4) are changed to optional for FR2.

- 2-1 (Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC) and 2-8 (UE power class) need to be considered by RAN2. Send LS to RAN2 to be aware of the views on 2-8.

- 2-10 (Multiple frequency band indication) and 2-12 (Multiple NS/P-Max) are not needed.

- 2-16 (PA architectures for intra-band EN-DC) and 2-17 (PA architectures for intra-band UL CA) can be set absent.

A LS needs to be sent to RAN2 to let RAN2 know RAN4 views and consider if there’s any signaling imapct. A draft LS is privided in [3].

- Reference

[1] RP-200501, “Way forward for mandatory features for wide-area IAB-nodes”, Ericsson

[2] TR 38.822, vf01

[3] R4-2003317, “LS on RAN4 IAB-MT feature list agreement”, CATT