3GPP TSG-RAN4 Meeting #94bis-e	R4-2003405

# Online, 20th - 30th Apr 2020

Agenda item:	6.7.3

Source:	Apple

Title:		On demod test case with max MIMO layer adaptation

Document for:	Discussion

# Introduction

The per-BWP max MIMO layer adaptation is a feature introduced under UE power saving feature in R16, and it can help UE to save power if network can indicate UE the maximum MIMO layer number by using the DCI command like active BWP change. However, we also need to consider the impact to other existing baseband requirements after introducing this max MIMO layer adaptation to serve power saving purpose. In last RAN4 meeting, a WF was approved in [1] and it postponed discussion on Demod requirement impacts due to MIMO layer adaption to the WI performance part.

In this contribution, we discuss the impact of max MIMO layer adaptation to demod requirement.

# Discussion on impacts on demod requirement

The previous max MIMO layer number is configured per-cell/per-CC via RRC signaling, which is almost static activity. However, in order to help UE to save power, the max MIMO layer number is updated to be configured per-BWP and also the DCI command can be used for maximum MIMO layer number change in the BWP. The power saving gain in max MIMO layer adaptation is because UE can adaptively switch off or deactivate some Rx antenna when the max MIMO layer number is reduced to a smaller number, e.g. change from 4 to 2. For instance, UE has to at least keep 4 Rx antenna ‘ON’ when it is configured with max_MIMO_layer_num=4; but as long as network configures UE to adjust max_MIMO_layer_num to 2, UE can only use 2 Rx antenna and turn off or deactivate the other Rx antennas to save power.

However, in the existing demodulation requirement in TS38.101-1, it is specified that,

## 7.2 Diversity characteristics The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n77, n78, n79 where the UE is required to be equipped with a minimum of four Rx antenna ports. This requirement applies when the band is used as a standalone band or as part of a band combination. For the requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands. Additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports.

And in TS38.101-4, it is specified that,

#### 5.1.1.2 Applicability of requirements for different number of RX antenna ports The number of RX antenna ports for different RF operating bands is up to UE declaration. The UE shall support 2 or 4 RX antenna ports for different RF operating bands. The operating bands, where 4 RX antenna ports shall be the baseline, are defined in Clause 7.2 of TS 38.101-1 [6]. The UE requirements applicability for UEs with different number of RX antenna ports is defined in Table 5.1.1.2-1. Table 5.1.1.2-1: Requirements applicability Supported RX antenna ports; Test type; Test list. UE supports only 2RX; PDSCH; All tests in Clause 5.2.2. PDCCH; All tests in Clause 5.3.2. PBCH; All tests in Clause 5.4.2. UE supports only 4RX or both 2RX and 4RX; PDSCH; All tests in Clause 5.2.3. PDCCH; All tests in Clause 5.3.3. PBCH; All tests in Clause 5.4.2 or 5.4.3 (Note). Note: Requirements for PBCH with 4Rx is up to UE declaration.

That means in certain bands (e.g. n7, n38, n41, n77, n78, n79) UE is mandated to use 4Rx and the corresponding demod test cases are designed based on the side condition of 4Rx SNR. For instance, in the section 5.2.2 and 5.2.3 of TS38.101-4, the minimum performance requirement for PDSCH are as below,

| 2Rx performance requirement | Table 5.2.2.1.1-4: Minimum performance for Rank 2 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 2-1; R.PDSCH.1-3.1 FDD; 10 / 15; 64QAM, 0.50; TDLA30-10; 2x2, ULA Low; 70; 19.4. 2-2; R.PDSCH.2-1.1 FDD; 20 / 30; 64QAM, 0.50; TDLA30-10; 2x2, ULA Low; 70; 19.7. |
| --- | --- |
| 4Rx performance requirement | Table 5.2.3.1.1-4: Minimum performance for Rank 2 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 2-1; R.PDSCH.1-3.1 FDD; 10 / 15; 64QAM, 0.50; TDLA30-10; 2x4, ULA Low; 70; 13.5. 2-2; R.PDSCH.2-1.1 FDD; 20 / 30; 64QAM, 0.50; TDLA30-10; 2x4, ULA Low; 70; 13.7. |

The side condition of 2Rx is almost 6 dB higher than 4Rx case, and therefore if UE with rank 2 is only using 2Rx for reception to save power, there is not sufficient diversity gain for UE to pass the 4Rx test cases. So it seems like that if we follow the existing demod requirement to mandate UE to use 4Rx on certain band, we cannot achieve any power saving gain from max MIMO layer adaptation from “4” to “2”. We suggest RAN4 to consider whether UE still needs to meet 4Rx demod requirement on those 4Rx-mandated bands when network configures the max_MIMO_layer_num=2 to UE to save power. Based on the comments from companies in last meeting, the option of simply setting max_MIMO_layer_num =4 in the all related test cases looks more acceptable to companies, so we propose,

Proposal 1: set the max_MIMO_layer_num =4 in the all related test cases applied for 4Rx-mandated bands

# Conclusion

In this contribution, we discuss the impact of max MIMO layer adaptation to demod requirement.

Proposal 1: set the max_MIMO_layer_num =4 in the all related test cases applied for 4Rx-mandated bands

# References

[1] R4-2002338, WF on RRM measurement relaxation for Power Saving, CATT, Vivo, RAN4 #94e meeting