3GPP TSG-RAN4 #94bis-e 	R4-2003421

Online, 20th – 30th Apr. 2020

Source:	Charter Communications

Title:	TP for TR 38.716-01-01 for CA_n48(3A) and CA_n48(4A)

Agenda item:			8.1.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal to include TP for TP for TR 38.716-01-01 for CA_n48(3A) and n48(4A)

as defined in WID [1].

# 2. Text Proposal

# ---Start of changes---

## 6.x	CA_2DL_n48(3A)_1UL_n48A, CA_2DL_n48(4A)_1UL_n48A

### 6.x.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: Supported bandwidth combinations for CA_2DL_n48(3A)_1UL _n48A

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_n48(3A) | - | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 |  |  | 1402 |  |
| NOTE 1: 5MHz is not applicable for 30/60kHz SCS NOTE 2: Parameter value accounts for both, the maximum frequency range of band n48 (150MHz), and the minimum frequency gaps in between NR non-contiguous component carriers |  |  |  |  |  |  |  |  |

Table 6.x.1-2: Supported bandwidth combinations for CA_2DL_n48(4A)_1UL _n48A

|  |  |  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration |  | Uplink CA configurations |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] |  | Bandwidth combination set |  |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_n48(4A) | - | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 1402 |  |
| NOTE 1: 5MHz is not applicable for 30/60kHz SCS NOTE 2: Parameter value accounts for both, the maximum frequency range of band n48 (150MHz), and the minimum frequency gaps in between NR non-contiguous component carriers |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.x.2	Co-existence studies

There are no co-existence issues for this combination.

### 6.x.3	REFSENS

There are no REFSENS exceptions for this combination. However, UL configuration for REFSENS needs to be captured after general principles for RX requirements have been agreed

---End of changes---

# Reference

[1] 		RP-200201, “Revised WID on Rel-16 NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) ”, Ericsson