3GPP TSG-RAN WG4 Meeting # 94-e-Bis 	R4-2003426

# Electronic Meeting, 20 – 30 Apr., 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR |  | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Demod requirement for 5G broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast -Performance |  |  |  |  |  |  | Date: |  | 2020/04/10 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add the deomod requirement for Rel-16 5G broadcast |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rel-16 5G broadcast demod requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The demod requirement for Rel-16 5G broadcast not be added. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4 (new), A.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections>>>

## 10.4 FDD with 5G Terrestrial Broadcast

For the requirements defined in this section, the difference between CRS EPRE and the MBSFN RS EPRE should be set to 16dB for subcarrier spacing as 0.37kHz, 7.8 dB for subcarrier spacing as 2.5kHz because the UE demodulation performance might be different when this condition is not met (e.g. in scenarios where power offsets are present, such as scenarios when reserved cells are present).

### 10.4.1	Minimum requirement for PMCH decoding

#### 10.4.1.1	Minimum requirement with 0.37kHz subcarrier spacing

The receive characteristic of MBMS is determined by the BLER. [Applicability rule TBD.]

For the parameters specified in Table 10.4.1.1-1 and [Reference channel TBD], the average downlink SNR shall be below the specified value for the BLER shown in Table 10.4.1.1-2.

Table 10.4.1.1-1: Test Parameters for Testing

| Parameter |  | Unit |  |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode in PCell |  |  | 1 |
| Subcarrier spacing for MBSFN cell |  | kHz | 0.37 kHz |
| Bandwidth |  | MHz | 10(Note 2) |
| Note 1: . Note 2: For both Pcell and Scell |  |  |  |

Table 10.4.1.1-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x1 | NA | NA | NA |
|  | MBMS Dedicated Cell | 10 | [TBD] | NA | [TBD] | 1x1 | 1 | [TBD] | ≥2 |
| 2 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x1 | NA | NA | NA |
|  | MBMS Dedicated Cell | 10 | [TBD] | NA | [TBD] | 1x1 | 1 | [TBD] | ≥2 |

#### 10.3.1.2	Minimum requirement with 2.5kHz subcarrier spacing

The receive characteristic of MBMS is determined by the BLER. [Applicability rule TBD.].

For the parameters specified in Table 10.4.1.2-1 and [Reference channel TBD], the average downlink SNR shall be below the specified value for the BLER shown in Table 10.4.1.2-2.

Table 10.4.1.2-1: Test Parameters for Testing

| Parameter |  | Unit |  |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | at antenna port |
| PDSCH transmission mode in PCell |  |  | 1 |
| Subcarrier spacing for MBSFN cell |  | kHz | 2.5 kHz |
| Bandwidth |  | MHz | 10(Note 2) |
| Note 1: . Note 2: For both Pcell and Scell |  |  |  |

Table 10.4.1.2-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x2 low | NA | NA | NA |
|  | MBMS Dedicated Cell | 10 | [TBD] | NA | [TBD] | 1x2 low | 1 | [TBD] | ≥2 |

### 10.4.2	Minimum requirement for CAS detection

#### 10.4.2.1	Minimum requirement for PBCH detection

For the parameters specified in Table 8.6.1-1 and repetition pattern specified in 10.4.2.1-1, the average probability of a miss-detected PBCH (Pm-bch) shall be below the specified value in Table 10.4.2.1-2. The downlink physical setup is in accordance with Annex C.3.2.

Table 10.4.2.1-1: PBCH repetition pattern

| l | Slot and symbol number pair ns',l' |  |
| --- | --- | --- |
|  | Normal cyclic prefix | Extended cyclic prefix |
| 0 | (0, 4) | - |
| 1 | (1, 4) | (0, 3) |
| 2 | (1, 5) | (1, 4) |
| 3 | (0, 3), (1, 6) | (1, 5) |

Table 10.4.2.1-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-bch (%) | SNR(dB) |  |
| 1 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x1 low | NA | NA | NA |
|  | MBMS Dedicated Cell | 10 | R.24 | NA | AWGN | 1x1 low | 1 | [TBD] | ≥2 |
| 2 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x2 low | NA | NA | NA |
|  | MBMS Dedicated Cell | 10 | R.24 | NA | EVA 162Hz | 1x2 low | 1 | [TBD] | ≥2 |

<<< End of changed sections>>>

<<< Start of changed sections>>>

## A.3.7 Reference measurement channels for PBCH performance requirements

Table A.3.7-1: Reference Channel FDD/TDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.21 | R.22 | R.23 | R.24 |
| Number of transmitter antennas |  | 1 | 2 | 4 | 1 |
| Channel bandwidth | MHz | 1.4 | 1.4 | 1.4 | 1.4 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 40/1920 | 40/1920 | 40/1920 | 40/4416 |
| Payload (without CRC) | Bits | 24 | 24 | 24 | 24 |

<<< End of changed sections>>>