3GPP TSG RAN WG4 Meeting #94bis-e	R4-2003441

# Online, 20th – 30th April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | <CR#> | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-3: Support of n77(2A) in DC_42_n77 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Softbank Corp. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R16_1BLTE_1BNR_2DL2UL |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | DC combos of Band 42_n77 are updated to add DL n77(2A). |  |  |  |  |  |  |  |  |
| Summary of change: |  | DC_42A/C_n77(2A) are added.(This paper is intended to skip a TP since there is no new technical study needed.) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DC_42A/C_n77(2A) is not supported properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4.1 (Table 5.5B.4.1-1) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Unaffected parts omitted]

5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_1A_n3A DC_1C_n3A | DC_1A_n3A DC_1C_n3A | DC_1_n3 |
| DC_1A_n5A | DC_1A_n5A | No |
| DC_1A_n7A DC_1A_n7B | DC_1A_n7A | No |
| DC_1A-1A_n7A DC_1A-1A_n7B | DC_1A_n7A | No |
| DC_1A_n8A | DC_1A_n8A | No |
| DC_1A_n28A | DC_1A_n28A | No |
| DC_1A_n38A DC_1C_n38A | DC_1A_n38A | No |
| DC_1A_n40A | DC_1A_n40A | No |
| DC_1A_n41A | DC_1A_n41A | No |
| DC_1A_n50A | DC_1A_n50A | No |
| DC_1A_n51A | DC_1A_n51A | No |
| DC_1A_n77A7 DC_1A_n77C7 | DC_1A_n77A | DC_1_n77 |
| DC_1A_n77(2A) | DC_1A_n77A | DC_1_n77 |
| DC_1A_n78A7 DC_1A_n78C7 | DC_1A_n78A | No |
| DC_1A_n78(2A)7 | DC_1A_n78A | No |
| DC_1A_n79A7 DC_1A_n79C7 | DC_1A_n79A | No |
| DC_2A_n5A | DC_2A_n5A | No |
| DC_2A-2A_n5A | DC_2A_n5A | No |
| DC_2A_n7A | DC_2A_n7A | No |
| DC_2A_n7(2A) | DC_2A_n7A | No |
| DC_2A_n12A | DC_2A_n12A | No |
| DC_2A_n38A | DC_2A_n38A | No |
| DC_2A-2A_n38A | DC_2A_n38A | No |
| DC_2A_n41A DC_2A_n41C DC_2C_n41A | DC_2A_n41A DC_2C_n41A | No |
| DC_2A-2A_n41A DC_2A_n41(2A) | DC_2A_n41A | No |
| DC_2A_n48A DC_2A_n48B | DC_2A_n48A | No |
| DC_2A_n66A | DC_2A_n66A | DC_2_n66 |
| DC_2A-2A_n66A | DC_2A_n66A | DC_2_n66 |
| DC_2A_n71A DC_2A_n71B DC_2C_n71A | DC_2A_n71A DC_2C_n71A | No |
| DC_2A-2A_n71A | DC_2A_n71A | No |
| DC_2A_n78A | DC_2A_n78A | DC_2_n78 |
| DC_2A_n78(2A) | DC_2A_n78A | DC_2_n78 |
| DC_2A-2A_n78A | DC_2A_n78A | DC_2_n78 |
| DC_3A_n1A DC_3C_n1A | DC_3A_n1A DC_3C_n1A | DC_3_n1 |
| DC_3A-3A_n1A | DC_3A_n1A | DC_3_n1 |
| DC_3A_n5A DC_3C_n5A | DC_3A_n5A DC_3C_n5A | DC_3_n5 |
| DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B | DC_3A_n7A DC_3A_n7B DC_3C_n7A | No |
| DC_3A-3A_n7A DC_3A-3A_n7B | DC_3A_n7A | No |
| DC_3A_n8A | DC_3A_n8A | No |
| DC_3A_n20A | DC_3A_n20A | No |
| DC_3A_n28A DC_3C_n28A | DC_3A_n28A DC_3C_n28A | No |
| DC_3A_n34A | DC_3A_n34A | No |
| DC_3A_n38A DC_3C_n38A | DC_3A_n38A | No |
| DC_3A_n40A | DC_3A_n40A | No |
| DC_3A_n41A DC_3C_n41A | DC_3A_n41A DC_3C_n41A | DC_3_n41 |
| DC_3A_n50A | DC_3A_n50A | No |
| DC_3A_n51A | DC_3A_n51A | No |
| DC_3A_n77A7 DC_3A_n77C7 | DC_3A_n77A | DC_3_n77 |
| DC_3A_n77(2A)7 | DC_3A_n77A | DC_3_n77 |
| DC_3A-3A_n77A | DC_3A_n77A | DC_3_n77 |
| DC_3A_n78A7 DC_3A_n78C7 DC_3C_n78A7 | DC_3A_n78A | DC_3_n78 |
| DC_3A_n78(2A)7 DC_3C_n78(2A)7 | DC_3A_n78A | DC_3_n78 |
| DC_3A-3A_n78A | DC_3A_n78A | DC_3_n78 |
| DC_3A_n79A7 DC_3A_n79C7 DC_3C_n79A7 | DC_3A_n79A DC_3C_n79A | No |
| DC_4A_n38A | DC_4A_n38A | No |
| DC_4A_n41A | DC_4A_n41A | No |
| DC_4A_n78A | DC_4A_n78A | No |
| DC_4A_n78(2A) | DC_4A_n78A | No |
| DC_5A_n2A DC_5B_n2A | DC_5A_n2A | No |
| DC_5A-5A_n2A | DC_5A_n2A | No |
| DC_5A_n7A | DC_5A_n7A | DC_5_n7 |
| DC_5A_n7(2A) | DC_5A_n7A | DC_5_n7 |
| DC_5A_n12A | DC_5A_n12A | No |
| DC_5A_n38A | DC_5A_n38A | DC_5_n38 |
| DC_5A_n40A | DC_5A_n40A | No |
| DC_5A_n48A DC_5A_n48B | DC_5A_n48A | No |
| DC_5A_n66A DC_5B_n66A | DC_5A_n66A | DC_5_n66 |
| DC_5A-5A_n66A | DC_5A_n66A | DC_5_n66 |
| DC_5A_n71A | DC_5A_n71A | No |
| DC_5A_n78A7 | DC_5A_n78A | No |
| DC_5A_n78(2A)7 | DC_5A_n78A | No |
| DC_5A_n79A | DC_5A_n79A | No |
| DC_7A_n1A DC_7C_n1A | DC_7A_n1A DC_7C_n1A | No |
| DC_7A-7A_n1A | DC_7A_n1A | No |
| DC_7A_n3A DC_7C_n3A | DC_7A_n3A DC_7C_n3A | No |
| DC_7A_n5A DC_7C_n5A | DC_7A_n5A DC_7C_n5A | DC_7_n5 |
| DC_7A-7A_n5A | DC_7A_n5A | DC_7_n5 |
| DC_7A_n8A | DC_7A_n8A | No |
| DC_7A-7A_n78A7 | DC_7A_n78A | No |
| DC_7A_n28A DC_7C_n28A | DC_7A_n28A DC_7C_n28A | No |
| DC_7A_n51A | DC_7A_n51A | No |
| DC_7A_n66A DC_7C_n66A | DC_7A_n66A | No |
| DC_7A-7A_n66A | DC_7A_n66A | No |
| DC_7A_n71A | DC_7A_n71A | No |
| DC_7A_n77A | DC_7A_n77A | No |
| DC_7A-7A_n77A | DC_7A_n77A | No |
| DC_7A_n78A7 DC_7C_n78A7 | DC_7A_n78A DC_7C_n78A | No |
| DC_7A_n78(2A)7 DC_7C_n78(2A)7 | DC_7A_n78A DC_7C_n78A | No |
| DC_8A_n1A | DC_8A_n1A | No |
| DC_8A_n3A | DC_8A_n3A | No |
| DC_8A_n28A | DC_8A_n28A | No |
| DC_8A_n34A | DC_8A_n34A | No |
| DC_8A_n39A | DC_8A_n39A | No |
| DC_8A_n40A7 | DC_8A_n40A | No |
| DC_8A_n41A DC_8A_n41C | DC_8A_n41A | No |
| DC_8A_n41(2A) | DC_8A_n41A | No |
| DC_8A_n77A7 | DC_8A_n77A | No |
| DC_8A_n77(2A)7 | DC_8A_n77A | No |
| DC_8A_n78A7 | DC_8A_n78A | No |
| DC_8A_n79A7 DC_8A_n79C | DC_8A_n79A DC_8A_n79C | No |
| DC_8A_n93A | DC_8A_n93A_ULSUP-TDM | N/A |
| DC_8A_n94A | DC_8A_n94A_ULSUP-TDM | N/A |
| DC_11A_n3A | DC_11A_n3A | No |
| DC_11A_n77A7 | DC_11A_n77A | No |
| DC_11A_n77(2A)7 | DC_11A_n77A | No |
| DC_11A_n78A7 | DC_11A_n78A | No |
| DC_11A_n79A7 | DC_11A_n79A | No |
| DC_12A_n2A | DC_12A_n2A | No |
| DC_12A_n5A | DC_12A_n5A | No |
| DC_12A_n7A DC_12A_n7(2A) | DC_12A_n7A | No |
| DC_12A_n25A | DC_12A_n25A | No |
| DC_12A_n38A | DC_12A_n38A | No |
| DC_12A_n66A | DC_12A_n66A | No |
| DC_12A_n78A DC_12A_n78(2A) | DC_12A_n78A | DC_12_n78 |
| DC_13A_n7A DC_13A_n7(2A) | DC_13A_n7A | No |
| DC_13A_n48A DC_13A_n48B | DC_13A_n48A | No |
| DC_13A_n66A | DC_13A_n66A | No |
| DC_13A_n71A | DC_13A_n71A | No |
| DC_13A_n78A DC_13A_n78(2A) | DC_13A_n78A | No |
| DC_18A_n3A | DC_18A_n3A | No |
| DC_18A_n77A7 | DC_18A_n77A | No |
| DC_18A_n78A7 | DC_18A_n78A | No |
| DC_20A_n91A | DC_20A_n91A_ULSUP-TDM | N/A |
| DC_20A_n92A | DC_20A_n92A_ULSUP-TDM | N/A |
| DC_18A_n79A7 | DC_18A_n79A | No |
| DC_19A_n77A7 DC_19A_n77C7 | DC_19A_n77A | No |
| DC_19A_n78A7 DC_19A_n78C7 | DC_19A_n78A | No |
| DC_19A_n79A7 DC_19A_n79C7 | DC_19A_n79A | No |
| DC_20A_n1A | DC_20A_n1A | No |
| DC_20A_n3A | DC_20A_n3A | No |
| DC_20A_n7A | DC_20A_n7A | DC_20_n7 |
| DC_20A_n8A | DC_20A_n8A | DC_20_n8 |
| DC_20A_n28A8,10,11,13 | DC_20A_n28A | No |
| DC_20A_n38A | DC_20A_n38A | No |
| DC_20A_n41A | DC_20A_n41A | DC_20_n41 |
| DC_20A_n50A | DC_20A_n50A | No |
| DC_20A_n51A | DC_20A_n51A | No |
| DC_20A_n77A7 | DC_20A_n77A | No |
| DC_20A_n78A7 | DC_20A_n78A | No |
| DC_21A_n77A7 DC_21A_n77C7 | DC_21A_n77A | No |
| DC_21A_n78A7 DC_21A_n78C7 | DC_21A_n78A | No |
| DC_21A_n79A7 DC_21A_n79C7 | DC_21A_n79A | No |
| DC_25A_n41A | DC_25A_n41A | No |
| DC_25A-25A_n41A | DC_25A_n41A | No |
| DC_26A_n25A | DC_26A_n25A | No |
| DC_26A_n41A | DC_26A_n41A | No |
| DC_26A_n77A7 | DC_26A_n77A | No |
| DC_26A_n78A7 | DC_26A_n78A | No |
| DC_26A_n79A7 | DC_26A_n79A | No |
| DC_28A_n3A | DC_28A_n3A | No |
| DC_28A_n5A8 | DC_28A_n5A | No |
| DC_28A_n7A DC_28A_n7B | DC_28A_n7A DC_28A_n7B | No |
| DC_28A_n51A | DC_28A_n51A | No |
| DC_28A_n8A | DC_28A_n8A | No |
| DC_28A_n41A | DC_28A_n41A | No |
| DC_28A_n50A | DC_28A_n50A | No |
| DC_28A_n77A7 DC_28A_n77C7 | DC_28A_n77A | No |
| DC_28A_n77(2A)7 | DC_28A_n77A | No |
| DC_28A_n78A7 DC_28A_n78C7 | DC_28A_n78A | No |
| DC_28A_n79A7 DC_28A_n79C7 | DC_28A_n79A | No |
| DC_30A_n2A | DC_30A_n2A | No |
| DC_30A_n5A | DC_30A_n5A | No |
| DC_30A_n66A | DC_30A_n66A | No |
| DC_38A_n78A7 | DC_38A_n78A | No |
| DC_39A_n40A3 | DC_39A_n40A | No |
| DC_39A_n41A DC_39C_n41A | DC_39A_n41A DC_39C_n41A | No |
| DC_39A_n78A5,7 | DC_39A_n78A | No |
| DC_39A_n79A7 | DC_39A_n79A | No |
| DC_40A_n1A | DC_40A_n1A | No |
| DC_40A_n41A3 DC_40C_n41A3 | DC_40A_n41A | No |
| DC_40A_n77A | DC_40A_n77A | No |
| DC_40A_n78A DC_40C_n78A | DC_40A_n78A DC_40C_n78A | No |
| DC_40A_n79A7,12 DC_40C_n79A7,12 | DC_40A_n79A | No |
| DC_41A_n77A DC_41C_n77A | DC_41A_n77A DC_41C_n77A | No |
| DC_41A_n77(2A) DC_41C_n77(2A) | DC_41A_n77A DC_41C_n77A | No |
| DC_41A_n78A DC_41C_n78A | DC_41A_n78A DC_41C_n78A | No |
| DC_41A_n79A6,7 DC_41C_n79A6,7 | DC_41A_n79A DC_41C_n79A | No |
| DC_42A_n51A | DC_42A_n51A | No |
| DC_42A_n77A3,4,9,11 DC_42A_n77C3,4,9,11 DC_42C_n77A3,4,9,11 DC_42C_n77C3,4,9,11 DC_42D_n77A3,4,9,11 DC_42D_n77C DC_42E_n77A3,4,9,11 DC_42E_n77C | N/A | N/A |
| DC_42A_n77(2A)3,4,9,11 DC_42C_n77(2A)3,4,9,11 | N/A | N/A |
| DC_42A_n78A3,4,9,11 DC_42A_n78C3,4,9,11 DC_42C_n78A3,4,9,11 DC_42C_n78C3,4,9,11 DC_42D_n78A3,4,9,11 DC_42D_n78C DC_42E_n78A3,4,9,11 DC_42E_n78C | N/A | N/A |
| DC_42A_n79A9 DC_42A_n79C9 DC_42C_n79A9 DC_42C_n79C9 DC_42D_n79A9 DC_42D_n79C DC_42E_n79A9 DC_42E_n79C | N/A | N/A |
| DC_46A_n78A2 DC_46C_n78A2 DC_46D_n78A2 DC_46E_n78A2 | N/A | N/A |
| DC_48A_n5A | DC_48A_n5A | No |
| DC_48A_n12A | DC_48A_n12A | No |
| DC_48A_n66A | DC_48A_n66A | No |
| DC_48A_n71A DC_48B_n71A DC_48C_n71A DC_48D_n71A | DC_48A_n71A | No |
| DC_66A_n2A | DC_66A_n2A | DC_66_n2 |
| DC_66A-66A_n2A | DC_66A_n2A | DC_66_n2 |
| DC_66A_n5A | DC_66A_n5A | DC_66_n5 |
| DC_66A-66A_n5A DC_66A-66A-66A_n5A | DC_66A_n5A | DC_66_n5 |
| DC_66A_n7A DC_66A-66A_n7A DC_66A-66A_n7(2A) | DC_66A_n7A | No |
| DC_66A_n12A | DC_66A_n12A | No |
| DC_66A_n25A | DC_66A_n25A | DC_66_n25 |
| DC_66A_n38A | DC_66A_n38A | No |
| DC_66A-66A_n38A | DC_66A_n38A | No |
| DC_66A_n41A DC_66A_n41C | DC_66A_n41A | No |
| DC_66A_n41(2A) | DC_66A_n41A | No |
| DC_66A_n48A DC_66A_n48B | DC_66A_n48A | No |
| DC_66A-66A_n48A DC_66A-66A_n48B | DC_66A_n48A | No |
| DC_66A_n71A DC_66C_n71A DC_66A_n71B | DC_66A_n71A | No |
| DC_66A-66A_n71A | DC_66A-66A_n71A | DC_66A-66A_n71A |
| DC_66A_n78A | DC_66A_n78A | No |
| DC_66A_n78(2A) | DC_66A_n78A | No |
| DC_66A-66A_n78A | DC_66A_n78A | No |
| DC_71A_n5A | DC_71A_n5A | No |
| DC_71A_n38A | DC_71A_n38A | No |
| DC_71A_n48A | DC_71A_n48A |  |
| DC_71A_n66A | DC_71A_n66A | No |
| DC_71A_n78A | DC_71A_n78A | No |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: The minimum requirements for intra-band contiguous or non-contiguous EN-DC apply. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 5: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 8: The frequency range in band n28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. NOTE 9: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. NOTE 10: The maximum power spectral density imbalance between downlink carriers is within [6] dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 11: The minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers is within [6] dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 12: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 13: The minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |

[Unaffected parts omitted]