3GPP TSG RAN WG4 Meeting #94bis-e	R4-2003454

Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:	8.4.2

Source:	SoftBank Corp.

Title:	TP for TR 37.716-21-11: EN-DC_3-8_n28

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_3-8_n28 was approved in RAN#87[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-8_n28 | CA_3-8 | n28 | - |

Table 2: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-8A_n28A | DC_3A_n28A DC_8A_n28A | CA_3A-8A | n28A |

3.  Co-existence

Based on the co-existence studies of DC_3A-n28A and DC_8A-n28A are captured in TR 37.716-11-11 and 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

4.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked.

Table 3: ΔTIB of constituent 2 band combo.

| ΔTIB | B3+B8 | B3+n28 | B8+n28 | Proposed Value |
| --- | --- | --- | --- | --- |
| B3 | 0.3 | 0.3 |  | 0.3 |
| B8 | 0.3 |  | 0.6 | 0.6 |
| n28 |  | 0.3 | 0.5 | 0.5 |

Table 4: ΔRIB of constituent 2 band combo.

| ΔRIB | B3+B8 | B3+n28 | B8+n28 | Proposed Value |
| --- | --- | --- | --- | --- |
| B3 | 0 | 0 |  | 0 |
| B8 | 0 |  | 0.2 | 0.2 |
| n28 |  | 0 | 0.1 | 0.1 |

Then the proposal for this 2LTE-1NR EN-DC is:

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-8_n28 | 3 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-8_n28 | 3 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.1 |

5.  REFSENS

As mentioned in section 3, REFSENS exceptions are not expected.

6.  Conclusion

This paper proposes study results of EN-DC 3-8_n28.

TP is shown in Annex.

7.  Reference

[1]  RP-200271  WID revision: Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL); rapporteur: Huawei

Annex.   Proposed text for TR37.716-21-11:

[Unchanged Parts Skipped]

5.1.x	DC_3-8_n28

### 5.1.x.1	Operating bands for DC_3-8_n28

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-8_n28 | CA_3-8 | n28 | - |

### 5.1.x.2	Configurations for DC_3-8_n28

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-8A_n28A | DC_3A_n28A DC_8A_n28A | CA_3A-8A | n28A |

### 5.1.x.3	Co-existence studies

There is no additional harmonic and intermodulation impact for the additional band receiver.

### 5.1.x.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.x.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-8_n28 | 3 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |

Table 5.1.x.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-8_n28 | 3 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.1 |

### 5.1.x.5	REFSENS requirements

As mentioned in 5.1.x.3, REFSENS exceptions are not expected.

[Unchanged Parts Skipped]