3GPP TSG RAN WG4 Meeting #94bis-e	R4-2003460

Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:	8.11.2

Source:	SoftBank Corp.

Title:	TP update for TR 38.716-03-02: CA_n3-n28-n77

Document for:	Approval

1	Introduction

TP for CA_n3-n28-n77, was approved in RAN4#94 [1] but REFSENS exceptions have errors and TBD parts. This TP is to update the REFSENS exception results for 3rd IMD.

2	Text Proposal

[Unchanged Parts Skipped]

5.1.7	CA_n3-n28-n77

5.1.7.1	Operating bands for CA

Table 5.1.7.1-1: CA band combination of band n3+n28+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

5.1.7.2	Channel bandwidths per operating band for CA

Table 5.1.7.2-1: Supported bandwidths per CA band combination of band n3+n28+n77

| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n28A-n77A | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

5.1.7.3	UE co-existence studies

The coexistence studies of UL Band n3 + Band n28, Band n3 + Band n77 and Band n28 + Band n77 have already been captured in the constituent fallback modes in TR 38.716-02-00. According to the coexistence studies in TR 38.716-02-00, the own Rx impact of the 3rd band is shown as the followings.

- 3rd, 4th and 5th order IMD generated by dual uplink of Band n3 + Band n28 may fall into own Rx of Band n77

- 3rd and 4th order IMD generated by dual uplink of Band n28 + Band n77 may fall into own Rx of Band n3.

- 3rd order IMD generated by dual uplink of Band n3 + Band n77 may fall into own Rx of Band n28.

5.1.7.4	∆TIB and ∆RIB values

For three DLs and two ULs of Band n3, n28 and n77, the same DTIB,c and DRIB,c values specified for 1 band UL for CA_n3-n28-n77 are used as below.

Table 5.1.7.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n77 | n3 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 5.1.7.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n77 | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

5.1.7.5	REFSENS requirements

Table 5.1.7.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. For the exception by dual uplink of Band n28 + Band n77, the same exception value in DC_3A-28A_n77A is used and for the exception by dual uplink of Band n3 + Band n28 and Band n3 + Band n77, the same exception values in DC_3A_n28A-n77A is used.

Table 5.1.7.5-1: MSD for the CA configuration

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n3A-n28A-n77A | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n77 | 4173 | 10 | 50 | 4173 | 15.9 | IMD3 ¦2*fBn3+fn28¦ |
| CA_n3A-n28A-n77A | n28 | 735 | 5 | 25 | 790 | N/A | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 17.0 | IMD3 ¦2*fBn28+fnn77¦ |
| CA_n3A-n28A-n77A | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |
|  | n28 | 715 | 5 | 25 | 770 | 15.3 | IMD3 ¦2*fBn3+fnn77¦ |

[Unchanged Parts Skipped]

7.  Reference

[1]  R4-2002670  TP for TR 38.716-03-02: CA_n3-n28-n77, SoftBank Corp.