3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2003678

# Electronic Meeting, 20th Apr – 30th Apr, 2020

Title: 	Discussion on URLLC high reliability test feasibility

Source: 	Huawei, HiSilicon

Agenda item:	6.9.1.1

Document for:	Discussion

Background

The test feasibility for URLLC ultra-low reliability were discussed during #94 e-meeting. According to the WF[1], some agreements were made and some open issues were remained to be discussed further:

Agreements:

Decision frequency (N) is 1 (i.e. early pass/fail evaluation on every detected block error)

SCS for UE: 15kHz for FDD, 30kHz for TDD

SCS for BS: 15kHz and 30kHz

TDD pattern: 7D1S2U (S=6:4:4) for UE

Slot aggregation factor:

- Slot aggregation factor is 1 if X (see slide 12) is <3dB

- Slot aggregation factor is [1, 2] if X>=3dB

CP-OFDM

- DFT-S-OFDM is FFS

Antenna configuration for BS: 1x2

Antenna configuration for UE: 2x2 + 2x4 (2x4 agreed in 2nd round) ULA low with applicability rule

No PT-RS for FR1

PUSCH configuration: Start symbol 0, length 14 symbols

PDSCH configuration: Mapping type A, start symbol 2, length 12 symbols

MCS table: Use low spectrum efficiency MCS

Number of RB and bandwidth for PDSCH

- Full bandwidth for 10MHz/15k SCS and 40MHz/30Kscs

PUSCH mapping type

- Type A and type B with applicability rule

Reformulate the discussion as follows:

  - Test requirement = SNR for 10^-5 + IM + [X]

  - X is FFS and could be zero

  - FFS whether X appears in the core spec or test spec, or is just considered part of IM

Open issues:

Requirements and tests for FR2:

  - Option 1: Create requirements for FR2. No explicit applicability rule needed. Option 2: Create requirements for FR2 with applicability rule Proponents of option 2 please clarify what applicability rule you propose

  - Option 3: Do not create requirements for FR2

TDD patterns for BS

- Proposals FR1

    - Option 1: 3D1S1U (S=10:2:2) for 15kHz, 7D1S2U (S=6:4:4) for 30kHz

    - Option 2: SU or DSUU

PT-RS and DM-RS configuration for PUSCH

  - Option 1: 1+0

  - Option 2: 1+1

  - Option 3: 1+0 and 1+1 with applicability rule

DM-RS configuration for PDSCH

- Option 1: Zero additional DM-RS

- Option 2: One additional DM-RS

Number of RB for PUSCH

  - Option 1: 25 RB

  - Option 2: 65 RB

  - Option 3: Full bandwidth for 5MHz/15k SCS and 10MHz/30Kscs

  - Option 4: Full bandwidth with 10MHz/15k SCS and 40MHz/30k SCS

  - Option 5: Full bandwidth

  - Other options not precluded

Bandwidth for PUSCH

  - Option 1: 5MHz for 15k SCS, 10MHz for 30k SCS

  - Option 2: 10MHz for 15k SCS, 40MHz for 30k SCS

  - Option 3: 5/10/20MHz for 15k SCS, 10/20/40/100MHz for 30k SCS, applicability rule to ensure just one test

  - Option 4: Option 1+option 2

  - Other options not precluded

Value for X

  - Option 1: X is zero

  - Option 2: Not higher than ~0.5 dB

  - Option 3: More than 1dB, prefer 3dB

  - Other options not precluded

Whether X is specified in the core spec and if so whether it is explicit or part of the IM

  - Option 1: Core spec and explicit

  - Option 2: Core spec and implicit (in IM)

  - Option 3: Not in core spec

  - Note: Issue is only applicable if X is non-zero

Whether X is specified in the conformance spec, and if so whether it is explicit or part of the TT

  - Option 1: Conformance spec and explicit

  - Option 2: Conformance spec and implicit (in TT)

  - Option 3: Not in Conformance spec

  - Note: Issue is only applicable if X is non-zero

Number of requirements

  - Option 1: zero requirements/tests

  - Option 2: One requirement/test

  - Option 3: >1 requirements, but only one test (using applicability rule)

  - Option 4: more than one requirement and more than one test

  - Other options not precluded

CQI testing at ultra-low BLER

  - Option 1: Define CQI testing with ultra-low BLER

  - Option 2: Do not define CQI testing with ultra-low BLER

In this paper, the open issues are discussed and our views are provided.

Discussion

Requirements and test for FR2

We do not see the necessary to define the requirements for URLLC FR2. From current deployment request, FR2 is not a common use case for URLLC.

Proposal 1: No need to define the requirements and test for FR2.

Discussion on UE parameters

Based on the agreed parameters, the remained open issues for UE parameters are discussed in this section. As the test is defined for high reliability and symbol length of 12 has been agreed on the last meeting, we propose to use 1+1 as the DM-RS configuration.

The agreed and proposed UE parameters are concluded in Table 2.2-1:

Table 2.2-1: simulation assumption for UE PDSCH test requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Duplex mode |  | FDD/TDD |
| Antenna configuration |  | 2x2, ULA low 2x4, ULA low |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PUSCH aggregation factor | 1 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
| CSI-RS configuration | PeriodicityAndOffset | 5 slots, 0 slots |
|  | nrofPorts | 2 |
|  | frequencyDomainAllocation | Row3, ‘000001 |
|  | firstOFDMSymbolInTimeDomain | 7 |
|  | cdm-Type | CDM2 |
|  | Density | 1 |
| Number of HARQ Processes |  | 1 |
| Propagation condition |  | AWGN |
| MCS Table |  | Table 3, MCS 5 |
| SCS and BW |  | FDD:15KHz, 10MHz TDD:30KHz, 40MHz |
| Frequency domain resource |  | Full BW |
| TDD pattern |  | 7D1S2U (S=6:4:4) |
| Testing metric |  | Target BLER: 1-10-5 |

Proposal 2: We propose DM-RS configuration of 1+1 for UE.

Discussion on BS parameters

The parameters for BS are discussed in this section. Regarding to the TDD patterns, we propose to use the existing configurations. The agreed and proposed parameters are summarised in Table 2.3-1:

Table 2.3-1: Simulation assumption for BS PUSCH test requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Waveform |  | CD-OFDM |
| Antenna configuration |  | 1x2, ULA low |
| PUSCH configuration | Mapping type | Type A and Type B |
|  | Starting symbol (S) | 0 |
|  | Length (L) | 14 |
|  | PUSCH aggregation factor | 1 |
| PUSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
| CSI-RS configuration | PeriodicityAndOffset | 5 slots, 0 slots |
|  | nrofPorts | 2 |
|  | frequencyDomainAllocation | Row3, ‘000001 |
|  | firstOFDMSymbolInTimeDomain | 7 |
|  | cdm-Type | CDM2 |
|  | Density | 1 |
| Number of HARQ Processes |  | 1 |
| Propagation condition |  | AWGN |
| MCS Table |  | Table 3, MCS 5 |
| SCS and BW |  | FDD:15KHz, 10MHz TDD:30KHz, 40MHz |
| Frequency domain resource |  | Full BW |
| TDD pattern |  | 3D1S1U (S=10:2:2) 7D1S2U (S=6:4:4) |
| Testing metric |  | Target BLER: 1-10-5 |

Proposal 3: TDD patterns 3D1S1U (S=10:2:2) for 15 KHz, 7D1S2U (S=6:4:4) for 30 KHz.

Proposal 4: DM-RS configuration of 1+1 for BS.

Proposal 5: Bandwidth of 10MHz for 15 KHz and 40 MHz for 30 KHz with full bandwidth frequency allocation.

Test method

The test method was summarised as follows：

Test requirement = SNR for 10^-5 + IM + [X]

We prefer the value of X should not be too large, about 0.5-0.8dB. There is no need to capture X in the core and conformance specifications. The meaning of X is to derive the SNR value.

Proposal 6: We propose the value of X is about 0.5-0.8dB.

Proposal 7: No need to capture X in the core and conformance specifications.

Number of tests to define

We prefer to define one requirement/test for the ultra-low BLER target. More tests will be defined with higher BLER target.

The experiment of testing the ultra-low BLER target tells us it takes very long time to achieve 10^-5, as the CQI reporting test is more complex than the single BLER test, we propose do not test the CQI table with 10^-5 BLER target.

Proposal 8: We propose to define one requirement/test for ultra-low BLER target.

Proposal 9: No need to define CQI tests with ultra-low BLER target.

Proposals

In this paper, following are proposed:

Proposal 1: No need to define the requirements and test for FR2.

Proposal 2: We propose DM-RS configuration of 1+1 for UE.

Proposal 3: TDD patterns 3D1S1U (S=10:2:2) for 15 KHz, 7D1S2U (S=6:4:4) for 30 KHz.

Proposal 4: DM-RS configuration of 1+1 for BS.

Proposal 5: Bandwidth of 10MHz for 15 KHz and 40 MHz for 30 KHz with full bandwidth frequency allocation.

Proposal 6: We propose the value of X is about 0.5-0.8dB.

Proposal 7: No need to capture X in the core and conformance specifications.

Proposal 8: We propose to define one requirement/test for ultra-low BLER target.

Proposal 9: No need to define CQI tests with ultra-low BLER target.

Reference

[1] R4-2002422, Way forward on parameters and test methodology for URLLC ultra-low BLER test, #94, Ericsson.