3GPP TSG-RAN WG4 Meeting #94bis-e										  	     		    R4-2003777

Electronic meeting, 20 – 30 April, 2020

Source: 	Nokia, Nokia Shanghai Bell

Title: 					IAB-MT receiver sensitivity declaration range in FR2

Agenda item:			6.5.2.2.2

Document for:			Approval

# 1	Introduction

Part of the Integrated Access and Backhaul work item is defining the RF requirements. In this contribution we discuss the IAB-MT receiver sensitivity requirements.

# 2	Discussion

In RAN4#94-e a WF on IAB-MT class definition was agreed in [1]. For FR2 IAB-MT it was agreed that EISREFSENS_50M within the OTA REFSENS RoAoA shall be declared by the manufacturer, the declaration shall be independent for IAB-MT and IAB-DU, and FRC for IAB-MT EISREFSENS is FFS. It was also agreed that if multiple IAB-MT power classes are agreed, multiple declaration ranges may be defined. In WF on IAB-MT class definition in [2], it was agreed that two IAB-MT classes shall be defined in rel-16.

The differences of the two classes are discussed more in detail in [3], where it is proposed that Wide Area IAB-MT shall be based on Macro/Micro deployment Scenario whereas the other class, called Local Area IAB-MT, shall be based on Pico deployment scenarios. When compared to BS classes, Macro and Micro are associated with Wide Area and Medium range BS, and Pico with Local Area BS. The current BS receiver sensitivity requirements in FR2 have a dependency on BS classes, i.e. deployment scenarios. Below is an extract from TS 38.104

EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50 MHz BS channel bandwidth. EISREFSENS_50M itself is not a requirement and although it is based on a reference measurement channel with 50 MHz BS channel bandwidth it does not imply that BS has to support 50 MHz BS channel bandwidth.

For Wide Area BS, EISREFSENS_50M is an integer value in the range -96 to -119 dBm. The specific value is declared by the vendor.

For Medium Range BS, EISREFSENS_50M is an integer value in the range -91 to -114 dBm. The specific value is declared by the vendor.

For Local Area BS, EISREFSENS_50M is an integer value in the range -86 to -109 dBm. The specific value is declared by the vendor.

It would be reasonable to merge the allowed declaration range of Wide Area BS and Medium range BS for Wide Area IAB-MT in FR2.

Proposal 1: Wide Area IAB-MT in FR2 shall be able to declare EISREFSENS_50M as an integer value in the range from -91 to -119 dBm

For the Local Area IAB-MT class the declaration is not as straight-forward. The receiver sensitivity is indirectly tied together with the output power via the antenna array size, and in case no output power restrictions are specified for Local Area IAB-MT, there would be no reason to restrict the sensitivity declaration range either. This is motivated by the fact that in case Local Area IAB-MT is able to reach as high output power as Wide Area IAB-MT, the antenna array it uses can also be up to the similar size as for Wide Area IAB-MT, resulting in similar sensitivity, when other factors are assumed equal.

Proposal 2: In case Local Area IAB-MT maximum output power is not restricted, it shall be able to declare as low sensitivity as Wide Area IAB-MT, i.e. the declaration range would be from -86 to -119 dBm.

# 3	Conclusion

In this contribution IAB-MT sensitivity declaration range in FR2 was discussed and the following proposals were made:

Proposal 1: Wide Area IAB-MT in FR2 shall be able to declare EISREFSENS_50M as an integer value in the range from -91 to -119 dBm

Proposal 2: In case Local Area IAB-MT maximum output power is not restricted, it shall be able to declare as low sensitivity as Wide Area IAB-MT, i.e. the declaration range would be from -86 to -119 dBm.

# 4	References

[1]	R4-2002483 WF on IAB RX RF requirement, Samsung

[2]	R4-2002494 WF on IAB-MT class definition, Samsung

[3]		R4-2003774, IAB-Node classification and related output power requirements, Nokia, Nokia Shanghai Bell