3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2004014

Electronic meeting, 20 – 30 April 2020

Agenda Item:	5.10.3.1

Source:	Ericsson

Title:	Discussion on the remaining issues on DL quality report for eMTC

Document for:	Discussion

# 1	Introduction

RAN4#94-e agreed with the way forward on the DL channel quality reporting [1]. This contribution discusses the remaining open issues for DL channel quality report for eMTC.

- Separate accuracy requirements for fixed AL24 and fixed RL=1 - Set -15 ≤ Es/Iot ≤ [-3] dB as the side condition to report the repetition level with the fixed AL=24. - Set Es/Iot ≥ [-3] dB as the side condition to report the aggregation level with the fixed RL=1. - Use static channel for test case as same as Rel-14 NB-IoT channel quality reporting. - Following 8-bit reporting table is agreed: Reported level; MPDCCH repetition level; MPDCCH aggregation level; CE mode. 0; No measurement reporting; No measurement reporting; A, B. 1; 1; 1; A. 2; 1; 2; A. 3; 1; 4; A. 4; 1; 8; A. 5; 1; 16; A. 6; 1; 24; A, B. 7; 2; 24; A, B. 8; 4; 24; A, B. 9; 8; 24; A, B. 10; 16; 24; A, B. 11; 32; 24; A, B. 12; 64; 24; A, B. 13; 128; 24; A, B. 14; 256; 24; A, B. - RAN4 waits for RAN2 conclusion on 2-bit reporting table.

# 2	Discussion

## 2.1	2-bit reporting

The remaining open issue of eMTC channel quality report is the 2-bit reporting table. According to RAN2#109-e agreements, it was confirmed that 2-bit reporting in MSG3 is supported.

Agreements

- RAN2 confirms that 2-bit CQI report in MSG3 is supported.

- Quality Report trigger in Connected Mode for eMTC is the same MAC CE as agreed for NB-IoT.

- When 2-bit CQI reporting enabled in MSG3 then 8-bit reporting shall also be enabled.

- Separate flags are not required to enable/disable CQI reporting per CE level.

Although RAN2 has discussed for several meetings whether to introduce 2-bit channel quality reporting, RAN2#107bis had already agreed 2-bit reporting uses R+F2+E MAC subheader.

Agreements

- For 8-bit DL quality report, same MAC CE is used for reporting in Msg3 and connected mode.

- For 8-bit DL quality report; LCID value for the quality report is transmitted in addition to the LCID value for UL-CCCH.

- Codepoint/index of “10001” is used for 8-bit DL quality report for eMTC.

- For EDT, 8-bit DL quality report has lower priority than MO data from UL-CCCH.

- For non-EDT, 8-bit DL quality report has higher priority than MO data not from UL-CCCH, i.e., one level above “data from any Logical Channel, except data from UL-CCCH”

- For non-EDT, R+F2+E MAC subheader is used for 2-bit DL quality report.

- 2 separate indicators are introduced in SIB to enable 8-bit and 2-bit DL quality report, i.e., FFS if it is possible to indicate 2-bit only.

Based on the confirmation, RAN4 need to define the mapping table as same as 8-bit reporting table agreed in RAN4#94-e. In our understanding, the MAC subheader with R+F2+E consists of 3 bits. Depending on the RAN2 agreements, UE can report 4 values if RAN2 decides R bit is used to indicate the channel quality, F2 and E bits are used for the channel quality values. This means UE sets R=0 in the case of no measurement. If UE reports the channel quality, UE sets R=1 and then it can use 2 bits, corresponding to 4 values. On the other hand, RAN2 may decide to assign 3 bits for the channel quality report, and in this case, UE can report 8 values, although one value should be used for ‘no measurement’.

Proposal 1: RAN4 wait for RAN2 decision for the number of reporting values for 2-bit channel quality report table.

## 2.2	Option 1: 4 values

In Rel-14 NB-IoT 2-bits channel quality report, UE can reports only 3 values {Rmax/8, Rmax, 4xRmax} because one value is used for ‘no measurement’. On the other hand, since eMTC UE can report 4 values for 2-bits reporting, we propose to report the channel quality value based on Rmax/8, Rmax/4, Rmax, and 4*Rmax.

Table 1 shows the reported value for Rmax based on our proposal. It is observed this table reports two or more repetition level 1s for Rmax <= 4, and it is not efficient. We therefore propose to modify the mapping table as shown in Table 2, where UE reports {1,2,4,8} if Rmax=1 or 2, and UE reports {1,2,4,16} if Rmax=4.

Table 1	2-bit channel quality mapping table.

|  | Rmax | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value 1 | Rmax / 8 | 1 | 1 | 1 | 1 | 2 | 4 | 8 | 16 | 32 |
| Value 2 | Rmax / 4 | 1 | 1 | 1 | 2 | 4 | 8 | 16 | 32 | 64 |
| Value 3 | Rmax | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 |
| Value 4 | 4 x Rmax | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 256 | 256 |

Table 2	Proposed 2-bit channel quality mapping table.

|  | Rmax | 1 or 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value 1 | Rmax / 8 | 1 | 1 | 1 | 2 | 4 | 8 | 16 | 32 |
| Value 2 | Rmax / 4 | 2 | 2 | 2 | 4 | 8 | 16 | 32 | 64 |
| Value 3 | Rmax | 4 | 4 | 8 | 16 | 32 | 64 | 128 | 256 |
| Value 4 | 4 x Rmax | 8 | 16 | 32 | 64 | 128 | 256 | 256 | 256 |

Proposal 2: RAN4 use the channel quality mapping table shown in Table 2 if RAN2 decide to assign 2 bits for the channel quality report.

## 2.3	Option 2: 8 values

If UE can report 8 values, we propose to define the table depending on Rmax as shown in Table 3. We derive this table so that UE can report the range from Rmax/8 to 4 x Rmax, which is same assumption as Rel-14 NB-IoT 2-bit table.

Table 3	Proposed 3-bit channel quality mapping table.

| Configured Rmax | Rmax <= 16 | Rmax >= 32 |
| --- | --- | --- |
| Value 1 | No measurement reporting | No measurement reporting |
| Value 2 | 1 | 4 |
| Value 3 | 2 | 8 |
| Value 4 | 4 | 16 |
| Value 5 | 8 | 32 |
| Value 6 | 16 | 64 |
| Value 7 | 32 | 128 |
| Value 8 | 64 | 256 |

Proposal 3: RAN4 use the channel quality mapping table as shown in Table 3 if RAN2 decide to assign 3 bits for the channel quality report.

# 3	Summary

Proposal 1: RAN4 wait for RAN2 decision for the number of reporting values for 2-bit channel quality report table.

Proposal 2: RAN4 use the channel quality mapping table shown in Table 2 if RAN2 decide to assign 2 bits for the channel quality report.

|  | Rmax | 1 or 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value 1 | Rmax / 8 | 1 | 1 | 1 | 2 | 4 | 8 | 16 | 32 |
| Value 2 | Rmax / 4 | 2 | 2 | 2 | 4 | 8 | 16 | 32 | 64 |
| Value 3 | Rmax | 4 | 4 | 8 | 16 | 32 | 64 | 128 | 256 |
| Value 4 | 4 x Rmax | 8 | 16 | 32 | 64 | 128 | 256 | 256 | 256 |

Proposal 3: RAN4 use the channel quality mapping table as shown in Table 3 if RAN2 decide to assign 3 bits for the channel quality report.

| Configured Rmax | Rmax <= 16 | Rmax >= 32 |
| --- | --- | --- |
| Value 1 | No measurement reporting | No measurement reporting |
| Value 2 | 1 | 4 |
| Value 3 | 2 | 8 |
| Value 4 | 4 | 16 |
| Value 5 | 8 | 32 |
| Value 6 | 16 | 64 |
| Value 7 | 32 | 128 |
| Value 8 | 64 | 256 |

# 4	References

- R4-2002268, “WF on Rel-16 MTC RRM Requirements”, Ericsson, Huawei, HiSilicon.