3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2004020

Electronic Meeting, 20 – 30 April 2020

Agenda Item:	5.10.4

Source:	Ericsson

Title:	Open issues on UE/BS demodulation requirements for Rel-16 eMTC

Document for:	Discussion

# 1	Introduction

RAN4#94-e agreed with the way forward on UE/BS demodulation performance for additional MTC enhancements, where several FFS(s) are listed [1]. This contribution discusses the open issues on UE/BS demodulation requirements.

- FFS to define new demodulation requirements of PUSCH with multi-TB scheduling. - FFS to define new demodulation requirements of PDSCH with multi-TB scheduling.

This contribution addresses the remaining open issues.

# 2	Multi-TB scheduling for PDSCH/PUSCH

According to the RAN1 agreements, PDSCH/PUSCH in Rel-16 supports up to 8 simultaneous transport block transmissions for CE Mode A and 4 simultaneous transport block transmissions for CE Mode B.

Figure 1 and Figure 2 illustrate the examples of multi-TB scheduling for DL and UL, respectively. These examples assume 4 simultaneous transport block transmissions. Although one DCI in MPDCCH can schedule multiple TBs, each TB is transmitted with the same resource allocation, MCS and repetition level. Moreover the same redundancy version sequence is used for each TB.

When the network configures the multi-TB scheduling with repetition, there are two configurations as shown in Figure 1 and Figure 2; one is the interleaved transmission where the transmitted transport block changes during the middle of repetition; another one is the non-interleaved transmission where the transmission is based on transport block.

Figure 1	Example of the downlink multi-TB transmission.

Figure 2	Example of the uplink multi-TB transmission.

RAN4#94-e discussed whether RAN4 defines PDSCH/PUSCH demodulation requirements with multi-TB transmission, and some company proposed to define the new demodulation requirements because it expects performance gain due to the time domain diversity in the case of interleaved transmission, although other companies do not propose to define the new requirements because no difference from the receiver algorithm point of view.

We performed PDSCH demodulation simulation assuming interleaved multi-TB transmission. Figure 3 (a) and (b) show our simulation result of PDSCH with interleaved multi-TB transmission assumed CE Mode A and CE Mode B, respectively. For the investigation, we have reused the existing eMTC PDSCH test parameters specified in TS36.101, such as MCS, repetition level or channel model. In these figures, 1TB corresponds to the single TB transmission and it is same transmission mode as Rel-13. It is observed from our simulation result that the difference between single TB transmission and interleaved multi-TB transmission (2TB or 4TB) is marginal with the existing RAN4 eMTC demodulation requirement parameters.

Observation: No performance difference between the single TB transmission and interleaved multi-TB transmission (2TB or 4TB) with the existing RAN4 eMTC demodulation requirement parameters.

Based on the observation, we propose not to define new PDSCH/PUSCH demodulation requirements with multi-TB scheduling.

Proposal 1: RAN4 does not define new PDSCH demodulation requirements with multi-TB scheduling.

Proposal 2: RAN4 does not define new PUSCH demodulation requirements with multi-TB scheduling.

| (a) CE Mode A | (b) CE Mode B |
| --- | --- |

Figure 3	Simulation results of PDSCH with interleaved multi-TB transmission.

# 3	Summary

Observation: No performance difference between the single TB transmission and interleaved multi-TB transmission (2TB or 4TB) with the existing RAN4 eMTC demodulation requirement parameters.

Proposal 1: RAN4 does not define new PDSCH demodulation requirements with multi-TB scheduling.

Proposal 2: RAN4 does not define new PUSCH demodulation requirements with multi-TB scheduling.

# 4	References

- R4-2002426, “Way forward on UE/BS demodulation performance for additional MTC enhancements for LTE”, Ericsson.