3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2004030

Electronic Meeting, 20 – 30 April 2020

Agenda Item:	6.17.2.1.3

Source:	Ericsson

Title:	Discussion on PDSCH demodulation performance with HST single tap

Document for:	Discussion

# 1	Introduction

One of the remaining issues in HST WI is NW assistance signaling for HST single tap. RAN4#94e discussed the several options as captured in the way forward [1]. We discuss the remaining issue for the UE demodulation requirements with HST single tap.

HST single tap​ - HST single tap requirements definition - Option A: Define requirements based on worst case, and whether to use single-shot or multi-shot depends on UE implement and should not be limited - Option B: Do not mandate the specific TRS processing for requirement definition and left it up to company decision. Provide HST RRM signaling during the demodulation test to inform UE about HST conditions. - No new network assistance signaling in addition to those which are already introduced is needed.

# 2	Discussion

RAN4#94-e agreed to define PDSCH HST single tap demodulation requirements based on the worst case and it is up to UE implementation what TRS processing method is used, e.g., single-shot or multi-shot. One remaining open issue is whether the TE provides the HST RRM signaling during the demodulation test.

We understand this signaling may help some UE to optimize their receiver algorithm, however we concern to assume the RRM signaling during the demodulation test. We should first point out the intention of this cell-based configuration flag is for RRM requirements to support up to 500km/h as TS38.331 specifies. According to TS38.133, this configuration flag enables shorter measurement period/enhancements in DRX, but it is up to network implementation, i.e. the network may not configure this flag when it does not use DRX or does not need to set the high speed RRM flag (e.g. when it just handles Rel-15 UEs and Rel-16 capable UEs in the same way). This means UE should not assume all the BSs will signal the high speed measurement flag even if it is deployed along the high speed train railway track.

HighSpeedConfigforNR information element

-- ASN1START

-- TAG-HIGHSPEEDCONFIGFORNE-START

HighSpeedConfigforNR-r16 ::=	SEQUENCE {

highSpeedEnhMeasFlagforNR-r16	ENUMERATED {true}	OPTIONAL, -- Need R

highSpeedEnhDemodFlagforNR-r16	ENUMERATED {true}	OPTIONAL, -- Need R

...

}

-- TAG-HIGHSPEEDCONFIGFORNE-STOP

-- ASN1STOP

HighSpeedConfigforNR field descriptions

highSpeedEnhMeasFlagforNR-r16 If the field is present, the UE shall apply the enhanced RRM requirements to support high speed up to 500 km/h as specified in TS 38.133 [14].

highSpeedEnhDemodFlagforNR-r16 If the field is present, the UE shall apply the enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h as specified in TS 38.101-4.

Another concern is that this flag is introduced from Rel-16. RAN4#94-e also discussed the release independent requirements for Rel-16 HST requirements [1] as follows:

Release independent - Whether Rel.16 HST requirements can be release independent from Rel-15 - Option 1: release independent for HST-SFN, HST single tap and multi-path fading - Option 2: not feasible for release independent for HST-SFN since signaling is introduced in Rel-16. - Option 3: release independent for multi-path fading - Check whether release independent of Single-tap and HST-SFN are feasible considering signaling introduced in Rel.-16.

If Rel-16 HST single tap test assumes the enhanced RRM flag highSpeedEnhMeasFlagforNR-r16, the requirement is NOT release independent from Rel-15 because Rel-15 UE does not understand the enhanced RRM flag [2]. Since we believe Rel-15 UE should also pass the single tap HST requirements discussed in Rel-16 HST WI, we should not assume the Rel-16 enhanced RRM signaling during the HST single tap test.

# 3	Summary

Proposal: Do not assume highSpeedEnhMeasFlagforNR-r16 during the HST single tap test.

# References

- R4-2002418, “WF on UE demodulation for NR HST”, CMCC.

- R4-2004031, “Discussion on release independent requirements for NR HST”, Ericsson.