3GPP TSG-RAN WG4 Meeting #94bis-e                                              R4-2004041

Electronic Meeting, April 20th – 30th, 2020

Source: 	Ericsson

Title:	TP to the TR 37.941: Conformance Framework

Agenda Item:	6.19.4

Document for:	Agreement

# Introduction

In this contribution we provide an update to the TP to the TR 37.941: Conformance Framework, to ensure that the external TR is aligned with other 3GPP specification guiding principles.  As step 8 from the OTA test frame work details a requirement for “establish[ing] budget format examples for each addressed test method in the form of lists of uncertainty contributions”.  Although during the work for NR experienced a compressed timeline and some requirements may have missed this framework, it is prudent to put in place this budget framework where possible for the OTA TR where possible.  For completeness, all requirements should have the budget format and if feasible this should be the ambition.

# References

[1]		RP-193225		Over the air (OTA) base station (BS) testing TR, WID

[2]		R4-2001807		Skeleton for TR 37.941 on OTA BS testing, Rel-15

# TP to the External TR on OTA BS testing

##### [Start of changes]

# 5	Conformance testing aspects

## 5.1	Conformance testing framework

OTA conformance testing framework for the directional and TRP test requirements was compiled based on the conducted conformance testing framework. The OTA conformance testing framework for AAS BS and NR BS considers additional aspects of the TRP measurements, e.g. sampling grid, additional MU contributors.

The OTA conformance testing framework for the TX directional and TRP test requirements is defined as follows:

For TX directional requirements, testing using the narrowest declared beamwidth in a single direction is sufficient for demonstrating conformance, except for the case of EVM in which case testing should be performed in all conformance directions of the OTA coverage range.

For test requirements, it is expected that one test method will be sufficient for each core requirement. Hence the traditional approach for devising test specifications can be followed. This is depicted in figure 5.1-1.

Figure 5.1-1: Core requirement to test requirement mapping

The following 11 points have been agreed as a framework for developing OTA test.

1)	Multiple test methods may exist for each requirement

2)	Each test method will require its own test procedure.

3)	A single conformance requirement applies for each core requirement, regardless of test procedure.

4)	Common maximum accepted test system uncertainty applies for all test methods addressing the same test requirement. Test methods producing significantly worse uncertainty than others at comparable cost should not impact the common maximum accepted test system uncertainty assessment.

5)	Common test tolerances apply for all test methods addressing the same test requirement.

6)	A common way of establishing the uncertainty result from all test methods' individual budgets is established.

7)	A common method of making an uncertainty budget (not a common uncertainty budget) is established.

8)	Establish budget format examples for each addressed test method in the form of lists of uncertainty contributions. Contributions that may be negligible with some EUT and substantial with others should be in this list. For each combination of measurement method and test parameter, develop a list with measurement uncertainties.

9)	Describe potential OTA test methods relevant for testing radiated transmit power and OTA sensitivity. The description requires information about the test range architecture and test procedure. Addressing each item in each uncertainty budget with respect to the expected distribution of the errors, the mechanism creating the error and how it interacts with properties of the EUT.

10)	Providing example uncertainty budgets in the TS will be useful in order to demonstrate the way a budget should be defined and how calculating its resulting measurement uncertainty is done, but the figures used in the examples will clearly be only examples and not applicable in general.

11)	Each test instance may require an individual uncertainty budget applicable for the combination of the test facility, the EUT and the test procedure and property tested. Here, the tester demonstrates that the uncertainty requirement is fulfilled during the conformance testing.

The linking of core requirements via test methods to conformance requirements is depicted in figure 5.1-2 based on the examples of EIRP and EIS requirements.

Figure 5.1-2: Examples of OTA core requirement to test requirement mapping

## 5.2	Uncertainty budget calculation principles

Uncertainty contributions listed in subclause 9, 10, 11, 12, 13, and 14 need to be calculated to provide an overall total measurement uncertainty for each OTA test method for the accompanying conformance requirement.

The uncertainty tables are presented with two stages in mind:

-	Stage 1: the calibration of the absolute level of the EUT measurement results is performed by means of using a calibration antenna (for example a standard gain horn) whose absolute gain is known at the frequencies of measurement,

-	Stage 2: the actual measurement with the EUT as either the transmitter or receiver (depending on the considered requirement) is performed.

The final uncertainty budget should comprise of a minimum 5 headings:

1)	The uncertainty source,

2)	Uncertainty value,

3)	Distribution of the probability,

4)	Divisor based on distribution shape,

5)	Sensitivity coefficient ci and its calculated standard uncertainty ui (based on uncertainty value, divisor and sensitivity coefficient ci).

NOTE:	All measurement uncertainty contributions are assumed independent, log and small, hence the errors induced by taking values in log are negligible.

The calculation of the uncertainty contribution is based on the ISO Guide [3] to the expression of uncertainty in measurement. Each individual uncertainty is expressed by its standard deviation (termed as ‘standard uncertainty’) and represented by symbol ui.

The procedure for forming the uncertainty budget can be as follows:

1)	Compile lists of individual uncertainty contributions for the requirement specific measurement, both in Stage 1 and Stage 2.

2)	Determine the standard uncertainty of each contribution ui by:

a)	Determining the distribution of the uncertainty (i.e. Gaussian, U-shaped, rectangular, or exponentially normal).

b)	Determining the maximum value of each uncertainty (unless the distributions is Gaussian).

c)	Calculating the standard uncertainty by dividing the uncertainty by  if the distribution is U-shaped, by 2 if the distribution exponentially normal and by  if the distribution is rectangular.

d)	Multiplying the standard uncertainty by the sensitivity coefficient ci.

3)	Convert the units into decibel, if necessary.

4)	Combine all the standard uncertainties by the Root of the Sum of the Squares (RSS) method.

5)	Combine the uncertainties in Stage 1 and Stage 2 into the combined uncertainty uc also by the RSS method:

.

6)	Multiply the result of the combined uncertainty uc by an expansion factor of 1.96 to derive expanded uncertainty ue at 95 % confidence level:

ue = 1.96 uc

##### [Unchanged Sections]

#### 9.2.2.3	Uncertainty budget format

Table 9.2.2.3-1: Indoor Anechoic Chamber uncertainty contributions
for AAS BS EIRP measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | A1-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna. | A1-2 |
| 3 | Quality of quiet zone | A1-3 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | A1-4a |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | A1-5 |
| 6 | Phase curvature | A1-6 |
| 7 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | C1-1 |
| 8 | Impedance mismatch in the receiving chain | A1-7 |
| 9 | Random uncertainty | A1-8 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | A1-9 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | A1-10 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer | A1-11 |
| 13 | Quality of quiet zone | A1-3 |
| 14 | Polarization mismatch for reference antenna | A1-4a |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | A1-5 |
| 16 | Phase curvature | A1-6 |
| 17 | Uncertainty of the Network Analyzer | C1-3 |
| 18 | Influence of the reference antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | A1-14 |
| 19 | Reference antenna feed cable loss measurement uncertainty | A1-13 |
| 20 | Influence of the receiving antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | A1-14 |
| 21 | Uncertainty of the absolute gain of the reference antenna | C1-4 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | A1-15 |

#### 9.2.2.4	MU value derivation, FR1

Table 9.2.2.4-1 captures derivation of the expanded measurement uncertainty values for EIRP accuracy measurements in Indoor Anechoic Chamber (Normal test conditions, FR1).

Table 9.2.2.4-1: Indoor Anechoic Chamber measurement uncertainty value derivation for EIRP accuracy measurements, Normal test conditions, FR1

| Indoor anechoic |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |  |
|  |  | f<3 GHz | 3<f<4.2 GHz | 4.2<f<6 GHz |  |  |  | f<3 GHz | 3<f<4.2 GHz | 4.2<f<6 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | 0.3 | Rectangular | 1.73 | 1 | 0.17 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.1 | 0.1 | 0.1 | Gaussian | 1.00 | 1 | 0.10 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0 | 0 | 0 | Rectangular | 1.73 | 1 | 0.00 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | 0.05 | Gaussian | 1.00 | 1 | 0.05 | 0.05 | 0.05 |
| 7 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | 0.26 | Gaussian | 1.00 | 1 | 0.14 | 0.26 | 0.26 |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | 0.33 | U-shaped | 1.41 | 1 | 0.10 | 0.23 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | 0.1 | Rectangular | 1.73 | 1 | 0.06 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.04 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.04 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.1 | 0.1 | 0.1 | Gaussian | 1.00 | 1 | 0.10 | 0.10 | 0.10 |
| 14 | Polarization mismatch between the reference antenna and the receiving antenna | 0.01 | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0 | 0 | 0 | Rectangular | 1.73 | 1 | 0.00 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | 0.05 | Gaussian | 1.00 | 1 | 0.05 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.2 | 0.2 | Gaussian | 1.00 | 1 | 0.13 | 0.20 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | 0.06 | Gaussian | 1.00 | 1 | 0.06 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.50 | 0.43 | 0.43 | Rectangular | 1.73 | 1 | 0.29 | 0.25 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0 | 0 | 0 | Rectangular | 1.73 | 1 | 0.00 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 0.44 | 0.54 | 0.54 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 0.87 | 1.06 | 1.06 |

##### [Unchanged Sections]

#### 9.2.3.3	Uncertainty budget format

# Table 9.2.3.3-1: CATR uncertainty budget format for EIRP measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | A2-1a |
| 2 | RF power measurement equipment | C1-1 |
| 3 | Standing wave between DUT and test range antenna | A2-2a |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | A2-3 |
| 5 | QZ ripple DUT | A2-4a |
| 19 | Miscellaneous uncertainty | B2-14 |
| 19 | Frequency flatness | A2-12 |
| Stage 1: Calibration measurement |  |  |
| 6 | Uncertainty of network analyser | C1-3 |
| 7 | Mismatch of receiver chain | A2-6 |
| 8 | Insertion loss variation of receiver chain | A2-3 |
| 9 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | A2-3 |
| 10 | Influence of the calibration antenna feed cable: a) Flexing cables, adapters, attenuators, connector repeatability | C1-4 |
| 11 | Uncertainty of the absolute gain of the calibration antenna | C1-4 |
| 12 | Misalignment positioning system | A2-1b |
| 13 | Misalignment of calibration antenna and test range antenna | A2-9 |
| 14 | Rotary Joints | A2-2b |
| 15 | Standing wave between reference calibration antenna and test range antenna | A2-4b |
| 16 | Quality of quiet zone | A2-11 |
| 20 | Switching uncertainty | A2-13 |

#### 9.2.3.4	MU value derivation, FR1

Table 9.2.3.3-1 captures derivation of the expanded measurement uncertainty values for EIRP accuracy measurements in CATR (Normal test conditions, FR1).

Table 9.2.3.3-1: CATR MU value derivation for EIRP accuracy measurements, Normal test conditions, FR1

| CATR |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |  |
|  |  | f<3 GHz | 3<f<4.2 GHz | 4.2<f<6 GHz |  |  |  | f<3 GHz | 3<f<4.2 GHz | 4.2<f<6 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error for EIRP | 0.00 | 0.00 | 0.00 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | 0.26 | Gaussian | 1.00 | 1 | 0.14 | 0.26 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | 0.21 | U-shaped | 1.41 | 1 | 0.15 | 0.15 | 0.15 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | 0.00 | 0.00 | Normal | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| 5 | QZ ripple DUT | 0.09 | 0.09 | 0.09 | Normal | 1.00 | 1 | 0.09 | 0.09 | 0.09 |
| 19 | Frequency flatness | 0.25 | 0.25 | 0.25 | Normal | 1.00 | 1 | 0.25 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| 6 | Uncertainty of the network analyzer | 0.13 | 0.20 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 | 0.00 |
| 7 | Mismatch of receiver chain | 0.13 | 0.33 | 0.33 | U-shaped | 1.41 | 1 | 0.09 | 0.23 | 0.23 |
| 8 | Insertion loss variation of receiver chain | 0.18 | 0.18 | 0.18 | Rectangular | 1.73 | 1 | 0.10 | 0.10 | 0.10 |
| 9 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | 0.00 | 0.00 | Normal | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| 10 | Influence of the calibration antenna feed cable: | 0.02 | 0.02 | 0.02 | U-shaped | 1.41 | 1 | 0.02 | 0.02 | 0.02 |
| 11 | Uncertainty of the absolute gain of the reference antenna | 0.50 | 0.43 | 0.43 | Rectangular | 1.73 | 1 | 0.29 | 0.25 | 0.25 |
| 12 | Misalignment positioning system | 0.00 | 0.00 | 0.00 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| 13 | Misalignment of calibration antenna and test range antenna | 0.50 | 0.50 | 0.50 | Exp. normal | 2.00 | 1 | 0.25 | 0.25 | 0.25 |
| 14 | Rotary Joints | 0.05 | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.03 | 0.03 | 0.03 |
| 15 | Standing wave between calibration antenna and test range antenna | 0.09 | 0.09 | 0.09 | U-shaped | 1.41 | 1 | 0.06 | 0.06 | 0.06 |
| 16 | QZ ripple calibration antenna | 0.01 | 0.01 | 0.01 | Normal | 1.00 | 1 | 0.01 | 0.01 | 0.01 |
| 20 | Switching uncertainty | 0.26 | 0.26 | 0.26 | Rectangular | 1.73 | 1 | 0.15 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 0.57 | 0.65 | 0.61 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 1.11 | 1.27 | 1.20 |

##### [Unchanged Sections]

#### 9.2.4.3	Uncertainty budget format

Table 9.2.4.3-1: One Dimensional Compact Range Chamber uncertainty contributions for AAS BS EIRP measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT and pointing error | A4-1 |
| 2 | Standing wave between DUT and test range antenna | A4-2a |
| 3 | Quiet zone ripple DUT | A4-3a |
| 4 | Phase curvature | A4-4a |
| 5 | Polarization mismatch between DUT and receiving antenna | A4-5a |
| 6 | Mutual coupling between DUT and receiving antenna | A4-6a |
| 7 | Measurement equipment | C1-1 |
| 8 | Impedance mismatch in receiving chain | A4-7 |
| 9 | RF leakage (DUT connector terminated and test range antenna connector cable terminated) | A4-8a |
| Stage 1-: Calibration measurement |  |  |
| 10 | Misalignment positioning system | A4-9 |
| 11 | Pointing error between reference antenna and test range antenna | A4-10 |
| 12 | Impedance mismatch in path to calibration antenna | A4-11 |
| 13 | Impedance mismatch in path to compact probe | A4-12 |
| 14 | Standing wave between reference antenna and test range antenna | A4-2b |
| 15 | Quiet zone ripple reference antenna | A4-3b |
| 16 | Phase curvature | A4-4b |
| 17 | Polarization mismatch between reference antenna and receiving antenna | A4-5b |
| 18 | Mutual coupling between reference antenna and receiving antenna | A4-6a |
| 19 | Measurement equipment | C1-1 |
| 20 | Influence of reference antenna feed cable (flexing cables, adapters, attenuators, connector repeatability) | A4-13 |
| 21 | Mismatch of receiver chain | A4-14 |
| 22 | Insertion loss of receiver chain | A4-15 |
| 23 | Uncertainty of absolute gain of reference antenna | C1-4 |
| 24 | RF leakage (SGH connector terminated and test range antenna connector cable terminated) | A4-8b |

#### 9.2.4.4	MU value derivation, FR1

Table 9.2.4.3-1 captures derivation of the expanded measurement uncertainty values for EIRP accuracy measurements in One Dimensional Compact Range.

Table 9.2.4.3-1: One Dimensional Compact Range MU value derivation for EIRP accuracy measurements, FR1

| One Dimensional Compact Range Chamber |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |  |
|  |  | f<3 GHz | 3<f<4.2 GHz | 4.2<f<6 GHz |  |  |  | f<3 GHz | 3<f<4.2 GHz | 4.2<f<6 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT and pointing error | 0.00 | 0.00 | 0.00 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| 2 | Standing wave between DUT and test range antenna | 0.18 | 0.18 | 0.18 | U-shaped | 1.41 | 1 | 0.13 | 0.13 | 0.13 |
| 3 | Quiet zone ripple DUT | 0.03 | 0.03 | 0.03 | Gaussian | 1.00 | 1 | 0.03 | 0.03 | 0.03 |
| 4 | Phase curvature AAS | 0.01 | 0.01 | 0.01 | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.01 |
| 5 | Polarization mismatch between DUT and receiving antenna | 0.05 | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.03 | 0.03 |
| 6 | Mutual coupling between DUT and receiving antenna | 0.00 | 0.00 | 0.00 | Rectangular | 1.73 | 1 | 0.00 | 0.00 | 0.00 |
| 7 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | 0.26 | Gaussian | 1.00 | 1 | 0.14 | 0.26 | 0.26 |
| 8 | Impedance mismatch in receiving chain | 0.01 | 0.01 | 0.01 | U-shaped | 1.41 | 1 | 0.00 | 0.01 | 0.01 |
| 9 | RF leakage (DUT connector terminated and test range antenna connector cable terminated) | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| 10 | Misalignment positioning system | 0.00 | 0.00 | 0.00 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| 11 | Pointing error between reference antenna and test range antenna | 0.00 | 0.00 | 0.00 | Rectangular | 1.73 | 1 | 0.00 | 0.00 | 0.00 |
| 12 | Impedance mismatch in path to reference antenna | 0.05 | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.04 | 0.04 | 0.04 |
| 13 | Impedance mismatch in path to compact probe | 0.03 | 0.03 | 0.03 | U-shaped | 1.41 | 1 | 0.02 | 0.02 | 0.02 |
| 14 | Standing wave between reference antenna and receiving antenna | 0.09 | 0.09 | 0.09 | U-shaped | 1.41 | 1 | 0.06 | 0.06 | 0.06 |
| 15 | Quiet zone ripple reference antenna | 0.18 | 0.18 | 0.18 | Gaussian | 1.00 | 1 | 0.18 | 0.18 | 0.18 |
| 16 | Phase curvature refernce antenna | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| 17 | Polarization mismatch between reference antenna and receiving antenna | 0.05 | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.03 | 0.03 |
| 18 | Mutual coupling between reference antenna and receiving antenna | 0.00 | 0.00 | 0.00 | Rectangular | 1.73 | 1 | 0.00 | 0.00 | 0.00 |
| 19 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | 0.26 | Gaussian | 1.00 | 1 | 0.14 | 0.26 | 0.26 |
| 20 | Influence of the reference antenna feed cable (flexing cables, adapters, attenuators, connector repeatability) | 0.08 | 0.08 | 0.08 | Rectangular | 1.73 | 1 | 0.05 | 0.05 | 0.05 |
| 21 | Mismatch of receiver chain | 0.20 | 0.30 | 0.30 | U-shaped | 1.41 | 1 | 0.14 | 0.21 | 0.21 |
| 22 | Insertion loss of receiver chain | 0.18 | 0.18 | 0.18 | Rectangular | 1.73 | 1 | 0.10 | 0.10 | 0.10 |
| 23 | Uncertainty of the absolute gain of the reference antenna | 0.50 | 0.43 | 0.43 | Rectangular | 1.73 | 1 | 0.29 | 0.25 | 0.25 |
| 24 | RF leakage (SGH connector terminated and test range antenna connector cable terminated. | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 0.46 | 0.56 | 0.56 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 0.90 | 1.10 | 1.10 |

##### [Unchanged Sections]

#### 9.2.6.3	Uncertainty budget format

#### 9.2.6.4	MU value derivation, FR1

Table 9.2.6.3-1 captures derivation of the expanded measurement uncertainty values for EIRP accuracy measurements in PWS.

Table 9.2.6.3-1: PWS measurement uncertainty value derivation for EIRP accuracy measurements, FR1

| Plane wave synthesizer |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |  |
|  |  | f<3 GHz | 3<f<4.2 GHz | 4.2<f<6 GHz |  |  |  | f<3 GHz | 3<f<4.2 GHz | 4.2<f<6 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
| A7-1a | Misalignment DUT & pointing error | 0.10 | 0.10 | 0.10 | Rectangular | 1.73 | 1 | 0.06 | 0.06 | 0.06 |
| C1-1 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | 0.26 | Gaussian | 1.00 | 1 | 0.14 | 0.26 | 0.26 |
| A7-2a | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for DUT antenna | 0.05 | 0.14 | 0.14 | Rectangular | 1.73 | 1 | 0.03 | 0.08 | 0.08 |
| A7-3 | RF leakage (calibration antenna connector terminated) | 0.09 | 0.09 | 0.09 | Normal | 1.00 | 1 | 0.09 | 0.09 | 0.09 |
| A7-4a | QZ ripple with DUT | 0.42 | 0.43 | 0.43 | Rectangular | 1.73 | 1 | 0.24 | 0.25 | 0.25 |
| A7-5 | Miscellaneous Uncertainty | 0.00 | 0.00 | 0.00 | Normal | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| A7-14 | System non-linearity | 0.06 | 0.06 | 0.06 | Rectangular | 1.73 | 1 | 0.04 | 0.04 | 0.04 |
| A7-13 | Frequency Flatness | 0.13 | 0.13 | 0.13 | Rectangular | 1.73 | 1 | 0.08 | 0.08 | 0.08 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer | 0.13 | 0.20 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 | 0.20 |
| A7-6 | Mismatch (i.e. reference antenna, network analyzer and reference cable) | 0.13 | 0.33 | 0.33 | U-shaped | 1.41 | 1 | 0.09 | 0.23 | 0.23 |
| A7-7 | Insertion loss variation | 0.18 | 0.18 | 0.18 | Rectangular | 1.73 | 1 | 0.10 | 0.10 | 0.10 |
| A7-3 | RF leakage (calibration antenna connector terminated) | 0.09 | 0.09 | 0.09 | Normal | 1.00 | 1 | 0.09 | 0.09 | 0.09 |
| A7-8 | Influence of the calibration antenna feed cable | 0.10 | 0.10 | 0.10 | Rectangular | 1.73 | 1 | 0.06 | 0.06 | 0.06 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.50 | 0.43 | 0.43 | Rectangular | 1.73 | 1 | 0.29 | 0.25 | 0.25 |
| A7-9 | Misalignment of positioning system | 0.00 | 0.00 | 0.00 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| A7-1b | Misalignment of calibration antenna & pointing error | 0.05 | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.03 | 0.03 |
| A7-10 | Rotary joints | 0.00 | 0.00 | 0.00 | U-shaped | 1.73 | 1 | 0.00 | 0.00 | 0.00 |
| A7-2b | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for calibration antenna | 0.12 | 0.12 | 0.12 | Rectangular | 1.73 | 1 | 0.07 | 0.07 | 0.07 |
| A7-4a | QZ ripple with calibration antenna | 0.20 | 0.20 | 0.20 | Rectangular | 1.73 | 1 | 0.12 | 0.12 | 0.12 |
| A7-11 | Switching uncertainty | 0.02 | 0.02 | 0.02 | Rectangular | 1.73 | 1 | 0.01 | 0.01 | 0.01 |
| A7-12 | Field repeatability | 0.06 | 0.12 | 0.12 | Normal | 1.00 | 1 | 0.06 | 0.12 | 0.12 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 0.50 | 0.60 | 0.60 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 0.98 | 1.18 | 1.18 |

##### [Unchanged Sections]

#### 9.4.2.3	Uncertainty budget format

Table 9.4.2.3-1: Indoor Anechoic Chamber uncertainty contributions
for OTA E-UTRA DL RS power measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | E1-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna. | E1-2 |
| 3 | Quality of quiet zone | E1-3 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | E1-4 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | E1-5 |
| 6 | Phase curvature | E1-6 |
| 8 | Impedance mismatch in the receiving chain | E1-8 |
| 9 | Random uncertainty | E1-9 |
| 23 | MU of TE derived from conducted specification (conducted MU / 1.96) | F2 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | E1-10 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | E1-11 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer | E1-12 |
| 13 | Quality of quiet zone | E1-3 |
| 14 | Polarization mismatch for reference antenna | E1-4 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | E1-5 |
| 16 | Phase curvature | E1-6 |
| 17 | Uncertainty of the Network Analyzer | F1 |
| 18 | Influence of the reference antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | E1-14 |
| 19 | Reference antenna feed cable loss measurement uncertainty | E1-15 |
| 20 | Influence of the receiving antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | E1-16 |
| 21 | Uncertainty of the absolute gain of the reference antenna | F1 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | E1-18 |

#### 9.4.2.4	MU value derivation, FR1

As the DL RS power is an absolute measurement most of the uncertainties form the EIRP accuracy remain the same. Also it can be noted that the measured signal is a wanted signal and hence will be beam formed in the same way as the wanted signal, hence any errors which may be dependent on the beam shape will be the same as for the EIRP accuracy measurement.

Table 9.4.2.3-1: Indoor Anechoic Chamber measurement uncertainty value derivation for OTA E-UTRA DL RS power measurement

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

##### [Unchanged Sections]

#### 9.4.3.3	Uncertainty budget format

Table 10.2.3.3.4.1-1: CATR uncertainty budget format for OTA E-UTRA DL RS power measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | E2-1 |
| 2 | RF power measurement equipment | F1 |
| 3 | Standing wave between DUT and test range antenna | E2-3 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 5 | QZ ripple DUT | E2-5 |
| 19 | Miscellaneous uncertainty | E2-2 |
| 23 | MU of TE derived from conducted specification (conducted MU / 1.96) | F2 |
| Stage 1: Calibration measurement |  |  |
| 6 | Uncertainty of network analyser | F1 |
| 7 | Mismatch of receiver chain | E2-7 |
| 8 | Insertion loss variation of receiver chain | E2-8 |
| 9 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 10 | Influence of the calibration antenna feed cable: a) Flexing cables, adapters, attenuators, connector repeatability | E2-9 |
| 11 | Uncertainty of the absolute gain of the calibration antenna | F1 |
| 12 | Misalignment positioning system | E2-11 |
| 13 | Misalignment of calibration antenna and test range antenna | E2-1 |
| 14 | Rotary Joints | E2-12 |
| 15 | Standing wave between reference calibration antenna and test range antenna | E2-3 |
| 16 | Quality of quiet zone | E2-5 |
| 20 | Switching uncertainty | E2-15 |

#### 9.4.3.4	MU value derivation, FR1

As the DL RS power is an absolute measurement most of the uncertainties form the EIRP accuracy remain the same. Also it can be noted that the measured signal is a wanted signal and hence will be beam formed in the same way as the wanted signal, hence any errors which may be dependent on the beam shape will be the same as for the EIRP accuracy measurement.

Table 6.4.3.4-1: CATR MU value derivation for OTA E-UTRA DL RS power measurement

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

##### [Unchanged Sections]

#### 9.4.3.3	Uncertainty budget format

Table 10.2.3.4.4-1: Near field test range uncertainty contributors in AAS OTA E-UTRA DL RS power measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
|  | Stage 2:, EIRP near field radiation pattern measurement and EIRP near field DUT power measurement |  |
| 1 | Axes Intersection | E3-1 |
| 2 | Axes Orthogonality | E3-2 |
| 3 | Horizontal Pointing | E3-3 |
| 4 | Probe Vertical Position | E3-4 |
| 5 | Probe H/V pointing | E3-5 |
| 6 | Measurement Distance | E3-6 |
| 7 | Amplitude and Phase Drift | E3-7 |
| 8 | Amplitude and Phase Noise | E3-8 |
| 9 | Leakage and Crosstalk | E3-9 |
| 10 | Amplitude Non-Linearity | E3-10 |
| 11 | Amplitude and Phase Shift in rotary joints | E3-11 |
| 12 | Channel Balance Amplitude and Phase | E3-12 |
| 13 | Probe Polarization Amplitude and Phase | E3-13 |
| 14 | Probe Pattern Knowledge | E3-14 |
| 15 | Multiple Reflections | E3-15 |
| 16 | Room Scattering | E3-16 |
| 17 | DUT support Scattering | E3-17 |
| 18 | Scan Area Truncation | E3-18 |
| 19 | Sampling Point Offset | E3-19 |
| 20 | Spherical Mode Truncation | E3-20 |
| 21 | Positioning | E3-21 |
| 22 | Probe Array Uniformity | E3-22 |
| 23 | Mismatch of receiver chain (i.e. between receiving antenna and measurement receiver) | E3-23 |
| 24 | Insertion loss of receiver chain | E3-24 |
| 25 | Uncertainty of the absolute gain of the probe antenna | E3-25 |
| 36 | MU of TE derived from conducted specification | F2 |
| 27 | Measurement repeatability - Positioning Repeatability | E3-27 |
|  | Stage 1: Calibration measurement |  |
| 28 | Uncertainty of network analyser | E |
| 29 | Mismatch of receiver chain | B4-29 |
| 30 | Insertion loss of receiver chain | B4-30 |
| 31 | Mismatch in the connection of the calibration antenna | B4-31 |
| 32 | Influence of the calibration antenna feed cable | B4-32 |
| 33 | Influence of the probe antenna cable | B4-33 |
| 34 | Uncertainty of the absolute gain of the calibration antenna | E |
| 35 | Short term repeatability | B4-35 |

#### 9.4.4.3	MU value derivation, FR1

Table 9.4.4.3-1: NFTR measurement uncertainty value derivation for OTA E-UTRA DL RS power measurement

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

##### [Unchanged Sections]

#### 9.5.2.3	Uncertainty budget format

Table 9.5.2.3-1: Indoor Anechoic Chamber uuncertainty contributions
for AAS BS OTA E-UTRA Total power dynamic range measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna (note 1) | E1-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna. (note 1) | E1-2 |
| 3 | Quality of quiet zone (note 1) | E1-3 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna (note 1) | E1-4 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna (note 1) | E1-5 |
| 6 | Phase curvature (note 1) | E1-6 |
| 8 | Impedance mismatch in the receiving chain (note 1) | E1-8 |
| 9 | Random uncertainty (note 1) | E1-9 |
| 23 | Uncertainty of conducted measurement (conducted MU / 1.96) | F2 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer (note 1) | E1-10 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna (note 1) | E1-11 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer (note 1) | E1-12 |
| 13 | Quality of quiet zone (note 1) | E1-3 |
| 14 | Polarization mismatch for reference antenna (note 1) | E1-4 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | E1-5 |
| 16 | Phase curvature (note 1) | E1-6 |
| 17 | Uncertainty of the Network Analyzer (note 1) | F1 |
| 18 | Influence of the reference antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability (note 1) | E1-14 |
| 19 | Reference antenna feed cable loss measurement uncertainty (note 1) | E1-15 |
| 20 | Influence of the receiving antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability (note 1) | E1-16 |
| 21 | Uncertainty of the absolute gain of the reference antenna (note 1) | F1 |
| 22 | Uncertainty of the absolute gain of the receiving antenna (note 1) | E1-18 |
| Note 1: These uncertainties cancel out as the requirement is differential |  |  |

#### 9.5.2.4	MU value derivation, FR1

As the output power dynamics are relative measurements most of the uncertainties form the EIRP accuracy cancel out as the same error will be applied to both of the measured OTA signals.

This includes all calibration errors, misalignment errors, impedance mismatch and mutual coupling.

As the both the measured OTA signal will have the same beam pattern quiet zone errors, phase curvature errors also can be expected to be the same for both signals.

The uncertainty budget descriptions are the same as those in table 6.4.2.3-1 with the addition descriptions in table 6.5.2.3-1.

The MU uncertainty assessment is shown in table 6.5.2.4-1, zero values have been omitted in the table for the sake of space, but still be considered as part of the budget.

Table 9.5.2.3-1: Indoor Anechoic Chamber measurement uncertainty value derivation for OTA total power dynamic range measurement

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

The same uncertainty assessments have been carried out for the UTRA dynamic range requirements, i.e.

- Power control steps,

- Power control dynamic range,

- Total power dynamic range,

- IPDL Time mask.

In each case the uncertainty for the conducted measurement is the same as that for the conducted MU in TS 25.141 [2] as follows:

- Power control steps, Uncertainty of conducted measurement = 0.1 dB, Expanded OTA uncertainty = 0.15 dB.

- Power control dynamic range, Uncertainty of conducted measurement = 1.1 dB, Expanded OTA uncertainty = 1.11 dB.

- Total power dynamic range, Uncertainty of conducted measurement = 0.3 dB, Expanded OTA uncertainty = 0.32 dB.

- IPDL Time mask, Uncertainty of conducted measurement = 0.7dB, Expanded OTA uncertainty = 0.71 dB.

##### [Unchanged Sections]

#### 9.5.3.3	Uncertainty budget format

The uncertainty budget descriptions are the same as those in table 9.5.2.3-1 with the addition descriptions in table 9.5.3.3-1.

Table 9.5.3.3-1: Indoor Anechoic Chamber uncertainty contributions
for OTA Total power dynamic range measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 23 | Uncertainty of conducted measurement (conducted MU / 1.96) | F2 |
| Stage 1: Calibration measurement |  |  |

#### 9.5.3.4	MU value derivation, FR1

As the output power dynamics are relative measurements most of the uncertainties form the EIRP accuracy cancel out as the same error will be applied to both of the measured OTA signals.

This includes all calibration errors, misalignment errors, impedance mismatch and mutual coupling.

As the both the measured OTA signal will have the same beam pattern quiet zone errors, phase curvature errors also can be expected to be the same for both signals.

The uncertainty budget descriptions are the same as those in table 6.4.3.3-1 with the addition descriptions in table 6.5.3.3-1.

The MU uncertainty assessment is shown in table 6.5.3.4-1, zero values have been omitted in the table for the sake of space, but still be considered as part of the budget.

Table 9.5.3.3-1: CATR MU value derivation for OTA total power dynamic range measurement

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

The same uncertainty assessments have been carried out for the UTRA dynamic range requirements, i.e.

- Power control steps,

- Power control dynamic range,

- Total power dynamic range,

- IPDL Time mask.

In each case the uncertainty for the conducted measurement is the same as that for the conducted MU in TS 25.141 [2] as follows:

- Power control steps: Uncertainty of conducted measurement = 0.1 dB, Expanded OTA uncertainty = 0.1 dB.

- Power control dynamic range: Uncertainty of conducted measurement = 1.1 dB, Expanded OTA uncertainty = 1.1 dB.

- Total power dynamic range: Uncertainty of conducted measurement = 0.3 dB, Expanded OTA uncertainty = 0.3 dB.

- IPDL Time mask: Uncertainty of conducted measurement = 0.7 dB, Expanded OTA uncertainty = 0.7 dB.

##### [Unchanged Sections]

#### 9.5.4.3	Uncertainty budget format

The uncertainty budget format are the same as those in table 9.5.2.3-1 (excluding uncertainties from the NF to FF transformation, since the transformation is not needed) with the addition descriptions in table 9.5.4.3-1:

Table 9.5.4.3-1: Near Field uncertainty contributions for OTA E-UTRA Total power dynamic range measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 23 | Uncertainty of conducted measurement (conducted MU / 1.96) | F2 |
| Stage 1: Calibration measurement |  |  |

#### 9.5.4.4	MU value derivation, FR1

As the output power dynamics are relative measurements most of the uncertainties form the EIRP accuracy cancel out as the same error will be applied to both of the measured OTA signals.

The uncertainty budget descriptions are the same as those in table 6.4.4.4-1 (excluding uncertainties from the NF to FF transformation, since the transformation is not needed) with the addition descriptions in table 6.5.4.3-1:

The MU uncertainty assessment is shown in table 6.5.4.4-1, zero values have been omitted in the table for the sake of space, but still be considered as part of the budget.

Table 9.5.4.3-1: NFTR MU value derivation for OTA E-UTRA total power dynamic range measurement

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

##### [Unchanged Sections]

##### 10.2.2.3		Uncertainty budget format

Table 10.2.2.3 -1: Indoor Anechoic Chamber uncertainty contributions
for AAS BS EIS measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 |  |  | Positioning misalignment between the AAS BS and the reference antenna | C1-1 |
| 2 |  |  | Pointing misalignment between the AAS BS and the transmitting antenna. | C1-2 |
| 3 |  |  | Quality of quiet zone | C1-3 |
| 4 |  |  | Polarization mismatch between the AAS BS and the transmitting antenna | C1-4 |
| 5 |  |  | Mutual coupling between the AAS BS and the transmitting antenna | C1-5 |
| 6 |  |  | Phase curvature | C1-6 |
| 7 |  |  | Uncertainty of the RF signal source generator | E |
| 8 |  |  | Impedance mismatch in the transmitting chain | C1-8 |
| 9 |  |  | Random uncertainty | C1-9 |
| Stage 1: Calibration measurement |  |  |
| 10 |  |  | Impedance mismatch between the transmitting antenna and the network analyzer | C1-10 |
| 11 |  |  | Positioning and pointing misalignment between the reference antenna and the transmitting antenna | C1-11 |
| 12 |  |  | Impedance mismatch between the reference antenna and network analyzer. | C1-12 |
| 13 |  |  | Quality of quiet zone | C1-3 |
| 14 |  |  | Polarization mismatch for reference antenna | C1-4 |
| 15 |  |  | Mutual coupling between the reference antenna and the transmitting antenna | C1-5 |
| 16 |  |  | Phase curvature | C1-6 |
| 17 |  |  | Uncertainty of the network analyzer | E |
| 18 |  |  | Influence of the reference antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | C1-14 |
| 19 |  |  | Reference antenna feed cable loss measurement uncertainty | C1-15 |
| 20 |  |  | Influence of the transmitting antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | C1-16 |
| 21 |  |  | Uncertainty of the absolute gain of the reference antenna | E |
| 22 |  |  | Uncertainty of the absolute gain of the transmitting antenna | C1-18 |

#### 10.2.2.4		MU value derivation, FR1

Table 10.2.2.3-1 captures derivation of the expanded measurement uncertainty values for OTA sensitivity measurements in Indoor Anechoic Chamber (Normal test conditions, FR1).

Table 10.2.2.3-1: IAC MU value derivation for OTA sensitivity measurement, FR1

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

#### 10.2.2.5		MU value derivation, FR2

Table 10.2.2.4-1 captures derivation of the expanded measurement uncertainty values for OTA sensitivity measurements in Indoor Anechoic Chamber (Normal test conditions, FR2).

Table 10.2.2.4-1: IAC MU value derivation for OTA sensitivity measurement, FR2

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

##### [Unchanged Sections]

#### 10.2.3.3	Uncertainty budget format

# Table 10.2.3.3-1: CATR uncertainty budget format for EIS measurement

| UID | Description of uncertainty contribution | Details in anex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | C2-1 |
| 2 | Standing wave between DUT and test range antenna | C2-2 |
| 3 | Signal Generator uncertainty | E |
| 4 | RF leakage & dynamic range, a) feeder package terminated b) DUT terminated | C2-3 |
| 5 | QZ ripple DUT | C2-4 |
| 6 | Miscellaneous uncertainty | C2-13 |
| Stage 1: Calibration measurement |  |  |
| 7 | Uncertainty of network analyser and/or signal generator | E |
| 8 | Mismatch of transmitter chain | C2-6 |
| 9 | Insertion loss of transmitter chain | C2-7 |
| 10 | RF leakage & dynamic range | C2-8 |
| 11 | Influence of the calibration antenna feed cable a) Flexing cables, adapters, attenuators & connector repeatability | C2-9 |
| 12 | Uncertainty of the absolute gain of calibration antenna | E |
| 13 | Uncertainty due to antenna mounting apparatus or rotary joints | C2-11 |
| 14 | Quality of quiet zone | C2-4 |
| 15 | Rotary Joints | C2-14 |
| 16 | Misalignment positioning system | C2-15 |
| 17 | Misalignment calibration antenna | C2-1 |
| 18 | Standing wave between DUT and test range antenna | C2-16 |
| 19 | Switching uncertainty | C2-17 |

#### 10.2.3.4	MU value derivation, FR1

Table 10.2.3.3-1 captures derivation of the expanded measurement uncertainty values for OTA sensitivity measurements in CATR (Normal test conditions, FR1).

Table 10.2.3.3-1: CATR MU value derivation for OTA sensitivity measurements, Normal test conditions, FR1

#### 10.2.3.5		MU value derivation, FR2

Table 10.2.3.4-1 captures derivation of the expanded measurement uncertainty values for OTA sensitivity measurements in CATR (Normal test conditions, FR2).

Table 10.2.3.4-1: CATR MU value derivation for OTA sensitivity measurement, FR2

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

##### [Unchanged Sections]

#### 10.2.4.3	Uncertainty budget format

# Table 10.2.4.3-1: One Dimensional Compact Range Chamber uncertainty contributions

for EIS measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT and pointing error | C3-1 |
| 2 | Standing wave between DUT and test range antenna | C3-2 |
| 3 | Quiet zone ripple DUT | C3-3 |
| 4 | Signal generator | C3-4, E |
| 5 | Phase curvature | C3-5 |
| 6 | Polarization mismatch between DUT and transmitting antenna | C3-6 |
| 7 | Mutual coupling between DUT and transmitting antenna | C3-7 |
| 8 | Measurement equipment | C3-8, E |
| 9 | Impedance mismatch in transmitting chain | C3-9 |
| 10 | RF leakage and dynamic range a) Feeder package terminated b) DUT terminated | C3-10 |
| Stage 1: Calibration measurement |  |  |
| 11 | Misalignment positioning system | C3-11 |
| 12 | Pointing error between reference antenna and test range antenna | C3-12 |
| 13 | Impedance mismatch in path to reference antenna | C3-13 |
| 14 | Impedance mismatch in path to compact probe | C3-14 |
| 15 | Standing wave between reference antenna and test range antenna | C3-2 |
| 16 | Quiet zone ripple reference antenna | C3-4 |
| 17 | Phase curvature | C3-5 |
| 18 | Polarization mismatch between reference antenna and transmitting antenna | C3-6 |
| 19 | Mutual coupling between reference antenna and transmitting antenna | C3-7 |
| 20 | Measurement equipment | C3-8, E |
| 21 | Influence of reference antenna feed cable (flexing cables, adapters, attenuators, connector repeatability) | C3-15 |
| 22 | Mismatch of transmitter chain | C3-16 |
| 23 | Insertion loss of transmitter chain | C3-17 |
| 24 | Uncertainty of absolute gain of reference antenna | C3-18, E |
| 25 | RF leakage (SGH connector terminated and test range antenna connector cable terminated) | C3-10 |

#### 10.2.4.4	MU value derivation, FR1

Table 10.2.4.3-1 captures derivation of the expanded measurement uncertainty values for OTA sensitivity measurements in One Dimensional Compact Range.

Table 10.2.4.3-1: One Dimensional Compact Range measurement uncertainty value derivation for OTA sensitivity measurements, FR1

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

##### [Unchanged Sections]

#### 10.2.5.3	Uncertainty budget format

# Table 10.2.5.3-1: Near field test range uncertainty contributors in AAS EIS measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
|  | Stage 2: EIS near field radiation pattern measurement and EIS near field DUT power measurement |  |
| 1 | Axes Intersection | C4-1 |
| 2 | Axes Orthogonality | C4-2 |
| 3 | Horizontal Pointing | C4-3 |
| 4 | Probe Vertical Position | C4-4 |
| 5 | Probe H/V pointing | C4-5 |
| 6 | Measurement Distance | C4-6 |
| 7 | Amplitude and Phase Drift | C4-7 |
| 8 | Amplitude and Phase Noise | C4-8 |
| 9 | Leakage and Crosstalk | C4-9 |
| 10 | Amplitude Non-Linearity | C4-10 |
| 11 | Amplitude and Phase Shift in rotary joints | C4-11 |
| 12 | Channel Balance Amplitude and Phase | C4-12 |
| 13 | Probe Polarization Amplitude and Phase | C4-13 |
| 14 | Probe Pattern Knowledge | C4-14 |
| 15 | Multiple Reflections | C4-15 |
| 16 | Room Scattering | C4-16 |
| 17 | DUT support Scattering | C4-17 |
| 18 | Scan Area Truncation | C4-18 |
| 19 | Sampling Point Offset | C4-19 |
| 20 | Spherical Mode Truncation | C4-20 |
| 21 | Positioning | C4-21 |
| 22 | Probe Array Uniformity | C4-22 |
| 23 | Mismatch of transmitter chain (i.e. between receiving antenna and vector signal generator) | C4-23 |
| 24 | Insertion loss of transmitter chain | C4-24 |
| 25 | Uncertainty of the absolute gain of the probe antenna | C4-25 |
| 26 | Vector Signal Generator | E |
| 27 | Measurement repeatability - Positioning Repeatability | C4-27 |
|  | Stage 1:, Calibration measurement |  |
| 28 | Uncertainty of network analyser | E |
| 29 | Mismatch of transmitter chain | C4-29 |
| 30 | Insertion loss of transmitter chain | C4-30 |
| 31 | Mismatch in the connection of the calibration antenna | C4-31 |
| 32 | Influence of the calibration antenna feed cable | C4-32 |
| 33 | Influence of the probe antenna cable | C4-33 |
| 34 | Uncertainty of the absolute gain of the calibration antenna | E |
| 35 | Short term repeatability | C4-35 |

#### 10.2.5.4	MU value derivation, FR1

Table 10.2.5.3-1 captures derivation of the expanded measurement uncertainty values for OTA sensitivity measurements in Near Field Test Range.

Standard uncertainty values for the signal generator, network analyzer and reference antenna are according to the test equipment uncertainty values, as captured in annex C.

Table 10.2.5.3-1: NFTR measurement uncertainty value derivation for OTA sensitivity measurements, FR1

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

##### [Unchanged Sections]

#### 10.2.6.3	Uncertainty budget format

Table 10.2.6.3-1: Plane Wave Synthesizer uncertainty contributions for EIS measurements

| UID | Uncertainty Source | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | E6-1 |
| 2 | RF signal generator | F.1 |
| 3 | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for DUT antenna | E6-2 |
| 4 | RF leakage (calibration antenna connector terminated) | E6-3 |
| 5 | QZ ripple with DUT | E6-4 |
| 6 | Miscellaneous Uncertainty | E6-5 |
| 20 | System non-linearity | E6-14 |
| Stage 1: Calibration measurement |  |  |
| 7 | Uncertainty of network analyzer | F.1 |
| 8 | Mismatch (i.e. reference antenna, network analyzer and reference cable) | E6-6 |
| 9 | Insertion loss variation | E6-7 |
| 4 | RF leakage (calibration antenna connector terminated) | E6-3 |
| 10 | Influence of the calibration antenna feed cable | E6-8 |
| 11 | Uncertainty of the absolute gain of the calibration antenna | F.1 |
| 12 | Misalignment of positioning system | E6-9 |
| 13 | Misalignment of calibration antenna & pointing error | E6-1 |
| 14 | Rotary joints | E6-10 |
| 15 | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for calibration antenna | E6-2 |
| 16 | QZ ripple with calibration antenna | E6-4 |
| 17 | Switching uncertainty | E6-11 |
| 18 | Field repeatability | E6-12 |
| 19 | Frequency flatness | E6-13 |

#### 10.2.6.4	MU value derivation, FR1

Table 10.2.6.3-1 captures derivation of the expanded measurement uncertainty values for OTA sensitivity measurements in PWS.

Table 10.2.6.3-1: PWS MU value derivation for OTA sensitivity measurements, FR1

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

##### [Unchanged Sections]

## 11.2	OTA BS output power

##### [Unchanged Sections]

#### 11.2.2.3	Uncertainty budget format

##### The uncertainty budget format remains the same as EIRP measurement derived in subclause 9.2.2.

#### 11.2.2.4	MU value derivation, FR1

Table 11.2.2.3-1 captures derivation of the expanded measurement uncertainty values for OTA BS output power measurements in Indoor Anechoic Chamber (Normal test conditions, FR1).

##### [Unchanged Sections]

#### 11.2.3.3	Uncertainty budget format

##### The uncertainty budget format remains the same as EIRP measurement derived in subclause 9.2.3.

#### 11.2.3.4	MU value derivation, FR1

Table 11.2.3.3-1 captures derivation of the expanded measurement uncertainty values for OTA BS output power measurements in CATR (Normal test conditions, FR1).

Table 11.2.3.3-1: CATR MU value derivation for OTA BS output power measurement, FR1

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

#### 11.2.3.5	MU value derivation, FR2

A CATR MU budget was assessed in order to determine acceptable MU for the EIRP accuracy measurement in FR2. The CATR test setup and calibration and measurement procedures for FR2 are expected to be similar to those of FR1, although the test chamber dimensions and associated MU values will scale due to the shorter wavelengths and larger relative array apertures.

Table 11.2.3.4-1 captures derivation of the expanded measurement uncertainty values for OTA BS output power measurements in CATR (Normal test conditions, FR2).

Table 11.2.3.4-1: CATR MU value derivation for OTA BS output power measurement, FR2

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

[Unchanged Sections]

#### 11.2.4.3	Uncertainty budget format

The uncertainty budget format remains the same as EIRP measurement derived in subclause 9.2.4.

#### 11.2.4.4	MU value derivation, FR1

Table 11.2.4.3-1 captures derivation of the expanded measurement uncertainty values for OTA BS output power measurements in NFTR (Normal test conditions, FR1).

Table 11.2.4.3-1: NFTR MU value derivation for power density pattern measurement, FR1

## Editor’s note: placeholder for the MU table based on the Excel spreadsheet

[Unchanged Sections]

#### 11.2.5.3	Uncertainty budget format

The uncertainty budget format remains the same as EIRP measurement derived in subclause 9.2.5.

#### 11.2.5.4	MU value derivation, FR1

Table 11.2.5.3-1 captures derivation of the expanded measurement uncertainty values for OTA BS output power measurements in Reverberation Chamber (Normal test conditions, FR1).

Table 11.2.5.3-1: Reverberation chamber MU value derivation for OTA BS output power, FR1

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

#### 11.2.5.5	MU value derivation, FR2

Table 11.2.5.4-1 captures derivation of the expanded measurement uncertainty values for OTA BS output power measurements in Reverberation Chamber (Normal test conditions, FR2).

Table 11.2.5.4-1: Reverberation chamber MU value derivation for OTA BS output power, FR2

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

[Unchanged Sections]

#### 11.2.6.3	Uncertainty budget format

The uncertainty budget format remains the same as EIRP measurement derived in subclause 9.2.6.

#### 11.2.6.4	MU value derivation, FR1

Table 11.2.6.3-1 captures derivation of the expanded measurement uncertainty values for OTA BS output power measurements in PWS.

Table 11.2.6.3-1: PWS MU value derivation for OTA BS output power

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

[Unchanged Sections]

#### 11.3.2.3	Uncertainty budget format

#### The uncertainty budget format remains the same as EIRP measurement derived in subclause 9.2.2.

#### 11.3.2.4	MU value derivation, FR1

Table 11.3.2.3-1: IAC MU value derivation for the absolute ACLR measurement

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

[Unchanged Sections]

#### 11.3.3.3	Uncertainty budget format

The uncertainty budget format remains the same as EIRP measurement derived in subclause 9.2.3.

#### 11.3.3.4	MU value derivation, FR1

Table 11.3.3.3-1: CATR MU value derivation for the EIRP measurement of the absolute OTA ACLR, FR1

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

Table 11.3.3.3-2: CATR MU value derivation for the EIRP measurement of the relative OTA ACLR, FR1

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

#### 11.3.3.5	MU value derivation, FR2

A CATR MU budget was assessed in order to determine acceptable MU for the EIRP accuracy measurement in FR2. The CATR test setup and calibration and measurement procedures for FR2 are expected to be similar to those of FR1, although the test chamber dimensions and associated MU values will scale due to the shorter wavelengths and larger relative array apertures. However, it is noted that in order to achieve the test instrument uncertainties that were assumed, calibration of the spectrum analyzer may be needed.

For relative ACLR, the MU budget for CATR was assessed as follows:

Table 11.3.3.4-1: CATR MU value derivation for the EIRP measurement of the absolute OTA ACLR, FR2

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

Table 11.3.3.4-2: CATR MU value derivation for the EIRP measurement of the relative OTA ACLR, FR2

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

[Unchanged Sections]

#### 11.3.4.3	Uncertainty budget format

The uncertainty budget format remains the same as EIRP measurement derived in subclause 9.2.5.

#### 11.3.4.4	MU value derivation, FR1

Table 11.3.4.3-1 captures derivation of the expanded measurement uncertainty values for OTA ACLR measurements in NFTR (Normal test conditions, FR1).

Table 11.3.4.3-1: NFTR MU value derivation for absolute ACLR measurement

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

[Unchanged Sections]

#### 11.4.2.3	Uncertainty budget format

The uncertainty budget format remains the same as EIRP measurement derived in subclause 9.2.2.

#### 11.4.2.4 	MU value derivation, FR1

Table 11.4.2.3-1 captures derivation of the expanded measurement uncertainty values for OTA OBUE or OTA SEM measurements in Indoor Anechoic Chamber (Normal test conditions, FR1).

Table 11.4.2.3-1: IAC MU value derivation for OTA OBUE or OTA SEM measurement, FR1

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

[Unchanged Sections]

#### 11.4.3.3	Uncertainty budget format

The uncertainty budget format remains the same as EIRP measurement derived in subclause 9.2.3.

#### 11.4.3.4	MU value derivation, FR1

Table 11.4.3.3-1 captures derivation of the expanded measurement uncertainty values for OTA OBUE or OTA SEM measurements in CATR (Normal test conditions, FR1).

Table 11.4.3.3-1: CATR MU value derivation for OTA OBUE or OTA SEM measurement, FR1

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

#### 11.4.3.5	MU value derivation, FR2

A CATR MU budget was assessed in order to determine acceptable MU for the EIRP accuracy measurement in FR2. The CATR test setup and calibration and measurement procedures for FR2 are expected to be similar to those of FR1, although the test chamber dimensions and associated MU values will scale due to the shorter wavelengths and larger relative array apertures. However, it is noted that in order to achieve the test instrument uncertainties that were assumed, calibration of the spectrum analyzer may be needed.

Table 11.4.3.4-1 captures derivation of the expanded measurement uncertainty values for OTA OBUE measurements in CATR (Normal test conditions, FR2).

Table 11.4.3.4-1: CATR MU value derivation for OTA OBUE measurement, FR2

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

[Unchanged Sections]

#### 11.4.4.3	Uncertainty budget format

The uncertainty budget format remains the same as EIRP measurement derived in subclause 9.2.5.

#### 11.4.4.4	MU value derivation, FR1

Refer to subclause 11.2.4.3 (i.e. the MU value derivation for the OTA BS output power measurement in NFTR) for MU value per point measurement.

##### [End of changes]