3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2004184

Electronic Meeting, April 20 – 30, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | - | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR-U inter-RAT requirements for IDLE/INACTIVE states |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic |  |  |  |  |  | Date: |  |  | 2020-04-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR-U inter-RAT requirements are currently missing in TS 36.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inter-RAT measurement requirements for carrier subject to CCA are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for cell change from LTE to NR-U is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section: 4.2.2.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change 1

##### 4.2.2.5.6	Measurements of NR cells subject to CCA

The UE shall be able to identify new inter-frequency cells subject to CCA and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP_CCA and Squal > SnonIntraSearchQ_CCA then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search.

If Srxlev ≤ SnonIntraSearchP_CCA or Squal ≤ SnonIntraSearchQ_CCA then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this subclause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS38.304 within Kcarrier_CCA * Tdetect,NR_Inter_CCA  if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least 5 dB in FR1 for reselections based on ranking or 6dB in FR1 for SS-RSRP reselections based on absolute priorities or 4dB in FR1 for SS-RSRQ reselections based on absolute priorities and provided that Md ≤ Md,max. Otherwise, if Md > Md,max the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1]. The parameter Kcarrier_CCA is the number of NR inter-frequency carriers indicated by the serving cell. An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B.x.y for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter_CCA provided Mm ≤ Mm,max. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier_CCA * Tmeasure,NR_Inter_CCA (see table 4.2.2.5.6-1) for identified lower or equal priority inter-frequency cells provided Mm ≤ Mm,max. Otherwise, the UE shall restart the measurement. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter_CCA/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 within Kcarrier_CCA * Tevaluate,NR_Inter_CCA when Treselection = 0 as specified in table 4.2.2.5.6-1 provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and

when rangeToBestCell is not configured:

-	the cell is at least 5dB better ranked in FR1 or.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them

-	the cell is at least 5dB better ranked in FR1 if the current serving cell is among them. or

-	6dB in FR1 for SS-RSRP reselections based on absolute priorities or

-	4dB in FR1 for SS-RSRQ reselections based on absolute priorities.

- when Me ≤ Me,max. Otherwise, if Me > Me,max the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1].

When evaluating cells for reselection, the SSB side conditions apply to both serving and inter-frequency cells.

If Treselection timer has a non zero value and the inter-frequency cell is satisfied with the reselection criteria, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in Table 4.2A.2.4-1 under the following conditions:

-	TSMTC_intra = TSMTC_inter = 160 ms; where TSMTC_intra and TSMTC_inter are periodicities of the SMTC occasions configured for the intra-frequency carrier and the inter-frequency carrier respectively, and

-	SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion [1].

Table 4.2.2.5.6-1: Tdetect,NR_Inter_CCA, Tmeasure,NR_Inter_CCA and Tevaluate,NR_Inter_CCA

| DRX cycle length [s] | Tdetect,NR_Inter_CCA [s] (number of DRX cycles) | Tmeasure,NR_Inter_CCA [s] (number of DRX cycles) | Tevaluate,NR_Inter_CCA [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 0.32x([36]+Md) {([36]+ Md) } | 0.32x([4]+Mm) {([4]+ Mm) } | 0.32x([16]+Me) {([16]+ Me) } |
| 0.64 | 0.64x([28]+ Md) {[28]+ Md } | 0.64x([2]+ Mm) {[2]+ Mm } | 0.64x([8]+ Me) {[8]+ Me } |
| 1.28 | 1.28x([25]+ Md) {[25]+ Md } | 1.28x([1]+ Mm) {[1]+ Mm } | 1.28x([5]+ Me) {[5]+ Me } |
| 2.56 | 2.56x([23]+ Md) {[23]+ Md } | 2.56x([1]+ Mm) {[1]+ Mm } | 2.56x([3]+ Me) {[3]+ Me } |
| Note 1: Mm ≤ Mm,max, where: Mm,max = [16] for DRX cycle = 0.32 seconds, Mm,max = [8] for DRX cycle = 0.64 seconds, Mm,max = [4] for DRX cycle = 1.28 seconds, Mm,max = [4] for DRX cycle = 2.56 seconds, Note 2: Md ≤ Md,max, where: Md,max = [4] * Mm,max, Note 3: Me ≤ Me,max, where: Me,max = [2] * Mm,max, |  |  |  |

End of change 1