3GPP TSG-RAN WG4 Meeting #94bis-e			  											R4-2004225

Electronic Meeting, 20th Apr. 2020 – 24th Apr. 2020

Source: 	Nokia, Optus

Title: 		TP for 37.716-31-11 to introduce DC_3-7-28_n40

Agenda Item: 			8.5.2 [DC_R16_3BLTE_1BNR_4DL2UL-Core]

Document for:	Agreement

# 1	Introduction

This contribution is a TP for TR 37.716-31-11 to introduce DC_3-7-28_n40.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 5.1.x	DC_3-7-28_n40

### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (four bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7-28_n40 | CA_3-7-28 | n40 | No |

### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-7A-28A_n40A | DC_3A_n40A DC_7A_n40A DC_28A_n40A | CA_3A-7A-28A | n40A |

### 5.1.x.3	∆TIB and ∆RIB values

For DC_3-7-28_n40, the TIB,c and RIB,c values are used from CA_3-7-28-40 in TS 36.101.

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n40 | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 28 | 0.3 |
|  | n40 | 0.9 |

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n40 | 3 | 0 |
|  | 7 | 0.3 |
|  | 28 | 0 |
|  | n40 | 0.8 |

*********************** End of the TP ***************************************************

# 3	References