3GPP TSG-RAN WG4 Meeting #94bis-e 	R4-2004233

# Electronic Meeting, 20th Apr. 2020 – 24th Apr. 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draft | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to introduce SEM and ACLR for NR-U to TS 38.101-1 with suffix |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2020-04-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As per RAN4 agreement, R4-1915979, a general spectrum emission mask (SEM) with additional specific SEMs in the case of non-transmitted channels are to be defined for NR-U. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A suffix, F, is added to clause 4.3 to contain the requirements specific for NR-U. In clause 6.5, the ACLR for PC4 (20dBm) is added and clause 6.5F is included with the specific spectrum emission mask for NR-U. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The will be no emission requirements specified for NR-U operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3, 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521 series |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*********************************** Start of Changes ****************************************

## 4.3	Specification suffix information

Unless stated otherwise the following suffixes are used for indicating at 2nd level clause, shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplement Uplink (SUL) |
| D | UL MIMO |
| F | NR Unlicensed (NR-U) |

A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional suffix clause  in clauses 5, 6 and 7. Where there is a difference in requirement between the general requirements and the additional clause requirements  in clauses 5, 6 and 7, the tighter requirements are applicable unless stated otherwise in the additional clause.

A terminal which supports more than one feature in clauses 5, 6 and 7 shall meet all of the separate corresponding requirements.

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly. For a terminal that supports SUL, the current version of the specification assumes the terminal is not configured with UL MIMO on SUL carrier.

******************************** Unchanged Sections Omitted ******************************

## 6.5	Output RF spectrum emissions

******************************** Unchanged Sections Omitted ******************************

#### 6.5.2.1	General

The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an adjacent channel leakage power ratio.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5.2.2	Spectrum emission mask

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in clause 6.5.3 are applicable.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

The power of any UE emission shall not exceed the levels specified in Table 6.5.2.2-1 for the specified channel bandwidth.

Table 6.5.2.2-1: General NR spectrum emission mask

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  | -24 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 | -25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  | -25 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  | -25 |  |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  | -25 |  |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  | -25 |  |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  | -25 |  |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  | -25 |  |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  | -25 |  |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  | -25 |  |  |  |  |  |
| ± 65-70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 70-75 |  |  |  |  |  |  |  |  |  | -25 |  |  |  |  |
| ± 75-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  | -25 |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  | -25 |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  | -25 |  |

#### 6.5.2.3	Additional spectrum emission mask

******************************** Unchanged Sections Omitted ******************************

#### 6.5.2.4	Adjacent channel leakage ratio

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.5.2.4.1	NR ACLR

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.4.1-1.

If the measured adjacent channel power is greater than –50 dBm then the NRACLR shall be higher than the value specified in Table 6.5.2.4.1-2.

Table 6.5.2.4.1-1: NR ACLR measurement bandwidth

| NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| NR ACLR measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 2 | Power class 3 | Power class 4 |
| --- | --- | --- | --- |
| NR ACLR | 31 dB | 30 dB | 27 dB |

##### 6.5.2.4.2	UTRA ACLR

******************************** Unchanged Sections Omitted ******************************

## 6.5F	Output RF spectrum emissions for NR-U

### 6.5F.1	Occupied bandwidth for NR-U

Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. The occupied bandwidth for all transmission bandwidth configurations (Resources Blocks) shall be less than the channel bandwidth specified in Table 6.5F.1-1.

Table 6.5F.1-1: Occupied channel bandwidth for NR-U

|  | NR channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 10 MHz | 20 MHz | 40 MHz | 60 MHz | 80 MHz |
| Occupied channel bandwidth (MHz) | 10 | 20 | 40 | 60 | 80 |

Note that in the case of non-transmitted channels due to failure of LBT these channels are not considered as a part of the bandwidth which shall contain 99 % of the total integrated mean power of the transmitted spectrum.

### 6.5F.2	Out of band emission for NR-U

#### 6.5F.2.1	General

This clause contains requirements for out of band emissions for UE configured for NR-U operation.

#### 6.5F.2.2	Spectrum emission mask

For channels assigned within band n46 the relative power of any UE emission shall not exceed the levels specified in Table 6.5F.2.2-1 for the specified channel bandwidth. The spectrum emission mask is defined relative to the maximum power density in a 1 MHz measurement bandwidth within the channel bandwidth.

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR-U channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in clause 6.5F.3 are applicable.

Table 6.5F.2.2-1: General spectrum emission mask for NR-U

| Spectrum emission limit (dBr) / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz |  | 20 MHz |  | 40 MHz |  | 60 MHz | 80 MHz | Measurement bandwidth |
| ± 0-1 | -20 ∆fOOB |  |  |  |  |  |  |  | [100kHz]3 |
| ± 1-5 | NOTE 1 | NOTE 1 |  | NOTE 1 |  | NOTE 1 |  | NOTE 1 | 1 MHz |
| ± 5-10 | NOTE 2 |  |  |  |  |  |  |  |  |
| ± 10-20 | -40 | NOTE 2 |  |  |  |  |  |  |  |
| ± 20-30 |  | -40 |  | NOTE 2 |  |  |  |  |  |
| ± 30-40 |  |  |  |  |  | NOTE 2 |  |  |  |
| ± 40-50 |  |  |  | -40 |  |  |  | NOTE 2 |  |
| ± 50-60 |  |  |  |  |  |  |  |  |  |
| ± 60-70 |  |  |  |  |  | -40 |  |  |  |
| ± 70-80 |  |  |  |  |  |  |  |  |  |
| ± 80-100 |  |  |  |  |  |  |  | -40 |  |
| NOTE 1: Given as: -20-8A ∆fOOB-1 where A= Channel Bandwidth2-1 NOTE 2: Given as: -16-12B ∆fOOB where B= Channel Bandwidth2 NOTE 3: The measured value shall be scaled by a factor equal to the ratio of the reference bandwidth (1 MHz) to the measurement bandwidth before the emission limit (dBr) is applied. |  |  |  |  |  |  |  |  |  |

#### 6.5F.2.2-1	Spectrum emission mask for non-transmitted channels

In the case of non-transmitted 20 MHz channel(s) on the edges of an assigned NR-U channel bandwidth the general spectrum emission mask specified in Table 6.5F.2.2-1 is applied to the remaining transmitted channels to form an additional spectrum emission mask. The additional spectrum emission mask is applied to the total bandwidth of the remaining transmitted channels.

The additional spectrum emission mask is floored at -28dBr.

The relative power of any UE emission shall not exceed the most stringent levels given by the initial general spectrum emission mask with full channel bandwidth and the additional spectrum emission mask with the channel bandwidth of the transmitted channels in the case of non-transmitted channels at the edge of an assigned NR-U channel bandwidth.

An exception to the spectrum emission requirements for the non-transmitted 20 MHz channels allows a single [2] MHz bandwidth to extend to [-28] dBc relative to total transmit power, or -20 dBm, whichever is the greatest.

#### 6.5F.2.3	Additional spectrum emission mask

Captured in clause 6.5.2.3 for bands supporting NR-U operation.

#### 6.5F.2.4	Adjacent channel leakage ratio

The requirements in clause 6.5.2.4 apply.

### 6.5F.3	Spurious emission for NR-U

The requirements in clause 6.5.3 apply.

************************************** End of Changes ***************************************