3GPP TSG-RAN WG4 Meeting # 94-e-Bis 	R4-2004270

# Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:	6.1.5.11

Source: 	Huawei, HiSilicon

Title: 	Discussion on L1-RSRP measurement requirement for NR-U

Document for:	Discussion

- Introduction

In last meeting RAN4 94e meeting, the RRM measurement for NR-U were discussed. There are some remaining issues about L1-RSRP measurement requirement. We further investigate these issues and present views in this paper.

- Discussion

Based on the discussions in the last RAN4#94e meeting, progress about L1-RSRP has been made in the following aspects:

- L1,max = 0 when timeRestrictionForChannelMeasurement is configured - Periodic and aperiodic L1-RSRP reporting delay reuses Rel-15 reporting delay - FFS: - Option 1: CSI-RS L1-RSRP measurements follow the same approach of SSB L1-RSRP measurements: - Same number of samples for both SSB based L1-RSRP measurement and CSI-RS - Same extension value L1 and L1-max as in SSB based L1-RSRP - Option 2: A new approach or conditions are needed for CSI-RS based L1-RSRP measurements under LBT failure. - Other options are not precluded. - Detailed UE behavior when receiving the MAC CE deactivation command for semi-persistent CSI reporting, in case of UL LBT failure for sending the ACK - Option 1: - If UE cannot transmit HARQ-ACK on MAC-CE deactivation due to UL CCA failure, UE continues to be in its previous state, i.e., it should measure and report L1-RSRP until it successfully transmits HARQ-ACK - Option 2: - For semi-persistent CSI reporting with PUCCH, if UE cannot transmit HARQ-ACK on the MAC CE deactivation due to the UL LBT failures, UE continues the L1-RSRP measurement and delay the L1-RSRP reporting. If UE does not receive deactivation command during the delay period, UE restart to transmit L1-RSRP reporting. FFS how to extend the delay.

UE behavior when receiving the MAC CE deactivation command for semi-persistent CSI reporting, in case of UL LBT failure for sending the ACK

Based on the discussion from the last meeting, when UE is not able to transmit HARQ feedback for deactivation command, UE shall continue the L1-RSRP measurement for both option 1 and option 2.

Observation 1: Based on the discussion from the last meeting, when UE is not able to transmit HARQ feedback for deactivation command, UE shall continue the L1-RSRP measurement for both option 1 and option 2.

For option 1, if UE keep reporting L1-RSRP results, it will waste UL resource since the reporting has been deactivated by NW. For option 2, it is preferred to delay the L1-RSRP reporting when the HARQ feedback cannot be transmitted after receiving the MAC CE deactivation command.

Proposal 1: It is preferred to delay the L1-RSRP reporting when the HARQ feedback cannot be transmitted after receiving the MAC CE deactivation command.

However, delaying the L1-RSRP reporting means UE shall store the measurement results which will lead to extra memory efforts. From our understanding, it is obvious that the UE could abandon the stored measurement results when exceeding the time limits.

Proposal 2: A time limit shall be defined when the L1-RSRP reporting is delayed. When exceeding the time limits, UE shall abandon the stored measurement results, where the time limit is FFS.

Besides the time limit mentioned above, the UE shall also abandon the measurement results when the HARQ feedback is retransmitted for the deactivation command.

Proposal 3: The UE shall also abandon the measurement results when the HARQ feedback is retransmitted for the deactivation command.

As mentioned in option 2, If UE does not receive deactivation command during the delay period, UE restart to transmit L1-RSRP reporting. Does it means that UE shall report the stored measurement results when restart the report transmission, and it need to be confirmed with RAN2 that whether it is possible for UE to report multiple L1-RSRP reports in one shot.

Proposal 4: It is FFS whether UE shall report the stored measurement results when restart the report transmission, and it need to be confirmed with RAN2 that whether it is possible for UE to report multiple L1-RSRP reports in one shot.

Proposal 5: The delay is FFS, and whether it is the same as the time limits in proposal 2.

CSI-RS from different transmission burst

RAN1 99 Agreement: A UE shall not average CSI-RS measurements for channel estimation across different transmission bursts from the UE's perspective. FFS: Potential issues due to AGC

The above issues arose during the discussion of RLM in last RAN4#94e meeting. Based on the above agreements from RAN1, UE shall not filtering the CSI-RS measurement across different transmission bursts from UE’s perspective. According to the definition from TS 37.213, the definition of transmission burst is:

A DL transmission burst is defined as a set of transmissions from an eNB/gNB without any gaps greater than 16us. Transmissions from an eNB/gNB separated by a gap of more than 16us are considered as separate DL transmission bursts. An eNB/gNB can transmit transmission(s) after a gap within a DL transmission burst without sensing the corresponding channel(s) for availability.

However, from our understanding, the DL transmission burst is invisible from UE’s perspective, which means it may impossible for UE to distinguish whether CSI-RSs are from different transmission burst. Besides, it shall also be considered for other CSI-RS based measurement (CSI-RS based RLM/BFD/CBD). It will greatly impact the measurement performance for the CSI-RS based measurement when the transmit power across different occasions are different, which also will leads to AGC problem.

Observation 2: the DL transmission burst is invisible from UE’s perspective, which means it may impossible for UE to distinguish whether CSI-RSs are from different transmission burst.

Observation 3: It will greatly impact the measurement performance for the CSI-RS based measurement when the transmit power across different occasions are different, which also will leads to AGC problem.

Based on the observation above, we present an accompanied LS [3] to RAN1 to confirm the observations from RAN4’s perspective.

Proposal 6: Send an LS to RAN1 about the observation from RAN4’ perspective about the above issues.

- Conclusions

Observation 1: Based on the discussion from the last meeting, when UE is not able to transmit HARQ feedback for deactivation command, UE shall continue the L1-RSRP measurement for both option 1 and option 2.

Proposal 1: It is preferred to delay the L1-RSRP reporting when the HARQ feedback cannot be transmitted after receiving the MAC CE deactivation command.

Proposal 2: A time limit shall be defined when the L1-RSRP reporting is delayed. When exceeding the time limits, UE shall abandon the stored measurement results, where the time limit is FFS.

Proposal 3: The UE shall also abandon the measurement results when the HARQ feedback is retransmitted for the deactivation command.

Proposal 4: It is FFS whether UE shall report the stored measurement results when restart the report transmission, and it need to be confirmed with RAN2 that whether it is possible for UE to report multiple L1-RSRP reports in one shot.

Proposal 5: The delay is FFS, and whether it is the same as the time limits in proposal 2.

Observation 2: the DL transmission burst is invisible from UE’s perspective, which means it may impossible for UE to distinguish whether CSI-RSs are from different transmission burst.

Observation 3: It will greatly impact the measurement performance for the CSI-RS based measurement when the transmit power across different occasions are different, which also will leads to AGC problem.

Proposal 6: Send an LS to RAN1 about the observation from RAN4’ perspective about the above issues.

References

[1] R4-2002278 WF on NR-U RRM requirements (Part 3), Nokia, Nokia Shanghai Bell

[2] TS 37.213

[3] R4-2004275