3GPP TSG RAN WG4 Meeting #94bis-e	R4-2004386

Electronic Meeting, 20 – 30 April, 2020

Source:		Rohde & Schwarz

Title:	TP to TR 38.827 to introduce probe layout and IFF based system

Agenda Item:	9.1.3.2

Document for:	Approval

# Introduction

Following the technical discussion and proposals presented in [2] and [3], this contribution is introducing the TP for the number of probes and probe layout to be used for NR FR2 MIMO OTA. It also introduces the IFF based approach.

# References

- R4-2002482,	“TR 38.827 v1.2.0 NR MIMO OTA”, CAICT, RAN4 #94-e, February 2020

- R4-2004382, “System layout analysis and proposal for NR FR2 MIMO OTA”, Rohde & Schwarz, RAN4 #94-e-Bis, April 2020

- R4-2004384, “IFF based system for NR FR2 MIMO OTA”, Rohde & Schwarz, RAN4 #94-e-Bis, April 2020

< Unchanged Text Deleted >

< Beginning of Changes >

### 6.2.3	3D Multi-Probe Anechoic Chamber (MPAC) for FR2

The 3D MPAC test method is the reference methodology for FR2 NR MIMO OTA testing. By arranging an array of probes around the Equipment Under Test (EUT), a spatial distribution of angles of arrival in the 3D MPAC system may be simulated to expose the EUT to a near field environment that appears to have originated from a complex multipath far field environment.

As illustrated schematically in Figure 6.2.3-1, signals propagate from the base station/communication tester to the EUT through a simulated multipath environment known as a spatial channel model, where appropriate channel impairments such as Doppler and fading are applied to each path prior to injecting all of the directional signals into the chamber simultaneously through the probe array. The resulting field distribution in the test zone is then integrated by the EUT antenna(s) and processed by the receiver(s) just as it would do so in any non-simulated multipath environment.

The 3D MPAC system with 4 dual polarized probes (illustrated with black dots in Figure 6.2.3-1) may be implemented with two approaches:

- IFF based method, also known as Compact Antenna Test Range, as baseline.

- DFF based method, using a minimum range length of 0.75m from the centre of the test zone.

Figure 6.2.3-1: 3D MPAC system layout for NR FR2 MIMO OTA testing

The 4 dual-polarized probes shall be placed using the following layout in Table 6.2.3-1:

Table 6.2.3-1: 3D MPAC probe layout

| Probe # | Azimuth in [°] | Elevation in [°] |
| --- | --- | --- |
| 1 | -51.00 | 17.50 |
| 2 | -41.00 | 15.00 |
| 3 | -16.00 | 15.00 |
| 4 | 9.00 | 15.00 |

< End of Changes >