3GPP TSG-RAN4 Meeting #94bis-e	R4-2004436

E-meeting, April 20th – April 30th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR Introduction of handover delay requirements for conditional handover (section 6.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CR capturing handover delay requirements for conditional handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR captures the handover delay requirements for conditional handover in section 6.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete requirements for handover delay requirements for conditional handover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 6.1, New section 6.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.141 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Compared to technically endorsed CR the requirements are captured in new section 6.1.4 (new number) named NR Conditional Handover. Section numbering updated accordingly. No other changes. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of technically endorsed R4-1915782. Updated version of R4-2001338 according to TP in discussion paper R4-2001337. |  |  |  |  |  |  |  |  |

### 6.1.4	NR Conditional Handover

#### 6.1.4.1	Introduction

The purpose of NR handover is to change the NR PCell to another NR cell. The requirements in this clause are applicable to SA NR, NE-DC and NR-DC. The requirements in this clause are applicable to handover and conditional handover.

##### 6.1.4.2	NR FR1 – NR FR1 conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR1 cell to NR FR1 cell.

##### 6.1.4.2.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause12 in TS 38.331 [2] plus the measurements time stated in clause 6.1.1.6.2 plus the conditional execution preparation time in clause 6.1.1.6.3 plus the interruption time stated in clause 6.1.1.6.4.

6.1.4.2.2	Measurement time

The measurement time delay is defined as the time between the RRC procedure delay and the point when the UE executes a handover to a target cell and preparation time starts.

The measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than T identify intra with index or T identify inter without index defined in clause 9.2.5.1 or clause 9.3.4. When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period T identify_intra_without_index or T identify_inter_with_index as defined in clause 9.2.5.1 or clause 9.3.4. If a cell which has been detectable at least for the time period T identify intra without index or T identify inter with index defined in clause 9.2.5.1 or clause 9.3.4 becomes undetectable for a period and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200 Tc while the measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used, an additional delay can be expected.

6.1.4.2.3	Preparation time

Preparation time is the time from when after measurement time and the time during which the UE prepares the target cell for conditional handover.

TCHO_execution is the time needed for preparing the conditional handover to the target cell.

TCHO_execution can be up [FFS].

6.1.4.2.4	Interruption time

The interruption time is the time between when the conditional handover execution is triggered in the UE and the time when the UE executes the conditional handover to the target cell.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tinterrupt = Tprocessing + TIU + T∆ ms

Where:

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3]

T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

##### 6.1.4.3	NR FR2 – NR FR1 conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR2 cell to NR FR1 cell.

The requirements in section 6.1.1.6 apply for this section.

##### 6.1.4.4	NR FR2 – NR FR2 conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR2 cell to NR FR2 cell.

The requirements in section 6.1.1.6 apply for this section.

##### 6.1.4.5	NR FR1 – NR FR2 conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR1 cell to NR FR2 cell.

The requirements in section 6.1.1.6 apply for this section.