3GPP TSG-RAN WG4 Meeting#94bis-e                	R4-2004459

e-Meeting, April 20th – April 30th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Mandatory GPs for NR Rel-16

Agenda item:	6.15.1.6

Document for:	Discussion

Introduction

For a number of meetings RAN4 has been discussing new mandatory gap patterns to be applicable from Rel-16 and for NR. In last meeting a number of papers discussed how new GPs could be introduced and under which conditions would need to be discussed and agreed in RAN4. A WF was agreed in [2].

In this paper we give a proposal how a new conditional mandatory GP can be introduced and the needed conditions.

Discussion

RAN4 meeting in Reno confirmed the following agreements related to introducing a new measurement GP:

- Confirmed R15 capability information elements cannot be changed in meaning by any new capability signaling introduced in R16

- Confirmed R15 gap pattern applicability rule is not changed due to introduction of additional mandatory gap patterns in R16

Hence, in general the introduction of new mandatory GPs cannot impact legacy signalling.

Additionally, there were concerns from companies if the new mandatory GP would impact LTE implementation. It was highlighted that the introduction of any new mandatory GP shall be done without making it mandatory for LTE and should not impact the LTE implementation.

This would need to be accounted during the definition of the new GP and following RAN4 would need to discuss the detailed applicability of the new mandatory GPs and the new GP. Additionally, if agreement is reached, an LS would need to be sent to RAN2 to inform RAN2 about what has been decided and let RAN2 design any necessary signalling.

Signalling and Capability

How to indicate the network that the UE support a certain new GP using existing (or modified) signalling has been discussed for some time in RAN4. However, we believe this is a RAN2 responsibility area which they can handle once RAN4 informs RAN2 which new GP are mandatory and the conditions when the new mandatory GPs can be used – i.e. the applicability. We do not think RAN4 need to spend discussing this further:

How to solve the signalling of support of new mandatory GPs is up to RAN2.

Additionally, there has been some discussions related to capability signalling and whether any new mandatory GP is introduced using some additional capability signalling. If RAN4 decides to introduce new mandatory GP in Rel-16 it seems to be evident that such GP is mandatory, and no additional capability is needed. Hence, new mandatory GP introduced are by nature mandatory and mandatory for all UEs – no capability signalling is needed.

No capability signalling is needed when introducing new mandatory gap patterns in Rel-16.

Applicability of new mandatory GP

In order to progress the work in RAN4 to introduce a new mandatory GP it needs to be clear when the new GP would be applicable and when not.

Looking at the many aspects that needs to be considered we have next split the analysis into two high level cases:

- Per-UE gap capable UE and;

- Per-FR gap capable UE.

The reason for this is that any new mandatory GP will be applicable in different configurations due to the gap pattern support – Per-UE or Per-FR – as explained next.

Per UE Gap UE

A UE supporting Per-UE gap will have one GP configured and it applies to the UE and for FR1 and FR2 simultaneously. For a UE only supporting common gap pattern we now look at the different deployment scenarios:

NR-SA and NR-DC:

When the UE is operating in NR-SA and NR-DC gap patterns are configured by gNB. Because UE is operating in NR mode only any new mandatory GP only impact NR and will not impact LTE signalling wise. However, if the UE also have LTE measurements configured while in NR-SA/DC the new gap pattern cannot be configured. Hence, new mandatory GP is applicable in NR SA and NR-DC only for NR gap assisted measurements:

- GP is configured by NR PCell

- Can be applied for both NR FR1 and NR FR2 gap assisted measurements

  - Provided no LTE gap assisted measurement are configured.

For a UE which support Per-UE gaps, if any LTE measurements are configured/active the new mandatory GP cannot be used.

EN-DC:

For a UE supporting Per-UE gap pattern operating in EN-DC mode cannot be configured with any new mandatory GP. Reason why a new mandatory GP is not applicable in EN-DC – even for NR gap assisted measurements – is that the measurement configuration and therefore also the measurement gap configuration is configured by the PCell. Hence, it will impact LTE – both UE and network.

NE-DC:

For the UE supporting per-UE GP, the GP is common for both LTE and NR. Therefore, applying any new gap pattern will impact the LTE legacy implementation. First of all the GP is configured by the PCell and will therefore impact legacy signalling. Secondly, because this is per-UE GP it will impact gap handling in LTE. This means that any new mandatory GP is not applicable in NE-DC for even the UE is only configured with NR gap assisted measurements.

Summary for UE supporting Per-UE gap pattern:

Based on former analysis we observe following for a UE only supporting common GP:

A new mandatory GP can be applied when UE is operating in NR-DC and NR-SA mode, when UE support common gaps. This is conditioned that no LTE gap assisted measurement are configured.

Any new mandatory GP cannot be applicable to a UE supporting common gaps when operating in EN-DC or NE-DC mode.

Any new mandatory GP cannot be applied for a UE supporting common gaps in LTE SA.

A new mandatory GP can be applied for a UE supporting common gaps in NR SA, provided no gap assisted LTE measurements are configured.

Per FR Gap UE

Next we analyse the applicability of any new mandatory gap pattern for a UE supporting Per-FR gap pattern. Such UE can have different gap patterns configured for FR1 and FR2. We look once again at each of the different deployment scenarios:

NR-SA and NR-DC:

When the UE support Per-UE GP, a new mandatory GP is applicable in NR SA and NR-DC for NR gap assisted measurements. Reasoning behind this is that the NR gap pattern is configured by NR PCell and does not impact LTE signalling. Any gap can be applied to NR gap assisted measurements in FR2 and conditionally in FR1 (if no LTE gap assisted measurements are configured):

- GP is configured by NR PCell

- Any new mandatory GP can be applied for NR FR2 gap assisted measurements

- Any new mandatory GP can be applied for NR FR1 gap assisted measurements

  - Provided no LTE gap assisted measurement are configured in FR1.

EN-DC with PSCell in FR1 or FR2:

For a UE supporting Per-FR gap pattern operating in EN-DC, any new mandatory GP can be applied for any gap assisted measurement on FR2 carriers based under certain conditions. Gap assisted measurements using the new mandatory GP can be configured in FR2 if:

- UE is operating in EN-DC mode and the GP is configured by gNB (NR).

Any Gap assisted measurements in FR1 shall use legacy GPs.

Additionally, it should be observed that gap assisted measurements using any new mandatory GP cannot be configured in FR2 if, in EN-DC, the GPs are configured by eNB (LTE). Reasoning is that this would impact the LTE signalling.

NE-DC with PCell in FR1 and FR2:

For any gap assisted measurement configured on FR2 carriers, the new mandatory GP is applicable. Additionally, any gap assisted measurements in FR1 cannot use new mandatory GP but shall be using legacy GPs.

Summary for a UE supporting Per-FR GP:

A new mandatory GP can be applied when the UE support Per-FR GP and operating in NR SA and NR-DC modes. The new mandatory GP is applicable for FR2 NR gap assisted measurements and FR1 NR gap assisted measurements provided no LTE gap assisted measurement are configured in FR1.

A new mandatory GP can be applied in for a UE operating in EN-DC, when the UE support Per-FR GP, provided that the new GP is for FR2 NR measurements and configured by gNB.

A UE supporting Per-FR GP in NE-DC, any new mandatory GP is applicable for any gap assisted measurement configured on FR2 carriers. Any gap assisted measurements in FR1 shall be using legacy GPs.

New GP cannot be applied for a UE supporting Per-FR gaps in LTE SA.

New GP can be applied for a UE supporting common gaps in NR SA in FR2 and FR1 provided no LTE gap assisted measurements are configured.

New Gap applicability summary

Above discussion and observations is summarized in the following applicability table:

| Measurement gap pattern configuration | Configuration | Measurement Purpose | Applicability of Gap Pattern Id x |
| --- | --- | --- | --- |
| Per UE Gap | EN-DC | any gap assisted measurements | No |
|  | NE-DC | any gap assisted measurements | No |
|  | NR-DC | NR FR1 and FR2 gap assisted measurements | YesNote 1 |
|  | NR SA | NR FR1 and FR2 gap assisted measurements | YesNote 1 |
|  | LTE SA | Any gap assisted measurements | No |
| Per FR Gap Note 3 | EN-DC | FR2 gap assisted measurement | YesNote 2 |
|  | EN-DC | FR2 gap assisted measurements | NoNote 4 |
|  | NE-DC | NR FR2 gap assisted measurements | yes |
|  | NE-DC | FR1 gap assisted measurements | No |
|  | NR-DC | NR FR1 and FR2 gap assisted measurements | YesNote 5 |
|  | NR SA | NR FR1 and FR2 gap assisted measurements | YesNote 5 |
|  | LTE SA | Any gap assisted measurements | No |
| Note 1: provided no LTE gap assisted measurements are configured Note 2: provided FR2 gap pattern is configured by gNB Note 3: FR1 and FR2 gap patterns can be configured by either eNB or gNB Note 4: If gap pattern for FR1 and FR2 are configured by eNB Note 5: provided no LTE gap assisted measurements are configured in FR1 |  |  |  |

Agree on the applicability table for the new mandatory GPs.

Open questions

The WF agreed in last meeting listed several open questions. here we try to give our views on the different aspects:

- Definition of NR only measurement:

  - NR only measurement in our view means, as discussed in former section, that gaps are used only for NR measurements. This need to consider the UE capability of Per-UE and Per-FR gap pattern support.

- Applicable scenarios of UE capability for NR only measurement:

  - this is captured in a very detailed manner in the applicability table in section 2.2 including all exceptions.

- Purpose of UE capability for NR only measurements:

  - Our view is that RAN4 informs in detail the new mandatory GPs and when they apply and leave the signaling design to RAN2.

- Applicable scenarios for additional mandatory gap patterns:

  - Please see applicability table in section 2.2

- Mandatory with capability signaling

  - Any new mandatory gap pattern is mandatory, and no capability is needed.

New mandatory Gap Pattern

RAN4 would additionally need to agree on which of the current GPs would be mandatory under the agreed applicability.

As existing GPs 12-23 can only be applied to FR2 gap assisted measurements, these GPs would not add value as new mandatory GP if the measurement purpose would be NR FR1 measurements.

Current existing mandatory GP#0 and GP#1 are both of 6ms gap length. Having new mandatory GP with shorter gap length would enable more system efficiency for the case where the carrier to be measured would have SSB burst length which is shorter than the maximum of 64 SSB bursts.

Currently the gap repetition period can only be 40ms or 80ms which impacts the overall expected measurement delay on cell detection and measurements of NR inter-frequency measurements. Having a new mandatory GP with gap a shorter gap repetition period would help in reducing the overall NR inter-frequency measurement delay.

Considering above we would prefer to focus on a new gap pattern which can be applied for both FR1 and FR2 measurements. Hence, the new mandatory GP would be one in the range of GP#[2 – 11]. This would ensure that the GP can be used for both NR gap assisted measurements in FR1 and FR2, while only sacrificing a potential reduction of the overall gap overhead in FR2 with an acceptable amount – between 0.5 – 1.5ms.

New mandatory GPs for FR1 and FR2 are among GP#2 – GP#11 gap patterns.

Out of the GP#2-GP#11 we would prefer to focus on a GP which can help in reducing the overall measurement gap overhead when a full 6ms measurement gap is not needed. Hence, in that sense we do not see a strong reason to mandate GP#4 or GP#5.

Considering selection between a measurement GP with 3ms or 4ms gap length we prefer the 4ms gap length GP as the new mandatory GP would have be used also for FR2 gap assisted measurements. One can expect that in FR1 the SSB bursts should easily fall within 3ms (4ms - FR1 gap considering switching delay of 0.5ms). For FR2 we would expect a larger number of SSB bursts and thereby a longer SSB burst period. A 4ms measurement gap length in FR2 would lead to an efficient measurement time of 3.5ms (4ms gap length – FR2 gap switching delay of 0.25ms). A 3.5ms measurement time would be a reasonable compromise to balance the existing gap length and having a shorter gap length for the cases where the deployments use less than maximum number of SSB burst.

New GPs for FR1 and FR2 are among GP#6 – GP#9 gap pattern.

For the GPs we have 4 different gap repetitions of 20, 40, 80 and 160ms. The impact from the repetition factor is 2-fold:

- each gap impacts the network scheduling opportunity and potentially the TP.

- measurement delay scales with the measurement gap repetition.

Which GP to be used would very much depend on the specific deployment and scenario. Short gap repetition period can be used to ensure shorter expected measurement delays while it would have greater impact on the gap overhead. A longer gap repetition period would ensure that measurement delay is same as with existing mandatory GPs while having additional benefit of lowered measurement gap overhead. We would prefer all 4 GP#6-GP#9 as mandatory GP from Rel-16.

From Rel-16 GP#6 – GP#9 are defined as mandatory GPs for FR1 and FR2.

agree that new mandatory GPs are applicable according to applicability table in section 2.2.3.

No new capability is introduced, and the new GPs are mandatory without capability signalling.

Conclusion

In this paper we give a proposal how a new conditional mandatory GP can be introduced and the needed conditions. Initially we propose:

- How to solve the signalling of support of new mandatory GPs is up to RAN2.

No capability signalling is needed when introducing new mandatory gap patterns in Rel-16.

During the discussion we made several observations:

Summary for UE supporting Per-UE gap pattern:

- A new mandatory GP can be applied when UE is operating in NR-DC and NR-SA mode, when UE support common gaps. This is conditioned that no LTE gap assisted measurement are configured.

Any new mandatory GP cannot be applicable to a UE supporting common gaps when operating in EN-DC or NE-DC mode.

Any new mandatory GP cannot be applied for a UE supporting common gaps in LTE SA.

A new mandatory GP can be applied for a UE supporting common gaps in NR SA, provided no gap assisted LTE measurements are configured.

Summary for a UE supporting Per-FR GP:

A new mandatory GP can be applied when the UE support Per-FR GP and operating in NR SA and NR-DC modes. The new mandatory GP is applicable for FR2 NR gap assisted measurements and FR1 NR gap assisted measurements provided no LTE gap assisted measurement are configured in FR1.

A new mandatory GP can be applied in for a UE operating in EN-DC, when the UE support Per-FR GP, provided that the new GP is for FR2 NR measurements and configured by gNB.

A UE supporting Per-FR GP in NE-DC, any new mandatory GP is applicable for any gap assisted measurement configured on FR2 carriers. Any gap assisted measurements in FR1 shall be using legacy GPs.

New GP cannot be applied for a UE supporting Per-FR gaps in LTE SA.

New GP can be applied for a UE supporting common gaps in NR SA in FR2 and FR1 provided no LTE gap assisted measurements are configured.

Based on which and based on additional discussion lead to following proposals:

Agree on the applicability table for the new mandatory GPs.

| Measurement gap pattern configuration | Configuration | Measurement Purpose | Applicability of Gap Pattern Id x |
| --- | --- | --- | --- |
| Per UE Gap | EN-DC | any gap assisted measurements | No |
|  | NE-DC | any gap assisted measurements | No |
|  | NR-DC | NR FR1 and FR2 gap assisted measurements | YesNote 1 |
|  | NR SA | NR FR1 and FR2 gap assisted measurements | YesNote 1 |
|  | LTE SA | Any gap assisted measurements | No |
| Per FR Gap Note 3 | EN-DC | FR2 gap assisted measurement | YesNote 2 |
|  | EN-DC | FR2 gap assisted measurements | NoNote 4 |
|  | NE-DC | NR FR2 gap assisted measurements | yes |
|  | NE-DC | FR1 gap assisted measurements | No |
|  | NR-DC | NR FR1 and FR2 gap assisted measurements | YesNote 5 |
|  | NR SA | NR FR1 and FR2 gap assisted measurements | YesNote 5 |
|  | LTE SA | Any gap assisted measurements | No |
| Note 1: provided no LTE gap assisted measurements are configured Note 2: provided FR2 gap pattern is configured by gNB Note 3: FR1 and FR2 gap patterns can be configured by either eNB or gNB Note 4: If gap pattern for FR1 and FR2 are configured by eNB Note 5: provided no LTE gap assisted measurements are configured in FR1 |  |  |  |

New mandatory GPs for FR1 and FR2 are among GP#2 – GP#11 gap patterns.

New GPs for FR1 and FR2 are among GP#6 – GP#9 gap pattern.

From Rel-16 GP#6 – GP#9 are defined as mandatory GPs for FR1 and FR2.

Agree that new mandatory GPs are applicable according to applicability table in section 2.2.3 (above)

No new capability is introduced, and the new GPs are mandatory without capability signalling.

References

- 38.133

- R4-2002248, WF on R16 NR RRM enhancements – mandatory gap patterns, ZTE