3GPP TSG-RAN WG4 Meeting #94-e-Bis		R4-2004553

E-meeting, 20 April – 30 April, 2020

Agenda item:	6.18.1.1

Source:	Intel Corporation

Title:	Simulation results for Normal CA requirements

Document for:	Discussion

# Introduction

In RAN4 #94-e paper with simulation assumptions for Normal CA requirements was approved [1].

In this paper we provide simulation results for Normal CA scenarios.

# Discussion

## FR1 TDD 15 kHz simulation results

In this section we provide alignment and impairment results for FR1 TDD scenarios with 15 kHz SCS. Figure 1 illustrates PDSCH throughput curves for different BW configurations and number of UE receive antennas.

| 2 Rx UE |  |
| --- | --- |
| 4 Rx UE |  |
| Figure 1. PDSCH simulation results for FR1 15 kHz. |  |

Summary of alignment simulation results for FR1 TDD scenarios with 15 kHz SCS is provided in Table 1.

Table 1. Alignment FR1 15 kHz simulation results

| Number of Rx Antenna | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 Rx | 10.9 | 11.1 | 11.2 | 11.1 | 11.3 | 11.1 | 11.6 | 12.0 |
| 4 Rx | 6.2 | 6.2 | 6.3 | 6.2 | 6.4 | 6.2 | 6.4 | 6.6 |

Summary of impairment simulation results for FR1 TDD scenarios with 15 kHz SCS is provided in Table 2.

Table 2. Impairment FR1 15 kHz simulation results

| Number of Rx Antenna | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 Rx | 12.9 | 13.1 | 13.2 | 13.1 | 13.3 | 13.1 | 13.6 | 14.0 |
| 4 Rx | 8.2 | 8.2 | 8.3 | 8.2 | 8.4 | 8.2 | 8.4 | 8.6 |

## FR2 simulation results

In this section we provide alignment and impairment results for FR2 scenarios with 120 kHz SCS. Figure 2 illustrates PDSCH throughput curves for different BW configurations and FRCs.

| Rank 1, MCS 13 | Rank 2, MCS 10 |
| --- | --- |
| Figure 2. PDSCH simulation results for FR2 120 kHz. |  |

Summary of alignment simulation results for FR2 scenarios with 120 kHz SCS is provided in Table 3.

Table 3. Alignment FR2 120 kHz simulation results

| FRC | 50 MHz | 100 MHz | 20 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| Rank 1, MCS 13 | 4.8 | 4.8 | 4.7 | 4.8 |
| Rank 2, MCS 10 | 8.1 | 7.9 | 8.0 | 8.1 |

Summary of impairment simulation results for FR2 scenarios with 120 kHz SCS is provided in Table 4

Table 4. Impairment FR2 120 kHz simulation results

| FRC | 50 MHz | 100 MHz | 20 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| Rank 1, MCS 13 | 6.8 | 6.8 | 6.7 | 6.8 |
| Rank 2, MCS 10 | 10.1 | 9.9 | 10.0 | 10.1 |

# Conclusion

In this paper we provided alignment and impairment simulation results for FR1 TDD 15 kHz and FR2 Normal CA scenarios.

# References

- R4-2002392 “Simulation assumptions for NR normal CA UE performance requirements”, Intel, RAN4 #94-e, February 2020.